Workplace Transport Risk Assessment

Health & Safety – Risk Assessments

Workplace transport incidents are one of the leading causes of fatalities on UK construction sites. This assessment covers the risks from vehicle and plant movements on site in accordance with the Workplace (Health, Safety and Welfare) Regulations 1992, CDM 2015 and HSE guidance HSG136. It must be reviewed whenever the site layout, traffic routes or types of vehicles/plant change.

| Document Ref: | Enter document reference |
| --- | --- |
| Revision: | Enter revision number |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract No: | Enter contract number |
| Assessed By: | Enter name and role |
| Approved By: | Enter name and role |

# Site Transport Information

| Site Speed Limit: | e.g. 10 mph on site roads, 5 mph in compound area |
| --- | --- |
| Types of Vehicles on Site: | e.g. HGVs, tippers, concrete wagons, low loaders, vans, cars, telehandlers, excavators, dumpers |
| Peak Vehicle Movements: | e.g. 30 deliveries per day, 50 plant movements per day |
| Peak Workforce: | Enter peak number of pedestrians on site |
| One-Way System: | Yes / No – describe |
| Pedestrian / Vehicle Segregation: | Describe the segregation arrangements |
| Traffic Management Plan Ref: | Enter TMP drawing reference |
| Public Highway Interface: | Describe access from public road (visibility, turning, Chapter 8 signage) |

# Hazard Assessment

Assess each hazard. Record the initial risk (before controls) and the residual risk (after controls). L = Likelihood (1–4), S = Severity (1–4), R = L x S.

| No. | Hazard | Who | Control Measures | L | S | R | L | S | R | Further Action |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Initial |  |  | Residual |  |  |  |
| 1 | Vehicle striking a pedestrian | All site personnel, visitors | Physical separation of pedestrian and vehicle routes (barriers, kerbs). Designated crossing points. High-vis mandatory. Speed limits enforced. One-way systems. Banksmen at blind spots. | 4 | 4 |  |  |  |  |  |
| 2 | Reversing vehicle striking a person | Pedestrians, banksmen, ground workers | Minimise reversing (one-way systems, turning areas). Reversing cameras and sensors on all vehicles. Banksman / signaller for all reversing. Reversing alarms operational. Exclusion zones during reversing. | 4 | 4 |  |  |  |  |  |
| 3 | Vehicle overturning on slopes, soft ground or ramps | Drivers, operators, nearby workers | Routes on firm, compacted ground. Gradient limits observed. Speed restrictions on slopes. Plant with ROPS. Seat belts worn. Avoid traversing slopes where possible. | 3 | 4 |  |  |  |  |  |
| 4 | Collision between vehicles / plant | Drivers, operators | One-way traffic flow where possible. Speed limits (typically 5–15 mph). Give-way signage at junctions. Adequate route width for passing. Mirrors at blind corners. Communication between operators. | 3 | 3 |  |  |  |  |  |
| 5 | Unsecured loads falling from vehicles | Driver, following vehicles, pedestrians | Loads secured before moving. Nets, straps, chains as appropriate. Check loads before departure and after each stop. Sheeting for loose materials. Tail gates closed and secured. | 3 | 3 |  |  |  |  |  |
| 6 | Vehicle leaving the site onto public highway with mud/debris | Public road users | Wheel wash at site exit. Road sweeper deployed as needed. Rumble grids at exit. Visual check of vehicle cleanliness before leaving site. Sheeting of loose loads. | 3 | 3 |  |  |  |  |  |
| 7 | Unauthorised or incompetent drivers / operators | All persons on site | All drivers must hold valid licence for vehicle class. All plant operators must hold valid competence card (CPCS / NPORS). Licences and cards checked at induction. No unauthorised use of vehicles or plant. | 2 | 4 |  |  |  |  |  |
| 8 | Poor visibility (dust, darkness, weather, blind spots) | Drivers, pedestrians | Adequate site lighting on all routes. Dust suppression. Clean windscreens and mirrors. High-vis clothing mandatory. Banksmen at blind spots. Suspend operations in very poor visibility. | 3 | 3 |  |  |  |  |  |
| 9 | Delivery vehicles unfamiliar with site | Delivery drivers, site personnel | Delivery driver induction at gatehouse. Escorted to unloading area by banksman. Speed limit communicated. Site map provided. No unsupervised movement on site. | 3 | 3 |  |  |  |  |  |
| 10 | Tyre blow-out or mechanical failure | Driver, nearby persons | Daily pre-use vehicle checks (tyres, brakes, steering, lights). Defective vehicles taken out of service immediately. Maintenance programme in place. Speed limits reduce severity. | 2 | 3 |  |  |  |  |  |
| 11 | Fuel spill during refuelling | Driver, environment | Designated refuelling area on hardstanding with bunding. Drip trays during refuelling. Spill kit available. No smoking. Engine off during refuelling. Trained operatives only. | 2 | 2 |  |  |  |  |  |
| 12 | Pedestrians on vehicle routes | Pedestrians | Dedicated pedestrian walkways separated from traffic routes. Crossing points with signage. High-vis mandatory. Induction covers traffic management. Pedestrian exclusion from loading/unloading areas. | 3 | 4 |  |  |  |  |  |

# Vehicle and Plant Types on Site

| No. | Vehicle / Plant Type | Quantity on Site | Reversing Aids? | FOPS / ROPS? | Beacon? | Pre-Use Check? |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | HGV tipper |  |  |  |  |  |
| 2 | Concrete mixer truck |  |  |  |  |  |
| 3 | Low loader |  |  |  |  |  |
| 4 | Transit van |  |  |  |  |  |
| 5 | Telehandler |  |  |  |  |  |
| 6 | 360° excavator |  |  |  |  |  |
| 7 | Dump truck (articulated) |  |  |  |  |  |
| 8 | Dump truck (rigid) |  |  |  |  |  |
| 9 | Roller / compactor |  |  |  |  |  |
| 10 | Mobile crane |  |  |  |  |  |
| 11 | Fork lift truck |  |  |  |  |  |
| 12 | Other |  |  |  |  |  |

# Action Plan

| No. | Action Required | Responsible | Target Date | Priority | Status |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |

# Assessor Sign-Off

| Assessed By: |  | Date: |  |
| --- | --- | --- | --- |
| Signature: |  | Review Date: |  |

# Site Manager Approval

| Approved By: |  | Date: |  |
| --- | --- | --- | --- |
| Signature: |  | Role: |  |

# Notes / Additional Comments

Enter any additional notes.

# Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |