Flood Response Procedure

Health & Safety – Seasonal Hazards

| Document Ref: | Enter document reference |
| --- | --- |
| Revision: | Enter revision number |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract No: | Enter contract number |
| Site Address: | Enter full site address |
| Prepared By: | Enter name and role |
| Approved By: | Enter name and role |

# Site Flood Risk Information

| EA Flood Zone: | Zone 1 (low) / Zone 2 (medium) / Zone 3a (high) / Zone 3b (functional floodplain) |
| --- | --- |
| Nearest Watercourse: | Enter name and distance from site |
| Historical Flooding: | Has the site flooded before? Describe known flood events. |
| Flood Risk Assessment Ref: | Enter FRA reference (if available from planning) |
| Site Lowest Level: | Enter the lowest point on site (AOD or relative) |
| Predicted Flood Level: | Enter predicted flood level for design event (if known) |
| Surface Water Risk: | Describe surface water flood risk (pluvial flooding from heavy rain) |
| Groundwater Risk: | Describe groundwater flooding risk (if applicable) |

# Flood Alert Trigger Levels

The Environment Agency issues flood alerts and warnings. The following actions must be taken at each alert level.

| EA Warning Level | Site Alert | Actions |
| --- | --- | --- |
| Flood Alert – Flooding is Possible | YELLOW | Monitor EA flood warnings. Brief workforce. Check pumps operational. Review this procedure. Confirm emergency contacts. Secure loose materials near water. |
| Flood Warning – Flooding is Expected | AMBER | Activate flood response. Move plant and materials from low-lying areas. Increase pump capacity. Deploy temporary flood barriers / sandbags. Secure fuel and chemical stores. Brief all personnel on evacuation routes. |
| Severe Flood Warning – Danger to Life | RED | EVACUATE all personnel from the site immediately. Do NOT enter floodwater. Account for all persons at assembly point. Shut down and isolate electrical supply if safe. Contact emergency services if persons are in danger. Do NOT return until the all-clear is given. |

# Flood Emergency Procedure

1. MONITOR – Check EA flood warnings at gov.uk/check-flooding or Floodline 0345 988 1188. Monitor river levels, weather forecast and site drainage.

2. ALERT – When flood warning is issued: notify site manager, brief all personnel, activate flood response team.

3. PROTECT – Deploy sandbags / flood barriers. Move plant, fuel, chemicals and materials to higher ground. Close bund valves. Increase pump capacity.

4. ISOLATE – If flooding is imminent: isolate electrical supply at the main distribution point (if safe to do so). Shut down plant and equipment.

5. EVACUATE – If a Severe Flood Warning is issued or water is rising rapidly: evacuate ALL personnel via the designated flood evacuation route to the assembly point on high ground.

6. ACCOUNT – Headcount at assembly point. Report any missing persons to emergency services immediately.

7. DO NOT ENTER FLOODWATER – 6 inches of fast-flowing water can knock a person off their feet. Do NOT drive through floodwater. Do NOT attempt to rescue persons from floodwater unless trained.

8. RECOVER – After waters recede: do NOT re-enter site until declared safe. Check structural stability. Check for contamination. Reinstate electrical supply with care (check for water ingress). Arrange clean-up.

# Pre-Flood Preparedness Checklist

| No. | Preparedness Check | Yes | No | N/A | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | Environment Agency flood alerts and warnings are being monitored (Floodline 0345 988 1188 or gov.uk/check-flooding) |  |  |  |  |
| 2 | Site flood risk assessment has been reviewed and is current |  |  |  |  |
| 3 | Site drainage system is clear, functional and has capacity for heavy rainfall |  |  |  |  |
| 4 | Dewatering pumps are operational with backup pumps available and tested |  |  |  |  |
| 5 | Fuel levels in pump generators are sufficient for 24+ hours continuous operation |  |  |  |  |
| 6 | Temporary flood barriers, sandbags or flood boards are available and accessible |  |  |  |  |
| 7 | Emergency sandbag supply is available on site (or contact details for rapid supply confirmed) |  |  |  |  |
| 8 | Fuel and chemical bunds are intact, valves closed, and have adequate freeboard for rainfall |  |  |  |  |
| 9 | Spill kits are stocked and accessible |  |  |  |  |
| 10 | Low-lying excavations have been assessed for flood risk and dewatering is adequate |  |  |  |  |
| 11 | Plant and equipment in flood-risk areas can be moved to higher ground quickly |  |  |  |  |
| 12 | Welfare units and site offices are raised or protected from floodwater ingress |  |  |  |  |
| 13 | Electrical distribution boards and temporary supplies are elevated above predicted flood level where possible |  |  |  |  |
| 14 | Emergency isolation points for electrical supply are identified and accessible |  |  |  |  |
| 15 | Evacuation routes have been identified that avoid low-lying and flood-prone areas |  |  |  |  |
| 16 | Assembly point is on high ground and accessible during a flood event |  |  |  |  |
| 17 | All personnel have been briefed on flood evacuation procedures |  |  |  |  |
| 18 | Emergency contact list is up to date (EA, emergency services, client, insurers) |  |  |  |  |
| 19 | Neighbouring properties and watercourse operators have been notified of site drainage arrangements |  |  |  |  |
| 20 | Pollution prevention measures are in place to prevent contaminated runoff during flooding |  |  |  |  |

# Emergency Contacts

| Contact | Name | Phone Number |
| --- | --- | --- |
| Emergency Services (999) |  | 999 |
| Environment Agency Floodline |  | 0345 988 1188 |
| Site Manager |  |  |
| Project Manager |  |  |
| Client Emergency Contact |  |  |
| Insurance Company |  |  |
| Pump Hire Company |  |  |
| Sandbag Supplier |  |  |
| Local Authority Flood Team |  |  |
| Neighbouring Properties |  |  |

# Dewatering Pump Register

| No. | Pump Type | Location | Capacity | Fuel Level | Operational? | Backup Available? |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |

# Flood Event Record

| Date | Time | EA Warning | Action Taken | Water Level | Recorded By |
| --- | --- | --- | --- | --- | --- |

# Post-Flood Recovery Checklist

| No. | Recovery Action | Done? | Initials |
| --- | --- | --- | --- |
| 1 | Site has been declared safe to re-enter by the site manager |  |  |
| 2 | Structural stability of excavations, temporary works and buildings checked by competent person |  |  |
| 3 | Electrical installations checked for water ingress before re-energising (by competent electrician) |  |  |
| 4 | Fuel and chemical stores checked for contamination or damage |  |  |
| 5 | Environmental pollution check – no contamination of watercourses or drains |  |  |
| 6 | Plant and equipment checked for water damage before re-starting |  |  |
| 7 | Welfare facilities checked, cleaned and operational |  |  |
| 8 | Drainage system cleared of debris and functional |  |  |
| 9 | Photographs taken of flood damage for insurance and records |  |  |
| 10 | Incident report completed and client / insurers notified |  |  |

# Sign-Off

| Prepared By: |  | Date: |  |
| --- | --- | --- | --- |
| Signature: |  | Role: |  |

| Approved By: |  | Date: |  |
| --- | --- | --- | --- |
| Signature: |  | Review Date: |  |

# Notes / Additional Comments

Enter any additional notes.

# Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |