Rooftop Safe Access Plan

Health & Safety – Working at Height

| Document Ref: | Enter document reference |
| --- | --- |
| Revision: | Enter revision number |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract No: | Enter contract number |
| Prepared By: | Enter name and role |
| Approved By: | Enter name and role |

# Roof Details

| Building / Structure: | Enter building name or reference |
| --- | --- |
| Roof Type: | e.g. Flat, pitched, metal deck, concrete slab, membrane, tiled, slate, profiled sheet |
| Roof Height Above Ground: | Enter height in metres |
| Roof Area (approx.): | Enter approximate area in m² |
| Roof Pitch / Slope: | e.g. Flat (< 10°), low pitch (10–30°), steep pitch (> 30°) |
| Fragile Surfaces Present: | Yes / No – if yes, describe (rooflights, fibre cement, deteriorated areas) |
| Parapet Wall Height: | Enter height in mm (if present) or state ‘None’ |
| Existing Anchor Points: | Yes / No – if yes, describe (type, locations, certification status) |
| Activities Planned on Roof: | e.g. Roofing installation, plant installation, antenna work, inspection, cleaning, waterproofing |
| Maximum No. of Workers: | Enter maximum number of workers on the roof at any one time |
| Duration of Roof Work: | Enter planned duration |
| Relevant Drawing Refs: | Enter drawing references showing roof layout, access, fragile areas, edge protection |

# Working at Height – Hierarchy of Control

The Work at Height Regulations 2005 require the following hierarchy to be applied:

| Priority | Principle | Examples |
| --- | --- | --- |
| 1 | AVOID work at height | Can the work be done from ground level? Can prefabricated elements be assembled at ground level and lifted into position? |
| 2 | PREVENT falls using collective measures | Edge protection (guard rails, barriers, parapets). Working platforms (scaffolding, MEWP). Netting below fragile surfaces. |
| 3 | MINIMISE the distance and consequence of falls | Safety nets, soft landing systems, airbags. Fall arrest systems (harness + lanyard + anchor) as last resort. Rescue plan for every fall arrest system. |

# Access and Egress

| No. | Check Item | Yes | No | N/A | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | Safe means of access to the roof has been identified and assessed (internal stairway, external scaffold stair tower, MEWP, ladder access with platform) |  |  |  |  |
| 2 | Access route is clear, unobstructed and adequately lit |  |  |  |  |
| 3 | Ladder access (if used) complies with the Work at Height Regulations 2005 – ladder secured, extends 1m above landing, safe angle (75° / 1:4 ratio) |  |  |  |  |
| 4 | Scaffold stair tower is erected by a competent person with a current scaffold inspection tag |  |  |  |  |
| 5 | Hatch, trapdoor or access opening has a secure guard when open and a means of closing securely |  |  |  |  |
| 6 | Access is restricted to authorised personnel only – signage and physical barriers in place |  |  |  |  |
| 7 | A permit-to-access system is in operation for all rooftop work |  |  |  |  |
| 8 | All personnel accessing the roof have been briefed on this safe access plan |  |  |  |  |

# Edge Protection

| No. | Check Item | Yes | No | N/A | Initials |
| --- | --- | --- | --- | --- | --- |
| 9 | Permanent edge protection (parapet wall or railing) exists at all roof edges and is adequate (≥1100mm height with toe board, mid-rail and top rail) |  |  |  |  |
| 10 | Where permanent edge protection is insufficient, temporary edge protection has been installed |  |  |  |  |
| 11 | Temporary edge protection is a collective measure (guard rail, barrier) rather than reliance on personal PPE |  |  |  |  |
| 12 | Edge protection is in place at all unprotected edges, openings, skylights and fragile areas |  |  |  |  |
| 13 | Scaffolding with guard rails has been erected around the roof perimeter where required |  |  |  |  |
| 14 | Fall arrest anchors or lifeline systems have been installed and certified where edge protection is not practicable |  |  |  |  |
| 15 | Safety nets have been considered and installed below where appropriate |  |  |  |  |
| 16 | All roof-level openings (stairwells, lift shafts, service risers) are covered or guarded |  |  |  |  |

# Fragile Surfaces

| No. | Check Item | Yes | No | N/A | Initials |
| --- | --- | --- | --- | --- | --- |
| 17 | All fragile surfaces have been identified (rooflights, skylights, fibre cement sheets, liner panels, glazed areas, deteriorated felt/asphalt) |  |  |  |  |
| 18 | Fragile surfaces are clearly marked with ‘FRAGILE ROOF – DO NOT WALK’ warning signs |  |  |  |  |
| 19 | Crawling boards, roof ladders or walkway systems are provided over or adjacent to fragile surfaces |  |  |  |  |
| 20 | Barriers are installed at least 2 metres from fragile surfaces to prevent inadvertent approach |  |  |  |  |
| 21 | Fall arrest netting is installed below fragile rooflights where persons may fall through |  |  |  |  |
| 22 | Workers have been briefed on the location and extent of all fragile surfaces |  |  |  |  |
| 23 | The roof plan (attached) clearly marks all fragile areas |  |  |  |  |

# Weather Conditions

| No. | Check Item | Yes | No | N/A | Initials |
| --- | --- | --- | --- | --- | --- |
| 24 | Wind speed has been checked and is below the safe limit for rooftop work (typically 25 mph sustained, or as specified in the method statement) |  |  |  |  |
| 25 | No lightning forecast – refer to the Lightning Risk Procedure |  |  |  |  |
| 26 | Roof surface is not icy, snow-covered or excessively wet (slip risk assessed) |  |  |  |  |
| 27 | Rain is not expected to create unsafe working conditions |  |  |  |  |
| 28 | Temperature and UV exposure have been considered (refer to Heat Stress Risk Assessment in summer) |  |  |  |  |
| 29 | Work will be suspended if weather conditions deteriorate and become unsafe |  |  |  |  |

# Rescue Plan and Emergency

| No. | Check Item | Yes | No | N/A | Initials |
| --- | --- | --- | --- | --- | --- |
| 30 | A rescue plan is in place for the recovery of a fallen or suspended worker |  |  |  |  |
| 31 | Rescue equipment is available on site (retrieval system, stretcher, lowering device) |  |  |  |  |
| 32 | All personnel on the roof are trained in the rescue procedure |  |  |  |  |
| 33 | Emergency services have been considered – can they access the roof area? |  |  |  |  |
| 34 | First aid provision is adequate and a first aider is available |  |  |  |  |
| 35 | Communication system is in place between roof workers and ground level (radio, mobile, visual) |  |  |  |  |

# PPE Requirements for Rooftop Work

| PPE Item | Requirement | Notes |
| --- | --- | --- |
| Hard hat with chin strap (EN 397) | Mandatory | Chin strap required for roof work to prevent loss in wind |
| Hi-vis vest / jacket (EN ISO 20471) | Mandatory | Visible to crane operators and ground personnel |
| Safety boots with ankle support (EN ISO 20345 S3) | Mandatory | Non-slip sole; ankle support for uneven surfaces |
| Full body harness (EN 361) | Where fall arrest is required | Must be face-fit tested. Inspect before each use. Attach to certified anchor. |
| Shock-absorbing lanyard (EN 355) | Where fall arrest is required | Max 2m length. Calculate total fall distance including deceleration. |
| Self-retracting lifeline (EN 360) | Where fall arrest is required | For extended movement along the roof |
| Safety glasses (EN 166) | Task-specific | When cutting, drilling, or in dusty conditions |
| Gloves (EN 388) | Task-specific | Cut-resistant for handling metal sheets; grip gloves for general work |

# Roof Plan Sketch

Attach a roof plan showing: access points, edge protection, fragile areas, exclusion zones, anchor points, walkway routes, material storage areas, emergency escape routes. Alternatively, sketch below.

Roof plan sketch area – mark access, edges, fragile areas, anchors, exclusion zones

# Authorisation to Access Roof

I confirm that all pre-access checks have been completed satisfactorily, edge protection and fall prevention measures are in place, the rescue plan is established, all personnel are briefed, and roof access may proceed.

| Authorised By: |  | Date: |  |
| --- | --- | --- | --- |
| Signature: |  | Role: |  |

# Site Manager Approval

| Name: |  | Date: |  |
| --- | --- | --- | --- |
| Signature: |  | Role: |  |

# Notes / Additional Comments

Enter any additional notes or comments here.

# Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |