Drainage CCTV Survey Report Form

| Document Ref: | Enter document reference |
| --- | --- |
| Revision: | Enter revision number |
| Survey Date: | DD/MM/YYYY |
| Project Name: | Enter project name |
| Contract No: | Enter contract number |
| Survey Company: | Enter CCTV survey company name |
| Operator Name: | Enter camera operator name |
| Prepared By: | Enter name and position |
| Approved By: | Enter name and position |

# Survey Details

| Survey Type: | Pre-construction / Post-construction / Adoption / Defect Investigation |
| --- | --- |
| Survey Standard: | WRc Manual of Sewer Condition Classification (MSCC5) / BS EN 13508-2 |
| Camera Type: | Enter camera make and model |
| Weather Conditions: | Dry / Light Rain / Heavy Rain / Other |
| Flow Conditions: | Dry / Low Flow / Medium Flow / High Flow |
| Drainage System: | Foul / Surface Water / Combined / Highway |
| Pipe Material: | Clay / PVC-U / Concrete / Ductile Iron / HDPE / GRP / Other |
| Nominal Diameter (mm): | Enter pipe diameter |

# Pipe Run Details

| Run Ref | From MH | To MH | Length (m) | Dia. (mm) | Material | Gradient | Survey Dir. |
| --- | --- | --- | --- | --- | --- | --- | --- |

# CCTV Observation Log

Record all observations in accordance with WRc MSCC5 coding system or BS EN 13508-2. Enter distance from start manhole, observation code, and description.

| No. | Run Ref | Distance (m) | Clock Ref | Observation / Defect Description | WRc Code | Severity (1-5) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |

## Common WRc MSCC5 Defect Codes (Reference)

| Code | Description | Category | Typical Scoring |
| --- | --- | --- | --- |
| CJ | Cracked joint | Structural | 1-3 |
| CC | Cracked circumferential | Structural | 1-5 |
| CL | Cracked longitudinal | Structural | 1-5 |
| B | Broken pipe | Structural | 3-5 |
| D | Deformed pipe | Structural | 1-5 |
| CO | Collapse | Structural | 5 |
| J | Displaced joint | Structural | 1-5 |
| OJ | Open joint | Structural | 1-4 |
| IR | Infiltration running | Service | 1-3 |
| ID | Infiltration dripping | Service | 1-3 |
| RS | Root mass (severe) | Service | 3-5 |
| RF | Root fine | Service | 1-3 |
| DE | Debris / deposits | Service | 1-3 |
| OB | Obstruction | Service | 1-5 |
| IC | Intruding connection | Construction | 1-3 |
| DC | Defective connection | Construction | 1-4 |

# Survey Summary and Assessment

| Total Runs Surveyed: |  |
| --- | --- |
| Total Length Surveyed (m): |  |
| Runs Passed: |  |
| Runs Failed / Requiring Action: |  |
| Runs Not Accessible: |  |
| Overall Structural Grade: | Grade 1 (Good) to Grade 5 (Collapsed) per MSCC5 |
| Overall Service Grade: | Grade 1 (Good) to Grade 5 (Blocked) |

Overall Assessment and Recommendations

Summarise the overall findings of the CCTV survey. Identify runs requiring remedial action, re-survey, or further investigation. State whether the drainage system is acceptable for adoption or handover. Reference any runs that could not be surveyed and the reasons.

# Remedial Actions Required

| No. | Run Ref | Defect / Issue | Remedial Action | Priority | Status |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |

# Attachments

| No. | Document | Reference / File |
| --- | --- | --- |
| 1 | CCTV video footage (USB/DVD/digital) |  |
| 2 | Still image captures of key observations |  |
| 3 | Manhole schedule and condition report |  |
| 4 | Drainage layout drawing (marked up) |  |
| 5 |  |  |

# Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| CCTV Operator |  |  |  |
| Site Engineer |  |  |  |
| Quality Manager |  |  |  |
| Client / Adopting Authority |  |  |  |

# Notes / Additional Comments

Enter any additional notes.

# Document Control - Revision History

| Rev | Date | Description | Author | Approved |
| --- | --- | --- | --- | --- |
| 0 |  | Initial issue |  |  |