GRP/FRP Installation Inspection Record

| Document Ref: | Enter document reference |
| --- | --- |
| Revision: | Enter revision number |
| Inspection Date: | DD/MM/YYYY |
| Project Name: | Enter project name |
| Contract No: | Enter contract number |
| Prepared By: | Enter name and position |
| Approved By: | Enter name and position |

# Installation Details

| Location / Area: | Describe installation location |
| --- | --- |
| Element Type: | Pipe / Tank / Channel / Weir Plate / Grating / Handrail / Cover / Structural Section / Other |
| Drawing Reference: | Enter drawing and detail numbers |
| Specification Clause: | Enter specification reference |
| Manufacturer: | Enter GRP/FRP manufacturer name |
| Product Reference: | Enter product name, model, and reference |
| Resin Type: | Polyester / Vinyl Ester / Epoxy / Phenolic / Other |
| Reinforcement Type: | Chopped strand mat / Woven roving / Unidirectional / Multiaxial |
| Chemical Resistance: | Specify chemicals and concentrations the element will be exposed to |
| UV Resistance: | Required / Not required – gel coat or UV stabiliser specified? |
| Fire Rating: | Specify fire performance requirements if applicable |
| Installer: | Enter installation contractor name |
| ITP Reference: | Enter ITP reference |

## A. Pre-Installation Checks

| No. | Inspection Item | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Material delivery checked against purchase order and approved submittals |  |  |  |  |
| 2 | Manufacturer's test certificates and declarations of conformity received and reviewed |  |  |  |  |
| 3 | Visual inspection of delivered components – no cracks, chips, delamination, or transit damage |  |  |  |  |
| 4 | Gel coat surface intact, consistent colour, no blistering, crazing, or fibre show-through |  |  |  |  |
| 5 | Dimensions checked against drawings and within specified tolerances |  |  |  |  |
| 6 | Wall thickness checked (micrometer or ultrasonic gauge) and within specification |  |  |  |  |
| 7 | Flanges, bolt holes, and connection details match mating components |  |  |  |  |
| 8 | Components stored correctly – flat, supported, protected from UV and impact damage |  |  |  |  |
| 9 | Method statement reviewed and approved for the installation activity |  |  |  |  |
| 10 | Substrate or supporting structure prepared and ready to receive GRP/FRP elements |  |  |  |  |

## B. Installation Quality

| No. | Inspection Item | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 11 | Components installed in the correct orientation and position per drawings |  |  |  |  |
| 12 | Alignment and level correct within specified tolerances |  |  |  |  |
| 13 | Bolted connections use correct fixings (stainless steel / GRP bolts as specified) |  |  |  |  |
| 14 | All bolt holes drilled cleanly with no delamination or fibre breakout at edges |  |  |  |  |
| 15 | Gaskets, seals, and O-rings installed at all flanged joints |  |  |  |  |
| 16 | Flanged joints torqued to manufacturer's recommended values (star pattern) |  |  |  |  |
| 17 | Adhesive bonded joints prepared and cured as per manufacturer's instructions |  |  |  |  |
| 18 | Laminated joints (hand lay-up on site) cured correctly with no air voids or dry patches |  |  |  |  |
| 19 | Cut edges sealed with resin to prevent moisture ingress into laminate |  |  |  |  |
| 20 | GRP pipework supported at correct centres with approved GRP or lined supports |  |  |  |  |
| 21 | Expansion joints, guides, and anchors installed where specified |  |  |  |  |
| 22 | No excessive bending, twisting, or stress induced during installation |  |  |  |  |

## C. Post-Installation and Testing

| No. | Inspection Item | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 23 | Visual inspection of completed installation – no visible damage or defects |  |  |  |  |
| 24 | Hydrostatic or pneumatic pressure test completed where required |  |  |  |  |
| 25 | Leak test passed on all joints and connections |  |  |  |  |
| 26 | Barcol hardness test carried out on site-laminated joints (minimum value per specification) |  |  |  |  |
| 27 | Spark test or holiday test on internal linings where specified |  |  |  |  |
| 28 | All test results recorded and within acceptance criteria |  |  |  |  |
| 29 | Touch-up and repair of any minor damage completed to manufacturer's procedure |  |  |  |  |
| 30 | GRP grating and covers loaded and checked for deflection within limits |  |  |  |  |
| 31 | Handrails and safety barriers load-tested where required |  |  |  |  |
| 32 | As-built records and photographs taken of completed installation |  |  |  |  |

# Test Records

| No. | Test Description | Specified Value | Actual Value | Location | Pass/Fail | Tester |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |

# Inspection Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| GRP Installer Supervisor |  |  |  |
| Site Engineer |  |  |  |
| Quality Inspector |  |  |  |
| Client / Supervisor |  |  |  |

# Notes / Additional Comments

Enter additional notes. Attach photographs, manufacturer certificates, and test results.

# Document Control - Revision History

| Rev | Date | Description | Author | Approved |
| --- | --- | --- | --- | --- |
| 0 |  | Initial issue |  |  |