Paint and Coating System Specification Checklist

| Document Ref: | Enter document reference |
| --- | --- |
| Inspection Date: | DD/MM/YYYY |
| Project Name: | Enter project name |
| Contract No: | Enter contract number |
| Inspected By: | Enter name and position |
| Approved By: | Enter name and position |

# Coating System Details

| Location / Elements: | Describe elements being coated |
| --- | --- |
| Substrate: | Carbon steel / Stainless steel / Galvanised / Concrete / Timber / Other |
| Corrosivity Category: | C1-C5 / CX / Im1-Im4 per BS EN ISO 12944-2 |
| Design Life: | Low (2-5yr) / Medium (5-15yr) / High (15-25yr) / Very High (>25yr) |
| Drawing Reference: | Enter drawing reference |
| Specification Clause: | Enter specification reference |
| Paint Manufacturer: | Enter paint system manufacturer |
| Paint Applicator: | Enter applicator company name |
| Applicator Approved?: | Yes / No – manufacturer-approved applicator? |

# Coating System Schedule

| Coat | Product Name / Ref | Colour / RAL | Spec DFT (µm) | No. of Coats | Application Method |
| --- | --- | --- | --- | --- | --- |
| Surface Preparation |  |  |  |  |  |
| Primer |  |  |  |  |  |
| Intermediate / Build Coat |  |  |  |  |  |
| Finish / Topcoat |  |  |  |  |  |
| Intumescent (if applicable) |  |  |  |  |  |
| Topcoat over Intumescent |  |  |  |  |  |

## A. Surface Preparation

| No. | Check Item | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Surface preparation grade specified and achieved (e.g., Sa 2.5 per BS EN ISO 8501-1) |  |  |  |  |
| 2 | Abrasive blast profile within specified range (measured with comparator or replica tape) |  |  |  |  |
| 3 | Surface free from oil, grease, and contaminants (solvent wipe per BS EN ISO 12944-4) |  |  |  |  |
| 4 | Mill scale, rust, and old coatings fully removed to specified standard |  |  |  |  |
| 5 | Weld spatter, sharp edges, and surface irregularities dressed before coating |  |  |  |  |
| 6 | Surface cleanliness verified visually and with dust tape test if required |  |  |  |  |
| 7 | Environmental conditions met: steel temperature >3°C above dew point |  |  |  |  |
| 8 | Relative humidity within manufacturer's limits (typically <85%) |  |  |  |  |
| 9 | Ambient temperature within manufacturer's application range |  |  |  |  |

## B. Coating Application

| No. | Check Item | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 10 | Paint batch numbers, manufacture dates, and shelf life checked and recorded |  |  |  |  |
| 11 | Paint mixed and thinned in accordance with manufacturer's data sheet |  |  |  |  |
| 12 | Stripe coating applied to edges, welds, bolt heads, and areas of difficult access |  |  |  |  |
| 13 | Each coat applied within specified overcoating intervals (min and max) |  |  |  |  |
| 14 | Wet film thickness (WFT) checked during application with wet film gauge |  |  |  |  |
| 15 | Dry film thickness (DFT) measured after curing with calibrated electronic gauge |  |  |  |  |
| 16 | DFT readings taken at required frequency (e.g., 5 readings per m²) |  |  |  |  |
| 17 | No runs, sags, holidays, pinholes, or dry spray visible |  |  |  |  |
| 18 | Finish colour and gloss match specification and approved samples |  |  |  |  |
| 19 | Intumescent coating (if applicable) applied to correct DFT for fire rating required |  |  |  |  |

# DFT Measurement Records

| No. | Element / Location | Coat | Spec Min (µm) | R1 | R2 | R3 | Average | Pass/Fail |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  |  |

## D. Post-Application Checks

| No. | Check Item | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 20 | All specified coats applied and DFT within acceptable range |  |  |  |  |
| 21 | Holiday / pinhole detection test carried out (low-voltage wet sponge or high-voltage spark) |  |  |  |  |
| 22 | Adhesion test carried out where specified (cross-cut or pull-off per BS EN ISO 4624) |  |  |  |  |
| 23 | Touch-up of damaged areas completed with same system and method |  |  |  |  |
| 24 | Curing times observed before handling, transport, or installation |  |  |  |  |
| 25 | Coated items protected during transport and installation |  |  |  |  |
| 26 | Site touch-up and repair of any transit or erection damage completed |  |  |  |  |
| 27 | Photographic record taken of completed coating system |  |  |  |  |
| 28 | Manufacturer's warranty documentation completed and submitted |  |  |  |  |

# Inspection Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Coating Inspector |  |  |  |
| Site Engineer |  |  |  |
| Paint Applicator Supervisor |  |  |  |
| Quality Manager |  |  |  |
| Client / Supervisor |  |  |  |

# Notes / Additional Comments

Enter additional notes. Attach paint manufacturer's data sheets, DFT gauge calibration certificates, and environmental condition records.

# Document Control - Revision History

| Rev | Date | Description | Author | Approved |
| --- | --- | --- | --- | --- |
| 0 |  | Initial issue |  |  |