Structural Connection Inspection Record

| Document Ref: | Enter document reference |
| --- | --- |
| Revision: | Enter revision number |
| Inspection Date: | DD/MM/YYYY |
| Project Name: | Enter project name |
| Contract No: | Enter contract number |
| Prepared By: | Enter name and position |
| Approved By: | Enter name and position |

# Connection Details

| Connection Reference: | Enter connection ID from structural drawings |
| --- | --- |
| Location / Gridline: | Enter gridline reference and level |
| Drawing Reference: | Enter structural drawing and detail number |
| Connection Type: | Bolted / Welded / Bolted and Welded / Pinned / Moment / Splice / Base Plate |
| Structural Elements: | Describe elements being connected (e.g., UB 457x191x67 to UC 305x305x97) |
| Execution Class: | EXC1 / EXC2 / EXC3 / EXC4 per BS EN 1090-2 |
| Steel Grade: | e.g., S275JR, S355J2, S355J0 |
| Steelwork Contractor: | Enter steelwork contractor name |
| ITP Reference: | Enter ITP reference for this activity |

## A. Bolted Connection Inspection

| No. | Inspection Item | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Bolt grade and size correct as per drawing (e.g., M20 Grade 8.8, M24 Grade 10.9) |  |  |  |  |
| 2 | Correct number of bolts installed in each group |  |  |  |  |
| 3 | Bolt orientation consistent (head up for vertical, head accessible side for horizontal) |  |  |  |  |
| 4 | Washers fitted correctly (hardened washers under rotating component) |  |  |  |  |
| 5 | All bolts visually tightened – no finger-tight or loose bolts |  |  |  |  |
| 6 | Pre-loaded (HSFG) bolts torqued to specified value and method used recorded |  |  |  |  |
| 7 | Torque wrench calibration certificate available and current |  |  |  |  |
| 8 | Bolt projections through nut within acceptable range (1-4 threads typically) |  |  |  |  |
| 9 | Hole alignment correct – no evidence of drift pins enlarging holes beyond tolerance |  |  |  |  |
| 10 | Packing plates fitted where specified, correct thickness and grade |  |  |  |  |
| 11 | No galvanic isolation issues between dissimilar metals |  |  |  |  |

## Bolt Torque / Tension Records

| Bolt No. | Size | Grade | Method | Torque (Nm) | Actual (Nm) | Pass/Fail | Initials |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |  |

## B. Welded Connection Inspection

| No. | Inspection Item | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 12 | Weld size and type correct as per drawing (fillet, butt, partial penetration) |  |  |  |  |
| 13 | Weld length matches specification |  |  |  |  |
| 14 | Weld profile smooth with no undercut, porosity, or surface cracks |  |  |  |  |
| 15 | No visible defects: spatter, arc strikes, or incomplete fusion |  |  |  |  |
| 16 | Weld preparation (root gap, bevel angle) was as specified prior to welding |  |  |  |  |
| 17 | Welding procedure specification (WPS) available and followed |  |  |  |  |
| 18 | Welder qualifications (BS EN ISO 9606) verified and current |  |  |  |  |
| 19 | NDT carried out where specified (UT, MPI, DPI) and results satisfactory |  |  |  |  |
| 20 | Preheat and interpass temperature requirements observed (if specified) |  |  |  |  |
| 21 | Post-weld heat treatment carried out where specified |  |  |  |  |
| 22 | Weld consumables correct grade and within hydrogen classification |  |  |  |  |

## C. General Connection and Fitment

| No. | Inspection Item | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 23 | Members correctly aligned and plumb/level within tolerance |  |  |  |  |
| 24 | Connection geometry matches the approved shop/fabrication drawings |  |  |  |  |
| 25 | Bearing surfaces clean, flat, and in full contact |  |  |  |  |
| 26 | Shims and packing in correct locations and secured |  |  |  |  |
| 27 | Grouting of base plates complete (where applicable) and cured |  |  |  |  |
| 28 | Fire protection to connection specified and programmed |  |  |  |  |
| 29 | Corrosion protection (intumescent, galvanising, paint) undamaged |  |  |  |  |
| 30 | Touch-up painting carried out where protection system has been damaged |  |  |  |  |
| 31 | Temporary erection bolts or connections removed (if required) |  |  |  |  |
| 32 | Connection accessible for future inspection and maintenance |  |  |  |  |

# Dimensional Check Records

| No. | Check Description | Specified | Actual | Tolerance | Pass/Fail |
| --- | --- | --- | --- | --- | --- |
| 1 | Column plumb (mm per m) |  |  |  |  |
| 2 | Beam level (mm over span) |  |  |  |  |
| 3 | Connection plate position |  |  |  |  |
| 4 | Bolt group spacing |  |  |  |  |
| 5 | Base plate level |  |  |  |  |
| 6 | Beam-to-column eccentricity |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |

# Inspection Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Steel Erector Supervisor |  |  |  |
| Site Engineer |  |  |  |
| Structural Inspector |  |  |  |
| Client / Supervisor Representative |  |  |  |

# Notes / Additional Comments

Enter additional notes. Attach photographs, NDT reports, and torque wrench calibration certificates.

# Document Control - Revision History

| Rev | Date | Description | Author | Approved |
| --- | --- | --- | --- | --- |
| 0 |  | Initial issue |  |  |