Waterproofing System Inspection Record

| Document Ref: | Enter document reference |
| --- | --- |
| Revision: | Enter revision number |
| Inspection Date: | DD/MM/YYYY |
| Project Name: | Enter project name |
| Contract No: | Enter contract number |
| Prepared By: | Enter name and position |
| Approved By: | Enter name and position |

# Waterproofing System Details

| Location / Area: | Describe area (e.g., basement slab, retaining wall, roof, wet room, podium deck) |
| --- | --- |
| Drawing Reference: | Enter applicable drawing number(s) |
| Specification Clause: | Enter specification reference |
| BS 8102 Grade: | Grade 1 (Basic) / Grade 2 (Utility) / Grade 3 (Habitable) per BS 8102:2009 |
| Waterproofing Type: | Type A (Barrier) / Type B (Structurally Integral) / Type C (Drained) / Combined |
| System Manufacturer: | Enter manufacturer name |
| Product Name / Ref: | Enter product name and reference |
| Membrane Type: | Sheet / Liquid Applied / Cementitious / Bentonite / Bituminous / Other |
| Application Method: | Torch-on / Self-adhesive / Spray-applied / Brush / Trowel / Mechanically fixed |
| Number of Coats / Layers: | Enter number specified |
| Minimum DFT (mm): | Enter minimum dry film thickness |
| Installer / Applicator: | Enter company name |
| Installer Approved?: | Yes / No – manufacturer-approved applicator |
| Warranty Period: | Enter warranty period (years) |

## A. Substrate Preparation

| No. | Inspection Item | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Substrate concrete is cured for the minimum period specified by the waterproofing manufacturer |  |  |  |  |
| 2 | Substrate is clean, dry, and free from dust, oil, grease, laitance, and loose material |  |  |  |  |
| 3 | Surface profile is suitable for the specified system (smooth, roughened, or as manufacturer requires) |  |  |  |  |
| 4 | All honeycombing, voids, and surface defects in the concrete have been repaired and made good |  |  |  |  |
| 5 | Construction joints have been prepared and treated as specified |  |  |  |  |
| 6 | Internal corners and fillets formed where required (typically 50mm radius or as specified) |  |  |  |  |
| 7 | Pipes, services, and penetrations are prepared with appropriate detailing |  |  |  |  |
| 8 | Primer applied where required by manufacturer, at correct coverage rate |  |  |  |  |
| 9 | Moisture content of substrate checked and within manufacturer's limits |  |  |  |  |

## B. Membrane / Coating Application

| No. | Inspection Item | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 10 | Material batch numbers and expiry dates checked and recorded |  |  |  |  |
| 11 | Material stored and handled in accordance with manufacturer's requirements |  |  |  |  |
| 12 | Ambient and substrate temperature within manufacturer's recommended range during application |  |  |  |  |
| 13 | Application carried out in dry conditions (no rain, frost, or standing water) |  |  |  |  |
| 14 | First coat / layer applied evenly and to correct thickness |  |  |  |  |
| 15 | Minimum overcoat time observed between coats / layers |  |  |  |  |
| 16 | Subsequent coats applied at right angles to previous coat (where applicable) |  |  |  |  |
| 17 | Dry film thickness (DFT) checked with gauge and recorded |  |  |  |  |
| 18 | Lap joints meet minimum overlap specified by manufacturer (typically 100-150mm) |  |  |  |  |
| 19 | All laps bonded and free from wrinkles, air pockets, or lifting |  |  |  |  |
| 20 | Sheet membrane fully bonded or mechanically fixed as specified |  |  |  |  |

## C. Details, Penetrations, and Terminations

| No. | Inspection Item | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 21 | Waterstops installed at construction joints as specified |  |  |  |  |
| 22 | Pipe and service penetrations detailed with proprietary collars, boots, or wrapping |  |  |  |  |
| 23 | Movement joints detailed with flexible membrane or bellows as designed |  |  |  |  |
| 24 | Termination details correct (chase, mechanical fixing, termination bar as specified) |  |  |  |  |
| 25 | Internal and external corners reinforced with additional membrane or fabric strip |  |  |  |  |
| 26 | Upstand heights meet minimum requirements (typically 150mm above finished level) |  |  |  |  |
| 27 | Day joints in membrane application properly formed and lapped |  |  |  |  |
| 28 | All details match the manufacturer's approved detail drawings |  |  |  |  |

## D. Testing and Verification

| No. | Inspection Item | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 29 | Visual inspection completed – no visible defects, tears, or disbondment |  |  |  |  |
| 30 | DFT measurements taken at agreed frequency and all readings within specification |  |  |  |  |
| 31 | Flood test carried out where required (duration and depth as specified) |  |  |  |  |
| 32 | Spark / holiday testing carried out on liquid-applied membranes (where specified) |  |  |  |  |
| 33 | Bond / adhesion test carried out where specified |  |  |  |  |
| 34 | Photographic record taken of completed waterproofing before protection / covering |  |  |  |  |
| 35 | All test results documented and within acceptance criteria |  |  |  |  |

## E. Protection and Completion

| No. | Inspection Item | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 36 | Protection board or fleece applied over membrane before backfill or screed |  |  |  |  |
| 37 | Protection board type and thickness as specified |  |  |  |  |
| 38 | Drainage layer (cavity drain or geo-composite) installed where specified (Type C systems) |  |  |  |  |
| 39 | Sump and pump systems installed and tested (where applicable) |  |  |  |  |
| 40 | No damage to membrane during protection installation or subsequent works |  |  |  |  |
| 41 | Backfill carried out carefully with no sharp objects in contact with membrane |  |  |  |  |
| 42 | Manufacturer's site inspection and approval obtained (where required for warranty) |  |  |  |  |

# Dry Film Thickness (DFT) Readings

| No. | Location | Specified Min (mm) | Reading 1 | Reading 2 | Reading 3 | Pass/Fail |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |

# Inspection Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Waterproofing Installer |  |  |  |
| Site Engineer |  |  |  |
| Quality Manager |  |  |  |
| Manufacturer's Representative |  |  |  |
| Client / Designer |  |  |  |

# Notes / Additional Comments

Enter additional notes, attach photographs, material certificates, or manufacturer's inspection reports.

# Document Control - Revision History

| Rev | Date | Description | Author | Approved |
| --- | --- | --- | --- | --- |
| 0 |  | Initial issue |  |  |