MEP Snagging Checklist

| Document Ref: | Enter document reference |
| --- | --- |
| Inspection Date: | DD/MM/YYYY |
| Project Name: | Enter project name |
| Contract No: | Enter contract number |
| Building / Zone: | Enter building, block, or zone reference |
| Floor / Level: | Enter floor or level |
| Room / Area: | Enter room or area reference |
| MEP Subcontractor: | Enter MEP subcontractor name(s) |
| Inspected By: | Enter name and position |
| Accompanied By: | Enter names and organisations |

## A. Electrical Installation - General

| No. | Inspection Item | Pass | Fail | N/A | Snag Ref | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Distribution boards correctly labelled with circuit schedules visible and accurate |  |  |  |  |  |
| 2 | All circuits identified and labelled at DB and at point of use |  |  |  |  |  |
| 3 | Cable containment lids fitted, covers in place, trunking complete |  |  |  |  |  |
| 4 | Cable tray and basket runs straight, level, properly supported at correct centres |  |  |  |  |  |
| 5 | No exposed or unterminated cables visible |  |  |  |  |  |
| 6 | Earthing and bonding connections complete and tight |  |  |  |  |  |
| 7 | Electrical test certificates available (BS 7671 compliance) |  |  |  |  |  |
| 8 | All circuit breakers and RCDs tested and operational |  |  |  |  |  |

## B. Electrical - Switches, Sockets, and Fittings

| No. | Inspection Item | Pass | Fail | N/A | Snag Ref | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 9 | Light switches level, plumb, and at correct height |  |  |  |  |  |
| 10 | Socket outlets level, undamaged, and operational (polarity confirmed) |  |  |  |  |  |
| 11 | Switch plates and socket faceplates consistent finish, no scratches or damage |  |  |  |  |  |
| 12 | Dimmer switches (where fitted) operating across full range |  |  |  |  |  |
| 13 | Light fittings correctly positioned, aligned, clean lenses, operational |  |  |  |  |  |
| 14 | Emergency lighting installed, tested, and certificate available |  |  |  |  |  |
| 15 | Exit signs illuminated and correctly positioned |  |  |  |  |  |
| 16 | Data and comms outlets installed, terminated, and tested |  |  |  |  |  |
| 17 | Isolator switches accessible, labelled, and operational |  |  |  |  |  |

## C. Fire Alarm and Detection Systems

| No. | Inspection Item | Pass | Fail | N/A | Snag Ref | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 18 | Fire alarm panel operational, tested, and cause-and-effect verified |  |  |  |  |  |
| 19 | Smoke and heat detectors correctly positioned per BS 5839 |  |  |  |  |  |
| 20 | Manual call points at correct locations and heights, operational |  |  |  |  |  |
| 21 | Sounders audible in all areas at required dB level |  |  |  |  |  |
| 22 | Visual alarm devices (VADs) fitted where specified |  |  |  |  |  |
| 23 | Fire alarm zoning correct and zone plan displayed at panel |  |  |  |  |  |
| 24 | Interfaced systems (door holders, dampers, lifts) tested |  |  |  |  |  |
| 25 | Fire alarm certificate and commissioning record available |  |  |  |  |  |

## D. Mechanical - Heating, Ventilation, and Air Conditioning

| No. | Inspection Item | Pass | Fail | N/A | Snag Ref | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 26 | Radiators / emitters level, securely fixed, valves accessible and operational |  |  |  |  |  |
| 27 | Radiators bled, system filled, balanced, and reaching design temperature |  |  |  |  |  |
| 28 | TRVs fitted and set correctly where specified |  |  |  |  |  |
| 29 | Ductwork joints sealed, no visible air leakage |  |  |  |  |  |
| 30 | Ductwork insulation complete, continuous, vapour barrier intact |  |  |  |  |  |
| 31 | Grilles and diffusers level, clean, undamaged, correctly sized |  |  |  |  |  |
| 32 | Air handling units operational, filters clean, access panels sealed |  |  |  |  |  |
| 33 | Fan coil units operational, controls functioning, condensate draining |  |  |  |  |  |
| 34 | Commissioning certificates available showing designed air flow rates achieved |  |  |  |  |  |
| 35 | BMS operational and all set points programmed |  |  |  |  |  |
| 36 | Pipework lagged, labelled with flow direction and service identification |  |  |  |  |  |

## E. Plumbing and Drainage

| No. | Inspection Item | Pass | Fail | N/A | Snag Ref | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 37 | Hot and cold water supplies connected, flowing, and at correct temperature |  |  |  |  |  |
| 38 | TMVs set and tested to correct outlet temperature |  |  |  |  |  |
| 39 | Sanitary ware securely fixed, undamaged, clean, and operational |  |  |  |  |  |
| 40 | Taps and mixers operational, no leaks, hot on left |  |  |  |  |  |
| 41 | Waste and overflow connections watertight under running water test |  |  |  |  |  |
| 42 | Toilet cisterns filling and flushing correctly, seats secure |  |  |  |  |  |
| 43 | Urinals operational and flushing correctly |  |  |  |  |  |
| 44 | Showers operational, thermostatic valves set, screens sealed |  |  |  |  |  |
| 45 | Floor drainage gullies and waste outlets clear and draining |  |  |  |  |  |
| 46 | Expansion vessels and pressure relief valves installed and tested |  |  |  |  |  |
| 47 | Water treatment (if applicable) installed and operational |  |  |  |  |  |
| 48 | Legionella risk assessment controls in place |  |  |  |  |  |

## F. Fire Protection Systems

| No. | Inspection Item | Pass | Fail | N/A | Snag Ref | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 49 | Sprinkler heads correctly positioned, undamaged, correct type per zone |  |  |  |  |  |
| 50 | Sprinkler system commissioned, tested, and certificates available |  |  |  |  |  |
| 51 | Dry riser / wet riser tested and certificate available |  |  |  |  |  |
| 52 | Fire extinguishers installed in correct locations and types |  |  |  |  |  |
| 53 | Fire dampers installed at all fire compartment penetrations and tested |  |  |  |  |  |
| 54 | Fire stopping complete at all service penetrations (both sides of barrier) |  |  |  |  |  |
| 55 | Fire stopping third-party certificated installer records available |  |  |  |  |  |

## G. Lift Installation (If Applicable)

| No. | Inspection Item | Pass | Fail | N/A | Snag Ref | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 56 | Lift car finishes clean, undamaged, and to specification |  |  |  |  |  |
| 57 | Lift doors operating correctly on all floors, no excessive gaps |  |  |  |  |  |
| 58 | Lift car and landing indicators and buttons operational |  |  |  |  |  |
| 59 | Emergency communication (phone/intercom) tested and operational |  |  |  |  |  |
| 60 | Lift commissioned and LOLER thorough examination certificate available |  |  |  |  |  |
| 61 | Lift shaft clean, lighting operational, ladder access safe |  |  |  |  |  |

## H. General MEP Quality

| No. | Inspection Item | Pass | Fail | N/A | Snag Ref | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 62 | All pipework, ductwork, and containment labelled and colour-coded per BS 1710 |  |  |  |  |  |
| 63 | Valve and damper tags fitted and correspond to schedule |  |  |  |  |  |
| 64 | Access panels fitted flush, operable, and providing adequate access |  |  |  |  |  |
| 65 | Plant room clean, equipment labelled, safety signage in place |  |  |  |  |  |
| 66 | All penetrations through fire-rated elements sealed with fire stopping |  |  |  |  |  |
| 67 | Ceiling tiles correctly seated after final fix MEP works |  |  |  |  |  |
| 68 | All commissioning certificates complete and available |  |  |  |  |  |
| 69 | O&M manuals drafted and submitted for review |  |  |  |  |  |
| 70 | Spare parts and specialist tools handed over as specified |  |  |  |  |  |

# Snag Summary

| Total Items Inspected: |  |
| --- | --- |
| Total Pass: |  |
| Total Fail (Snags): |  |
| Total N/A: |  |
| Area Status: | Ready for Handover / Snags Outstanding / Re-Inspection Required |
| Re-Inspection Date: | DD/MM/YYYY |

# Inspection Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| MEP Inspector |  |  |  |
| MEP Subcontractor Rep |  |  |  |
| Site Manager |  |  |  |
| Client Representative |  |  |  |

# Notes / Additional Comments

Enter additional notes and photograph references.

# Document Control - Revision History

| Rev | Date | Description | Author | Approved |
| --- | --- | --- | --- | --- |
| 0 |  | Initial issue |  |  |