Post-Weld Heat Treatment Record

Record of post-weld heat treatment (PWHT / stress relief) for weld joints requiring thermal treatment after welding. Completed as a permanent quality record with the time-temperature chart appended. In accordance with BS EN 13480-4 (Metallic industrial piping — Fabrication and installation), ASME B31.3, PD 5500, BS EN 1090-2, and the approved PWHT procedure.

Project and Certificate Details

| Document Ref: | Enter document reference |
| --- | --- |
| PWHT Certificate No.: | Enter unique PWHT certificate number |
| Revision: | Enter revision number |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| PWHT Contractor / Operator: | Enter heat treatment company name |
| Prepared By: | Name / role |
| Approved By: | Name / role |

Weld Joint and Material Details

| Weld Joint Number(s): | Enter weld joint number(s) subject to PWHT |
| --- | --- |
| ISO / Drawing Reference: | Enter isometric or GA drawing reference |
| System / Line Reference: | Enter pipeline or system designation |
| Component / Equipment: | Enter description of component (e.g. pipe spool, vessel, structural connection) |
| Material Specification: | Enter parent material standard and grade |
| Material Group (BS EN ISO 15608): | Enter material group number |
| Wall Thickness (mm): | Enter wall thickness at weld joint |
| Pipe OD / Component Size: | Enter outside diameter or component dimensions |
| WPS Reference: | Enter WPS number requiring PWHT |
| Welding Process Used: | e.g. 141 (GTAW), 111 (SMAW), 135 (MAG) |

PWHT Parameters (As Specified)

| PWHT Procedure Reference: | Enter approved PWHT procedure document reference |
| --- | --- |
| PWHT Standard: | e.g. BS EN 13480-4, ASME B31.3, PD 5500, BS EN 1090-2, AWS D1.1 |
| Heating Method: | e.g. electric resistance pads, induction coil, furnace |
| Heating Equipment ID: | Enter equipment identification / serial number |
| Number of Thermocouples: | Enter number of TCs attached to the joint |
| Thermocouple Type: | e.g. Type K (chromel-alumel), Type J, Type N |
| Thermocouple Locations: | Describe TC positions (e.g. 12, 3, 6, 9 o'clock; weld centre, HAZ, parent) |
| Data Recorder ID: | Enter chart recorder or data logger identification |
| Recorder Calibration Date: | DD / MM / YYYY |
| Insulation Type and Thickness: | e.g. ceramic fibre blanket, 50mm thick, extending 3x thickness each side of weld |
| Heating Rate (deg C/hr): | Enter maximum permitted heating rate (typically 55-200 deg C/hr depending on thickness) |
| Soaking Temperature Range (deg C): | Enter required temperature range (e.g. 580-620 deg C for C-Mn steel) |
| Soaking Time at Temperature: | Enter required hold time (e.g. 1 hour per 25mm thickness, minimum 30 minutes) |
| Cooling Rate (deg C/hr): | Enter maximum permitted cooling rate (typically controlled to 250 deg C then free air) |
| Temperature Gradient Limit (deg C): | Enter maximum permitted temperature difference between TCs during soak |

Actual PWHT Results

| PWHT Start Date / Time: | DD / MM / YYYY at HH:MM |
| --- | --- |
| PWHT End Date / Time: | DD / MM / YYYY at HH:MM |
| Total Cycle Duration: | Enter total hours from start of heat-up to end of controlled cool |
| Actual Heating Rate (deg C/hr): | Enter actual rate achieved |
| Actual Soaking Temperature (deg C): | Enter actual temperature range recorded across all TCs |
| Actual Soaking Time: | Enter actual duration at soaking temperature |
| Maximum Temperature Recorded (deg C): | Enter peak temperature from any TC |
| Minimum Temperature During Soak (deg C): | Enter lowest TC reading during soak period |
| Temperature Gradient During Soak (deg C): | Enter maximum difference between TCs during soak |
| Actual Cooling Rate (deg C/hr): | Enter actual cooling rate achieved |
| Temperature at Which Insulation Removed: | Enter temperature at which cooling became uncontrolled |
| Chart / Data Log Reference: | Enter time-temperature chart or digital data file reference |
| PWHT Cycle Within Specification: | Yes / No — confirm all parameters met WPS and code requirements |

Post-PWHT Testing

Record hardness testing and any NDT required after PWHT. Hardness testing is mandatory per most codes to confirm stress relief has been achieved.

| Hardness Test Required After PWHT: | Yes / No — if yes, enter results below |
| --- | --- |
| Hardness Test Method: | e.g. HV10 (Vickers), HB (Brinell), HRC (Rockwell) |
| Hardness Values — Weld Metal: | Enter values (e.g. 180, 175, 185 HV10) |
| Hardness Values — HAZ: | Enter values (e.g. 195, 200, 190 HV10) |
| Hardness Values — Parent Metal: | Enter values (e.g. 160, 165, 155 HV10) |
| Maximum Acceptable Hardness: | Enter per specification (e.g. 250 HV10 for C-Mn steel per BS EN ISO 15614-1) |
| Hardness Test Result: | Pass / Fail |
| NDT After PWHT: | Enter NDT required and report reference (e.g. UT, MPI per specification) |
| NDT Result: | Acceptable / Not Acceptable |

Overall PWHT Result

| Overall PWHT Result: | Pass / Fail |
| --- | --- |
| Non-Conformances: | Enter any deviations from specified parameters and NCR ref if raised |
| Remedial Actions: | Describe any remedial action taken (e.g. re-PWHT, engineering assessment) |

Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| PWHT Operator |  |  | DD / MM / YYYY |
| Welding Inspector (CSWIP / PCN) |  |  | DD / MM / YYYY |
| Welding Coordinator (IWE / IWT) |  |  | DD / MM / YYYY |
| QC Manager / Site Engineer |  |  | DD / MM / YYYY |
| Client / Third Party Inspector |  |  | DD / MM / YYYY |

Appendices

| Appendix A — Time-Temperature Chart: | Attached / Not Attached — enter chart reference |
| --- | --- |
| Appendix B — Thermocouple Location Sketch: | Attached / Not Attached |
| Appendix C — Hardness Test Report: | Attached / Not Applicable |
| Appendix D — NDT Report (Post-PWHT): | Attached / Not Applicable |
| Appendix E — Equipment Calibration Certs: | Attached — recorder, TCs |

Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| A |  | Initial issue |  |  |