| MOTOR ALIGNMENT AND ROTATION CHECK |  |
| --- | --- |
| Document Ref: | Enter document reference |
| Revision: | Enter revision number |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| Prepared By: | Name, position and company |
| Approved By: | Name, position and company |

This record documents motor-to-driven-equipment shaft alignment and rotation direction verification. Correct alignment is critical to equipment life, efficiency and reliability. Alignment must be carried out with calibrated equipment and recorded before the coupling guard is refitted. Rotation must be verified with the motor uncoupled before first start.

1. Equipment and Motor Details

| Equipment Tag Number: |  |
| --- | --- |
| Equipment Description: | e.g. Pump, fan, compressor, gearbox, conveyor drive |
| P&ID Reference: |  |
| Motor Tag Number: |  |
| Motor Manufacturer: |  |
| Motor Model / Frame: |  |
| Motor Serial Number: |  |
| Rated Power (kW): |  |
| Rated Speed (rpm): |  |
| Rated Voltage (V): |  |
| Full Load Current (A): |  |
| Coupling Type: | Flexible jaw / Grid / Disc / Gear / Fluid / Direct (close-coupled) / Other |
| Coupling Manufacturer: |  |
| Coupling Size / Model: |  |
| Required Rotation Direction: | CW / CCW (viewed from drive end) |
| Drawing Reference: | Enter GA or installation drawing |

2. Pre-Alignment Checks

| No. | Pre-Alignment Check | Done | N/A | Comments |
| --- | --- | --- | --- | --- |
| 1 | Motor and driven equipment baseplates level and grouted (grout cured) |  |  |  |
| 2 | Hold-down bolts tightened to specification |  |  |  |
| 3 | Soft foot check completed — all four motor feet bearing evenly (max 0.05mm) |  |  |  |
| 4 | Coupling halves clean and free from damage |  |  |  |
| 5 | Coupling element / insert in good condition and correct specification |  |  |  |
| 6 | Coupling gap set per manufacturer specification |  |  |  |
| 7 | Piping strain check — no excessive pipe loads on equipment flanges |  |  |  |
| 8 | All shims clean, flat and free from burrs |  |  |  |
| 9 | Motor free to slide on baseplate for adjustment |  |  |  |
| 10 | Driven equipment shaft turns freely by hand |  |  |  |
| 11 | Motor shaft turns freely by hand |  |  |  |

3. Alignment Method and Equipment

| Alignment Method: | Laser alignment / Dial gauge (rim and face) / Reverse indicator / Straight edge (rough only) |
| --- | --- |
| Alignment Tool Make / Model: |  |
| Serial Number: |  |
| Calibration Date: | DD / MM / YYYY |
| Calibration Certificate Ref: |  |
| Thermal Growth Compensation Applied: | Yes / No (if Yes, state offset values — vertical and horizontal) |
| Thermal Growth Reference: | Manufacturer data / Calculation / Default formula |

4. Alignment Readings

Record initial (as-found) and final (as-left) readings for offset and angularity in both horizontal and vertical planes.

| Parameter | Horizontal | Vertical | Tolerance | Pass / Fail |
| --- | --- | --- | --- | --- |
| INITIAL READINGS (AS FOUND) |  |  |  |  |
| Offset (parallel misalignment) |  |  |  |  |
| Angularity (angular misalignment) |  |  |  |  |
| Coupling Gap |  |  |  |  |
| Soft Foot (max) |  |  |  |  |
| FINAL READINGS (AS LEFT) |  |  |  |  |
| Offset (parallel misalignment) |  |  |  |  |
| Angularity (angular misalignment) |  |  |  |  |
| Coupling Gap |  |  |  |  |
| Soft Foot (max) |  |  |  |  |

5. Shim Record

| Motor Foot - DE Left: | Enter total shim thickness (mm) and number of shims |
| --- | --- |
| Motor Foot - DE Right: |  |
| Motor Foot - NDE Left: |  |
| Motor Foot - NDE Right: |  |
| Lateral Adjustment: | Enter direction and amount (mm) |
| Maximum Number of Shims per Foot: | Typically max 5 shims per foot (consolidate if more) |

6. Rotation Direction Check

Motor must be uncoupled from driven equipment before performing bump test. Confirm rotation matches the required direction shown on the equipment drawing / P&ID.

| Rotation Check Method: | Phase rotation meter / Bump test / VSD parameter check |
| --- | --- |
| Expected Direction: | CW / CCW (viewed from drive end) |
| Observed Direction: | CW / CCW |
| Rotation Correct: | Yes / No |
| If Incorrect, Action Taken: | Swap two phases at motor terminals / VSD parameter change |
| Phase Sequence Verified (3-phase): | L1-L2-L3 confirmed |
| Bump Test Witnessed By: | Name |
| Motor Uncoupled for Bump Test: | Yes / No (motor must be uncoupled before bump test) |

7. Overall Result

| Alignment Result: | Within tolerance / Out of tolerance (state action) |
| --- | --- |
| Rotation Correct: | Yes / No |
| Coupling Guard Refitted: | Yes / No |
| Alignment Report Attached: | Yes / No (attach laser printout or dial gauge readings) |
| Ready for Commissioning: | Yes / No |
| NCR Raised: | Yes / No (reference) |

8. Sign-Off

| Role | Print Name | Signature | Date |
| --- | --- | --- | --- |
| Alignment Technician / Fitter |  |  |  |
| Mechanical Engineer |  |  |  |
| Electrical Engineer |  |  |  |
| Commissioning Engineer |  |  |  |
| Client / Engineer's Representative |  |  |  |

9. Notes / Additional Comments

Enter any additional notes or attach laser alignment report printout

10. Document Control

| Rev | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |

References: Manufacturer's Installation Manual, BS 3790 (belt drives), ISO 10816 (vibration), Laser Alignment Tool Manual, Project Specification, ITP