| VALVE INSTALLATION AND TEST RECORD |  |
| --- | --- |
| Document Ref: | Enter document reference |
| Revision: | Enter revision number |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| Prepared By: | Name, position and company |
| Approved By: | Name, position and company |

This record documents the installation and testing of each valve on the project. It must be completed for every valve listed on the valve schedule. Valve testing must comply with BS EN 12266 or API 598 as specified. Actuated valves must have functional tests completed before the system is placed into service.

1. Valve Details

| Valve Tag Number: |  |
| --- | --- |
| P&ID Reference: | Enter P&ID drawing number and revision |
| Line / Pipeline Reference: |  |
| Valve Type: | Gate / Globe / Ball / Butterfly / Check (swing/wafer) / Plug / Diaphragm / Knife gate / Pressure reducing / Other |
| Valve Manufacturer: |  |
| Model Number: |  |
| Serial Number: |  |
| Nominal Size (DN): | Enter mm |
| Pressure Rating: | PN6 / PN10 / PN16 / PN25 / PN40 / Class 150 / Class 300 / Other |
| Body Material: | Cast iron / Ductile iron / Carbon steel / Stainless steel / Bronze / PVC / HDPE / Other |
| Trim Material: |  |
| End Connection: | Flanged / Wafer / Screwed / Butt weld / Socket weld / Compression |
| Actuator Type: | Manual (handwheel/lever) / Electric / Pneumatic / Hydraulic / None |
| Actuator Make / Model: |  |
| Fail Position (actuated): | Fail open / Fail closed / Fail in position / N/A |
| Normal Position: | Normally open / Normally closed |
| Service / Media: | Describe fluid (potable water, sewage, process chemical, gas, steam, etc.) |
| Design Temperature: | Enter °C |
| Design Pressure: | Enter bar |

2. Installation Checks

| No. | Installation Checks | Done | N/A | Comments / Initials |
| --- | --- | --- | --- | --- |
| 1 | Valve received, inspected for transport damage and storage condition verified |  |  |  |
| 2 | Material certificate / test certificate received and verified against specification |  |  |  |
| 3 | Valve tag number and data plate match specification and P&ID |  |  |  |
| 4 | Flow direction arrow checked and valve installed in correct orientation |  |  |  |
| 5 | Flanges aligned, gaskets correctly positioned (new gaskets used) |  |  |  |
| 6 | Bolts installed, tightened to specification in correct sequence (star pattern) |  |  |  |
| 7 | Bolt torque values recorded (attach torque record if required) |  |  |  |
| 8 | Valve stem orientation correct (accessible for operation and maintenance) |  |  |  |
| 9 | Valve opens and closes freely through full travel (manual operation check) |  |  |  |
| 10 | Actuator mounted and aligned correctly |  |  |  |
| 11 | Actuator travel limits set and verified (open and close stops) |  |  |  |
| 12 | Valve position indicator functioning correctly |  |  |  |
| 13 | Pipework supports adjacent to valve adequate (no excessive strain on valve body) |  |  |  |
| 14 | Valve accessible for operation and maintenance (adequate clearance) |  |  |  |
| 15 | Valve labelled / tagged with permanent identification |  |  |  |
| 16 | Bypass and drain connections made (if applicable) |  |  |  |
| 17 | Insulation requirements noted and communicated (if applicable) |  |  |  |

3. Testing and Functional Checks

| No. | Testing and Functional Checks | Done | N/A | Comments / Initials |
| --- | --- | --- | --- | --- |
| 18 | Valve included in pipeline hydrostatic / pressure test (pass confirmed) |  |  |  |
| 19 | Seat leakage test carried out (if required by specification) |  |  |  |
| 20 | Actuator function test — open and close from control system |  |  |  |
| 21 | Actuator torque / thrust adequate for valve operation |  |  |  |
| 22 | Emergency handwheel override tested (actuated valves) |  |  |  |
| 23 | Fail-safe position verified (remove signal / air, confirm fail position) |  |  |  |
| 24 | Position feedback signals verified at control panel / SCADA |  |  |  |
| 25 | Limit switches set and tested (open and closed positions) |  |  |  |
| 26 | Interlock functions tested (if applicable) |  |  |  |
| 27 | Stroke time recorded (open to close and close to open) |  |  |  |

4. Test Results Summary

| Pressure Test Included: | Yes / No (test certificate reference) |
| --- | --- |
| Seat Leakage Rate: | Zero / Within acceptable limit / Failed (per BS EN 12266 / API 598) |
| Actuator Stroke Time (open): | Enter seconds |
| Actuator Stroke Time (close): | Enter seconds |
| Torque at Seating: | Enter Nm (if measured) |
| Position Feedback Verified: | Yes / No |
| Overall Test Result: | PASS / FAIL |
| If Failed, Action Taken: | Describe corrective action, NCR reference |

5. Sign-Off

| Role | Print Name | Signature | Date |
| --- | --- | --- | --- |
| Mechanical Engineer / Installer |  |  |  |
| Commissioning Engineer |  |  |  |
| Quality Manager |  |  |  |
| Client / Engineer's Representative |  |  |  |

6. Notes / Additional Comments

Enter any additional notes

7. Document Control

| Rev | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |

References: BS EN 12266, API 598, BS EN 1074 (water supply valves), BS EN 593 (butterfly valves), BS EN 1984 (steel gate valves), PED 2014/68/EU, Project Specification, ITP