| FLANGE JOINT INTEGRITY CHECKLIST |  |
| --- | --- |
| Document Ref: | Enter document reference |
| Revision: | Enter revision number |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| Prepared By: | Name, position and company |
| Approved By: | Name, position and company |

This checklist documents the assembly of a flanged joint to ensure joint integrity. It must be completed for every safety-critical and pressure-rated flanged joint. Joint assembly must follow ASME PCC-1 guidelines (or equivalent) and be carried out by personnel trained in correct flange assembly procedures. Incorrect assembly is a leading cause of flange leaks.

1. Joint Details

| Joint Tag / Reference: | Enter unique joint identification number |
| --- | --- |
| P&ID Reference: |  |
| Line Number: |  |
| Joint Location: | Describe location on site / within system |
| Nominal Bore (DN): | Enter mm |
| Pressure Rating: | PN10 / PN16 / PN25 / PN40 / Class 150 / Class 300 / Other |
| Flange Type: | Slip-on / Weld neck / Blind / Threaded / Lap joint / Socket weld |
| Flange Standard: | BS EN 1092-1 / ASME B16.5 / BS 4504 / Other |
| Flange Material: | Carbon steel / Stainless steel / Ductile iron / GRP / Other |
| Service / Media: | Describe fluid (water, sewage, gas, chemical, steam, compressed air) |
| Design Pressure (bar): |  |
| Design Temperature (°C): |  |
| Gasket Type: | Full face / Ring type (IBC) / Spiral wound / Kammprofile / PTFE / Rubber / Other |
| Gasket Material: | Non-asbestos fibre / PTFE / Graphite / EPDM / Nitrile / Stainless spiral wound + filler |
| Gasket Standard / Size: | Enter standard reference and dimensions |
| Gasket Manufacturer: |  |
| Gasket Batch / Lot Number: |  |
| Bolt Type: | Stud bolt / Hex head / Through bolt |
| Bolt Material / Grade: | Grade 8.8 / B7 / L7 / A2-70 / A4-80 / Other |
| Bolt Size: | e.g. M16, M20, M24 |
| Number of Bolts: |  |
| Bolt Length: | Enter mm |
| Nut Material / Grade: | Grade 8 / 2H / A2-70 / A4-80 / Other |
| Washers Used: | Yes / No (type: flat / hardened / Nord-Lock) |
| Lubricant Applied: | Yes / No (type: copper slip / MoS2 / Teflon / specified lubricant) |

2. Pre-Assembly Checklist

All items must be verified before bolt tightening commences.

| No. | Pre-Assembly Check | Done | N/A | Initials |
| --- | --- | --- | --- | --- |
| 1 | Flange faces inspected — clean, free from damage, corrosion, scoring and old gasket material |  |  |  |
| 2 | Flange face finish suitable for gasket type (RF, FF, RTJ as required) |  |  |  |
| 3 | Flange alignment checked — parallel within tolerance (max gap variation per ASME PCC-1) |  |  |  |
| 4 | Flange bore alignment checked — concentric (no offset exceeding pipe bore ID tolerance) |  |  |  |
| 5 | Bolt holes aligned without forcing (bolts should pass through freely) |  |  |  |
| 6 | No pipe strain pulling flanges together (piping properly supported) |  |  |  |
| 7 | Gasket is correct type, size and material per specification |  |  |  |
| 8 | Gasket visually inspected — no damage, compression marks or defects |  |  |  |
| 9 | Gasket centred correctly on flange face (spiral wound inner ring centred on bore) |  |  |  |
| 10 | Bolts, nuts and washers correct grade, size and quantity per specification |  |  |  |
| 11 | Bolts, nuts and washers inspected — clean, undamaged threads, no corrosion |  |  |  |
| 12 | Thread lubricant applied per specification (note: torque values must match lubricant condition) |  |  |  |
| 13 | Torque wrench calibrated and within range for specified torque values |  |  |  |
| 14 | Bolt tightening sequence planned (star / cross pattern per ASME PCC-1) |  |  |  |
| 15 | Number of tightening passes planned (minimum 3 passes: hand tight, 30%, 60%, 100%) |  |  |  |
| 16 | Joint identified on system drawing / isometric |  |  |  |

3. Bolt Tightening Record

| Required Torque (Nm): | Enter per specification or ASME PCC-1 / BS EN 1591 |
| --- | --- |
| Pass 1 Torque (Nm): | Enter (typically hand tight or snug) |
| Pass 2 Torque (Nm): | Enter (typically 30–50% of final) |
| Pass 3 Torque (Nm): | Enter (typically 60–70% of final) |
| Pass 4 — Final Torque (Nm): | Enter (100% of specified torque) |
| Final Check Pass: | Yes / No (final pass at 100%, no further bolt movement — 'torque to turn') |
| All Bolts Reached Final Torque: | Yes / No |
| Torque Wrench Used: | Make, model, serial, calibration date |
| Torque Mark Applied: | Yes / No (paint line across bolt, nut and flange) |

4. Post-Assembly and Leak Test

| Joint Included in Pressure Test: | Yes / No (test certificate reference) |
| --- | --- |
| Leak Test Result: | No leak / Leak detected (describe location and action) |
| Joint Tagged / Labelled: | Yes / No (tag number applied) |
| Joint Added to Joint Register: | Yes / No |
| Photographic Record Taken: | Yes / No (photo reference) |

5. Sign-Off

By signing, the assembler confirms the joint was assembled in accordance with the specification and this checklist. The competent person confirms the assembly was witnessed or verified.

| Role | Print Name | Signature | Date |
| --- | --- | --- | --- |
| Joint Assembler / Fitter |  |  |  |
| Competent Person (Flange Assembly) |  |  |  |
| Quality Inspector |  |  |  |
| Client / Engineer's Representative |  |  |  |

6. Notes / Additional Comments

Enter any additional notes or attach bolt tightening diagram

7. Document Control

| Rev | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |

References: ASME PCC-1 (Guidelines for Pressure Boundary Bolted Flange Joint Assembly), BS EN 1591 (Flange Connections), BS EN 1092-1, ASME B16.5, HSE Safety Alert, Project Specification, ITP