MEWP Pre-Use Inspection Checklist

Daily pre-use inspection checklist for Mobile Elevating Work Platforms (MEWPs) including scissor lifts, boom lifts (cherry pickers), spider lifts, and trailer-mounted platforms. To be completed by the operator or a competent person before each use. In accordance with LOLER 1998, PUWER 1998, BS EN 280 (Mobile elevating work platforms — Design, calculations, safety requirements and test methods), IPAF guidance, and the project working at height procedures.

MEWP and Operator Details

| Date: | DD / MM / YYYY |
| --- | --- |
| MEWP Type: | e.g. scissor lift, cherry picker, boom lift, spider lift, trailer mount |
| Make and Model: | Enter manufacturer and model |
| Fleet / ID Number: | Enter unique plant ID number |
| Serial Number: | Enter serial number |
| Maximum Platform Height: | Enter maximum working height (m) |
| Maximum SWL (Platform): | Enter maximum platform load (kg or persons) |
| Current Thorough Examination Date: | DD / MM / YYYY (LOLER Reg. 9 — 6-monthly for MEWPs) |
| Thorough Examination Expiry: | DD / MM / YYYY |
| Examination Report Ref: | Enter report of thorough examination reference |
| Operator Name: | Enter operator name |
| Operator IPAF / CPCS Card No.: | Enter IPAF PAL card or CPCS card number and category |
| Card Expiry Date: | DD / MM / YYYY |
| Site / Location: | Enter site name and area of use |
| Inspected By: | Enter inspector name and role |

Pre-Use Inspection Checklist

Mark each item OK, Defect, or NA. Any item marked Defect must be reported immediately and the MEWP must not be used until the defect is rectified.

| No. | Inspection Item | OK / Defect / NA | Comments / Action |
| --- | --- | --- | --- |
| 1 | Operator holds valid IPAF PAL card or CPCS card for this category of MEWP |  |  |
| 2 | Current thorough examination certificate available on the machine (within 6 months) |  |  |
| 3 | Machine logbook / maintenance records available and up to date |  |  |
| 4 | Manufacturer operating manual available on the machine |  |  |
| 5 | Platform guardrails, mid-rails, and toe boards secure and undamaged |  |  |
| 6 | Platform floor in good condition — no holes, excessive wear, or slippery surfaces |  |  |
| 7 | Platform gate / access gate closes and latches securely |  |  |
| 8 | Lanyard anchorage point clearly marked and in good condition |  |  |
| 9 | Emergency lowering controls accessible and clearly labelled at ground level |  |  |
| 10 | Emergency lowering function tested and operating correctly |  |  |
| 11 | Upper controls — all functions tested (raise, lower, extend, retract, slew, drive) |  |  |
| 12 | Ground controls — all functions tested and override working correctly |  |  |
| 13 | Engine / motor starting and running smoothly — no abnormal noise or vibration |  |  |
| 14 | Battery charge level adequate for planned work (electric machines) |  |  |
| 15 | Hydraulic system — no visible leaks from hoses, rams, cylinders, or connections |  |  |
| 16 | Hydraulic oil level checked and within acceptable range |  |  |
| 17 | Tyres / wheels in good condition — correct pressure, no damage or excessive wear |  |  |
| 18 | Outriggers / stabilisers extending and retracting correctly (if fitted) |  |  |
| 19 | Outrigger pads available and in good condition |  |  |
| 20 | Chassis and boom structure — no visible cracks, damage, deformation, or corrosion |  |  |
| 21 | Boom pins and locking devices secure |  |  |
| 22 | Slew ring and turntable — no excessive play or abnormal noise |  |  |
| 23 | Tilt alarm / sensor operational (alerts when machine exceeds safe inclination) |  |  |
| 24 | Overload protection system operational (if fitted) |  |  |
| 25 | Limit switches tested — travel, height, and boom limits functioning |  |  |
| 26 | Horn / audible warning device operational |  |  |
| 27 | Flashing beacon operational (if fitted) |  |  |
| 28 | Lights operational (work lights, travel lights if self-propelled) |  |  |
| 29 | Fire extinguisher present and in date (if required by site rules) |  |  |
| 30 | Ground conditions assessed — firm, level, and capable of supporting machine weight |  |  |
| 31 | Overhead obstructions and power lines identified and safe clearances confirmed |  |  |
| 32 | Exclusion zone established beneath elevated platform where required |  |  |

Inspection Result

| Overall Inspection Result: | Fit for Use / Not Fit for Use |
| --- | --- |
| Defects Found: | Detail any defects identified |
| Defects Reported To: | Enter name and role |
| MEWP Quarantined: | Yes / No — if yes, confirm lock-out applied and keys removed |

Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| MEWP Operator |  |  | DD / MM / YYYY |
| Site Supervisor / Foreman |  |  | DD / MM / YYYY |
| Plant Manager (if defects found) |  |  | DD / MM / YYYY |

Notes / Additional Comments

Enter any additional notes, defect rectification confirmations, or references to rescue plans here.

Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| A |  | Initial issue |  |  |