Surface Preparation and Blast Profile Record

Record of surface preparation (abrasive blasting, power tool cleaning, or other methods) and blast profile verification prior to protective coating application. Completed as a permanent quality record in accordance with BS EN ISO 8501-1 (Visual assessment of surface cleanliness), BS EN ISO 8503 (Surface roughness characteristics), BS EN ISO 8502 (Tests for assessment of surface cleanliness), and the project coating specification.

Project Details

| Document Ref: | Enter document reference |
| --- | --- |
| Revision: | Enter revision number |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| Blasting / Coating Contractor: | Enter specialist contractor name |
| Prepared By: | Name / role |
| Approved By: | Name / role |

Item and Coating System

| Item / Structure Reference: | Enter piece mark, tag, or structure reference |
| --- | --- |
| Drawing Reference: | Enter coating / painting drawing or specification clause |
| Substrate Material: | e.g. carbon steel, stainless steel, galvanised, concrete |
| Previous Coating (if applicable): | Describe existing coating system if recoating |
| Surface Area (m²): | Enter approximate area prepared |
| Coating Specification Reference: | Enter protective coating specification document ref |
| Coating System: | e.g. System 1 — epoxy primer + MIO + polyurethane topcoat |

Surface Preparation Details

| Surface Preparation Method: | e.g. dry abrasive blast, wet blast, power tool clean, hand tool clean, UHP water jetting |
| --- | --- |
| Preparation Standard: | e.g. BS EN ISO 8501-1 Sa 2.5, Sa 3, St 2, St 3; SSPC-SP5, SP6, SP10 |
| Required Cleanliness Grade: | Enter grade (e.g. Sa 2.5 = near-white metal blast) |
| Achieved Cleanliness Grade: | Enter actual grade achieved and confirmed by comparator |
| Abrasive Type: | e.g. steel grit G25, aluminium oxide, garnet, copper slag |
| Abrasive Size / Grade: | Enter grit size range |
| Abrasive Cleanliness: | Confirm abrasive tested for chloride and oil contamination |
| Compressed Air Quality: | Confirm blotter test passed (free from oil and moisture) per BS EN ISO 8573 |
| Anchor Profile Required (microns): | Enter specified profile depth range (e.g. 50-75 microns) |
| Actual Anchor Profile (microns): | Enter measured profile using replica tape or profilometer |
| Profile Measurement Method: | e.g. Testex Press-O-Film replica tape, Elcometer surface profile gauge |
| Number of Profile Readings: | Enter number of readings taken and locations |

Environmental Conditions

| Ambient Temperature (deg C): | Enter ambient temperature at time of blasting |
| --- | --- |
| Relative Humidity (%): | Enter RH — typically max 85% for blast operations |
| Steel Surface Temperature (deg C): | Enter measured surface temperature |
| Dew Point (deg C): | Enter calculated dew point |
| Surface Temp Above Dew Point (deg C): | Enter difference (minimum 3 deg C above dew point per BS EN ISO 8502-4) |
| Wind Speed: | Enter if outdoor — confirm adequate for dust / debris control |

Inspection Checklist

| No. | Inspection Item | Y / N / NA | Comments |
| --- | --- | --- | --- |
| 1 | Surface free from oil, grease, and soluble contaminants prior to blasting (solvent wipe or degreasing carried out) |  |  |
| 2 | Mill scale, rust, and previous coating fully removed to specified cleanliness grade |  |  |
| 3 | No residual flash rust or oxidation on blasted surface at time of inspection |  |  |
| 4 | Anchor profile within specified range — confirmed by replica tape or profilometer |  |  |
| 5 | Surface free from embedded abrasive, dust, and debris (compressed air blow-off completed) |  |  |
| 6 | Soluble salt contamination tested and within limits (Bresle patch test per BS EN ISO 8502-6 / -9) |  |  |
| 7 | All sharp edges ground to minimum 2mm radius or chamfered prior to coating |  |  |
| 8 | Weld spatter, slag, and undercut defects removed by grinding prior to coating |  |  |
| 9 | All inspection hold points released before coating application commences |  |  |
| 10 | Abrasive blast area contained and environmental controls in place (dust, waste, noise) |  |  |
| 11 | Environmental conditions within specification (temperature, humidity, dew point margin) |  |  |
| 12 | Surface prepared within the specified recoat interval (typically 4 hours or before flash rust) |  |  |
| 13 | Photographic record taken of prepared surface showing cleanliness and profile |  |  |

Inspection Result

| Overall Surface Preparation Result: | Pass / Fail |
| --- | --- |
| Surface Approved for Coating: | Yes / No — coating must be applied within recoat interval |
| Non-Conformances: | Enter details of any deficiencies and NCR ref if raised |

Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Blast Operator / Foreman |  |  | DD / MM / YYYY |
| Coating Inspector (NACE / ICORR / FROSIO) |  |  | DD / MM / YYYY |
| QC Inspector / Site Engineer |  |  | DD / MM / YYYY |
| Client / Third Party Inspector |  |  | DD / MM / YYYY |

Notes / Additional Comments

Enter any additional notes, Bresle test results, profile reading data, photographic references, or replica tape samples retained here.

Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| A |  | Initial issue |  |  |