Surface Preparation Inspection Record

Record of surface preparation prior to coating application. Covers abrasive blast cleaning, power tool cleaning, or hand preparation. Documents cleanliness grade, surface profile, and environmental conditions at time of inspection. In accordance with BS EN ISO 8501-1 (visual cleanliness), BS EN ISO 8503 (surface profile), and SSPC/NACE standards.

Project Details

| Document Ref: | Enter document reference |
| --- | --- |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| Structure / Component: | Enter element or component reference |
| Drawing Reference: | Enter coating specification drawing |

Surface Preparation Details

| Preparation Method: | e.g. abrasive blast (dry grit), wet blast, power tool (St 3), hand preparation (St 2), UHP water jetting |
| --- | --- |
| Abrasive Type (if blast): | e.g. steel grit G25, copper slag, garnet — enter grade and size |
| Cleanliness Grade Specified: | e.g. Sa 2½ (BS EN ISO 8501-1), SSPC-SP10 (Near White) |
| Cleanliness Grade Achieved: | Enter assessed grade — compare to visual standard ISO 8501-1 |
| Surface Profile Specified (µm): | Enter required Rz or Ry5 profile depth range |
| Surface Profile Measured (µm): | Enter measured profile — testex tape / stylus method |
| Profile Measurement Method: | e.g. Testex replica tape, stylus gauge (BS EN ISO 8503-5) |
| Soluble Salt Level (µg/cm²): | Enter if measured — Bresle patch method (BS EN ISO 8502-6) |
| Maximum Permitted Salt Level: | Enter spec limit (typically <20 µg/cm² chloride) |
| Dust Level (BS EN ISO 8502-3): | Enter dust quantity rating (1-5) and size class (0-5) |
| Oil / Grease Contamination: | None detected / Present — describe degreasing carried out |
| Surface Defects: | Enter any pitting, laminations, weld spatter, sharp edges requiring treatment |
| Substrate Temperature (°C): | Enter — must be >3°C above dew point |
| Ambient Temperature (°C): | Enter ambient temperature |
| Relative Humidity (%): | Enter RH — typically max 85% |
| Dew Point (°C): | Enter calculated dew point |
| Time Window Before Coating: | Enter maximum time before first coat must be applied (typically 4 hours) |
| Inspected By: | Enter coating inspector name and NACE/BGAS-CSWIP certification |

Notes / Additional Comments

Enter any additional notes or supporting documentation references here.

Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Coating Inspector |  |  | DD / MM / YYYY |
| Site Engineer |  |  | DD / MM / YYYY |
| QA Manager |  |  | DD / MM / YYYY |

Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| A |  | Initial issue |  |  |