| HAV (HAND-ARM VIBRATION) EXPOSURE REGISTER |  |
| --- | --- |
| Document Ref: | Enter document reference |
| Revision: | Enter revision number |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| Site Address: | Enter site address |
| Prepared By: | Name, position and company |
| Approved By: | Name, position and company |

This register records daily hand-arm vibration (HAV) exposure for all operatives using vibrating tools or equipment. It is a requirement of the Control of Vibration at Work Regulations 2005 to assess and manage vibration exposure. Use the HSE exposure calculator or exposure points system to calculate daily A(8) exposure.

Exposure Limits

| Parameter | Value | Action |
| --- | --- | --- |
| Exposure Action Value (EAV) | 2.5 m/s² A(8) | Introduce controls, health surveillance, provide information and training |
| Exposure Limit Value (ELV) | 5 m/s² A(8) | Must not be exceeded — stop work, review controls immediately |

1. Daily Exposure Register

Record each operative's daily vibration exposure. One row per person per tool. Sum partial A(8) values for cumulative daily exposure.

| No. | Name | Company | Tool / Equipment | Vibration (m/s²) | Trigger Time (min) | Exposure Points | Partial A(8) | Cumul. A(8) | Above EAV? | Above ELV? | Action Taken |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |  |  |  |  |  |
| 21 |  |  |  |  |  |  |  |  |  |  |  |
| 22 |  |  |  |  |  |  |  |  |  |  |  |
| 23 |  |  |  |  |  |  |  |  |  |  |  |
| 24 |  |  |  |  |  |  |  |  |  |  |  |
| 25 |  |  |  |  |  |  |  |  |  |  |  |

2. Common Tool Vibration Reference Data

Indicative values only. Always use manufacturer's declared vibration values where available. Actual exposure depends on tool condition, material and operator technique.

| Tool Type | Typical Vibration (m/s²) | Time to EAV (mins) | Time to ELV (mins) |
| --- | --- | --- | --- |
| Breaker (hand-held, medium) | 12–25 | 15–65 | 60–260 |
| Angle grinder (4.5” / 9”) | 4–12 | 65–600 | 260–2400 |
| Hammer drill | 6–20 | 25–265 | 100–1060 |
| Circular saw (hand-held) | 3–8 | 150–1060 | 600–4250 |
| Vibrating plate compactor | 4–12 | 65–600 | 260–2400 |
| Vibrating poker (concrete) | 6–18 | 30–265 | 120–1060 |
| Needle scaler | 5–20 | 25–400 | 100–1600 |
| Impact wrench | 3–8 | 150–1060 | 600–4250 |
| Strimmer / brush cutter | 3–7 | 200–1060 | 800–4250 |
| Chain saw | 4–8 | 150–600 | 600–2400 |

3. Sign-Off

| Role | Print Name | Signature | Date |
| --- | --- | --- | --- |
| Site Manager / Supervisor |  |  |  |
| H&S Manager / Advisor |  |  |  |

4. Document Control

| Rev | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |

References: Control of Vibration at Work Regulations 2005, HSE L140 Hand-Arm Vibration, HSE Vibration Exposure Calculator, INDG175, HAVS Tier Assessment