Road Marking and Signage Installation Checklist

Checklist for the installation of road markings, signs, and associated street furniture. Covers material specification, positioning, visibility, retro-reflectivity, and compliance with TSRGD 2016 (Traffic Signs Regulations and General Directions) and the Traffic Signs Manual.

Project Details

| Document Ref: | Enter document reference |
| --- | --- |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| Location: | Enter road / junction / area |
| Drawing Reference: | Enter signage and markings layout drawing reference |

Road Markings

| Marking Type: | e.g. centre line, edge line, give way, stop line, hatching, zig-zag, word marking |
| --- | --- |
| Marking Material: | e.g. thermoplastic screed, thermoplastic spray, cold applied, preformed |
| Colour: | White / Yellow — confirm correct per TSRGD Sch 6 and Sch 7 |
| Width (mm): | Enter measured width — confirm matches TSRGD requirement |
| Retro-Reflectivity (mcd/lux/m²): | Enter measured retro-reflectivity — confirm meets RL min value |
| Skid Resistance (SRV): | Enter if measured — min 45 SRV for road markings |
| Surface Preparation: | Describe preparation method (sweep, prime, dry surface) |
| Application Temperature (°C): | Enter material application temperature |
| Markings Comply with Drawing: | Yes / No — confirm position, dimensions, and layout correct |

Signage

| Sign Reference: | Enter sign diagram number per TSRGD (e.g. Diag 601.1, 602, 670) |
| --- | --- |
| Sign Size: | Enter dimensions (mm) — confirm correct size for road speed |
| Sign Mounting Height (mm): | Enter measured height to bottom of sign (min 900mm footway, 1500mm cycleway, 2100mm over carriageway) |
| Post Type: | e.g. steel, aluminium, passive safe — enter post specification |
| Foundation: | Enter foundation type (root fixed, flanged, socket) |
| Retro-Reflectivity Class: | Enter Class RA1, RA2, or RA3 per BS EN 12899-1 |
| Sign Face Condition: | Clean, undamaged, no stickers or graffiti |
| Visibility Check: | Confirm sign is visible from required distance for road speed |
| Correct Orientation: | Confirm sign faces approaching traffic at correct angle |
| Signs Comply with Drawing: | Yes / No — confirm type, position, and height correct |

Notes / Additional Comments

Enter any additional notes, sketches, photographic references, or supporting documentation here.

Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Signing and Lining Foreman |  |  | DD / MM / YYYY |
| Site Engineer |  |  | DD / MM / YYYY |
| Highway Inspector |  |  | DD / MM / YYYY |

Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| A |  | Initial issue |  |  |