Kerb and Edging Line and Level Check

Quality check record for kerb, edging, and channel installation. Records line, level, joint spacing, haunching, and backing dimensions against the design specification. In accordance with the Specification for Highway Works (SHW) Series 1100 and project-specific requirements.

Project Details

| Document Ref: | Enter document reference |
| --- | --- |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| Location / Chainage: | Enter road name, chainage from-to, or area reference |
| Drawing Reference: | Enter design drawing and revision |

Kerb Details

| Kerb Type: | e.g. HB2 half batter, BN bullnosed, splayed, dropped, transition, quadrant |
| --- | --- |
| Kerb Size: | Enter dimensions (e.g. 125 x 255 x 915mm) |
| Material: | e.g. precast concrete to BS EN 1340, natural granite, conservation |
| Manufacturer / Supplier: | Enter supplier name and batch reference |
| Foundation / Bed: | Enter bedding material (e.g. ST1 concrete) and thickness (mm) |
| Haunching: | Enter haunching specification and dimensions (mm) |
| Backing: | Enter backing specification and dimensions (mm) |

Line and Level Checks

| Line Check Method: | e.g. string line, laser, total station — describe method |
| --- | --- |
| Line Tolerance: | Enter permitted tolerance (typically +/- 6mm from design line) |
| Maximum Deviation Measured: | Enter worst-case deviation from design line (mm) |
| Level Check Method: | e.g. levelling staff and auto level, total station |
| Design Level (mAOD): | Enter at representative points |
| Actual Level Measured (mAOD): | Enter measured levels |
| Level Tolerance: | Enter permitted tolerance (typically +/- 6mm) |
| Maximum Deviation Measured: | Enter worst-case level deviation (mm) |
| Joint Spacing: | Enter measured joint width (typically 3-5mm) |
| Joints Filled: | Yes / No — enter jointing material |
| Overall Result: | Pass / Fail / Remedial Action Required |

Notes / Additional Comments

Enter any additional notes, sketches, photographic references, or supporting documentation here.

Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Setting-Out Engineer |  |  | DD / MM / YYYY |
| Foreman / Ganger |  |  | DD / MM / YYYY |
| Site Engineer (verification) |  |  | DD / MM / YYYY |

Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| A |  | Initial issue |  |  |