Street Lighting Installation Record

Record of street lighting column and lantern installation. Covers column foundation, column erection, lantern fitting, electrical connection, and commissioning. In accordance with BS 5489 (Code of practice for road lighting), BS EN 13201 (Road lighting), and the adopting authority specification.

Project Details

| Document Ref: | Enter document reference |
| --- | --- |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| Location: | Enter column location (chainage, plot, road name) |
| Column Reference No.: | Enter unique column ID from lighting design |

Column Details

| Column Manufacturer: | Enter manufacturer |
| --- | --- |
| Column Type: | e.g. 6m, 8m, 10m, 12m — straight, post-top, side-entry, decorative |
| Column Material: | e.g. steel (galvanised), aluminium, GRP, stainless steel |
| Column Serial No.: | Enter serial number |
| Foundation Type: | e.g. root planted, flanged base with bolts, socket foundation |
| Foundation Depth (mm): | Enter depth of foundation or root |
| Concrete Foundation: | Enter concrete spec, pour date, and cube reference if applicable |
| Column Plumb: | Yes / No — check with spirit level on two faces |
| Column Height (m): | Enter actual installed height |
| Bracket / Outreach (m): | Enter bracket length if side-entry |

Lantern Details

| Lantern Manufacturer: | Enter manufacturer |
| --- | --- |
| Lantern Model: | Enter model and wattage |
| Lantern Type: | LED / SON / CDM — enter type |
| Colour Temperature (K): | Enter colour temperature (e.g. 3000K, 4000K) |
| Tilt Angle (degrees): | Enter tilt — confirm zero upward light (0% ULR for environmental zones) |
| Lantern Serial No.: | Enter serial number |

Electrical and Commissioning

| Cable Size: | Enter supply cable size (e.g. 6mm² 3-core SWA) |
| --- | --- |
| Fuse Rating: | Enter fuse size in column base |
| Earth Electrode Installed: | Yes / No — enter resistance value (ohms) |
| Earth Resistance (ohms): | Enter measured value — max 200 ohms per BS 7671 |
| Insulation Resistance (MΩ): | Enter IR test result |
| Polarity Check: | Correct / Incorrect |
| Photocell / NEMA Socket: | Fitted / Not fitted — enter type |
| Commissioning Date: | DD / MM / YYYY |
| Light Output Verified: | Yes / No — confirm operational |

Notes / Additional Comments

Enter any additional notes, sketches, photographic references, or supporting documentation here.

Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Lighting Installer |  |  | DD / MM / YYYY |
| Electrical Supervisor |  |  | DD / MM / YYYY |
| Site Engineer |  |  | DD / MM / YYYY |
| Adopting Authority Inspector |  |  | DD / MM / YYYY |

Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| A |  | Initial issue |  |  |