Water Pump Pre-Use Inspection Checklist

| Document Ref: | Enter document reference |
| --- | --- |
| Revision: | Enter revision number |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract No.: | Enter contract number |
| Prepared By: | Enter name and role |
| Approved By: | Enter name and role |

# Equipment Details

| Pump Type: | e.g. Submersible / Centrifugal / Diaphragm / Trash | Power Source: | Diesel / Petrol / Electric / Hydraulic |
| --- | --- | --- | --- |
| Manufacturer: | Enter manufacturer | Model: | Enter model |
| Asset / Serial No.: | Enter asset number | Pump Capacity: | Enter L/min or m³/hr |
| Location / Area: | Enter location | Purpose: | e.g. Excavation dewatering |
| Inspected By: | Enter name | Inspection Date: | DD / MM / YYYY |

# Pre-Use Inspection Checklist

Mark each item as ‘Yes’ (satisfactory), ‘No’ (defective), or ‘N/A’ (not applicable). Do not operate the pump if any safety-critical item is marked ‘No’ until the defect has been rectified.

| No. | Inspection Item | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| General Condition |  |  |  |  |  |
| 1 | Pump body / casing free from cracks, dents, and corrosion |  |  |  |  |
| 2 | All guards and covers in place and securely fastened |  |  |  |  |
| 3 | Rating plate / asset identification tag legible |  |  |  |  |
| 4 | Lifting points (if fitted) undamaged and clearly marked |  |  |  |  |
| 5 | Pump positioned on stable, level surface away from flood risk |  |  |  |  |
| 6 | Adequate ventilation provided (for diesel / petrol units in enclosed areas) |  |  |  |  |

| Engine / Motor (Diesel / Petrol Units) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 7 | Engine oil level checked and within operating range |  |  |  |  |
| 8 | Fuel level adequate with no leaks from tank, lines, or connections |  |  |  |  |
| 9 | Coolant level checked (if water-cooled engine) |  |  |  |  |
| 10 | Air filter clean and correctly seated |  |  |  |  |
| 11 | Battery condition satisfactory (if electric start) |  |  |  |  |
| 12 | Exhaust system secure, free from leaks, and directed away from personnel |  |  |  |  |
| 13 | Engine starts without difficulty and runs smoothly |  |  |  |  |
| 14 | Emergency stop / kill switch operational |  |  |  |  |

| Electrical (Electric Pumps) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 15 | Supply cable free from cuts, abrasion, and exposed conductors |  |  |  |  |
| 16 | Plug and connector undamaged with no signs of overheating |  |  |  |  |
| 17 | Earth continuity confirmed and tested |  |  |  |  |
| 18 | RCD / circuit protection available, tested, and operational |  |  |  |  |
| 19 | Motor runs without excessive noise, vibration, or overheating |  |  |  |  |

| Suction and Delivery |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 20 | Suction hose free from splits, kinks, perishing, and blockages |  |  |  |  |
| 21 | Suction strainer / foot valve present, clean, and undamaged |  |  |  |  |
| 22 | All hose couplings secure with locking clips / cam locks in place |  |  |  |  |
| 23 | Delivery hose free from damage and securely connected |  |  |  |  |
| 24 | Priming mechanism operational (if self-priming type) |  |  |  |  |
| 25 | Non-return valve fitted and operational (if required) |  |  |  |  |
| 26 | Discharge point directed to approved location (settlement tank, drainage, etc.) |  |  |  |  |

| Seals, Gaskets and Fittings |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 27 | Mechanical seal / packing gland in good condition with no excessive leakage |  |  |  |  |
| 28 | All joints and flanges tight with no leaks under pressure |  |  |  |  |
| 29 | Impeller free from blockage and rotating freely |  |  |  |  |
| 30 | Pressure gauge / vacuum gauge operational (if fitted) |  |  |  |  |
| 31 | Drain plug secure and in good condition |  |  |  |  |

| Safety and Environmental |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 32 | Drip tray / spill containment provided beneath pump and fuel storage |  |  |  |  |
| 33 | Spill kit available and stocked within reach of pump location |  |  |  |  |
| 34 | Fuel stored in approved bunded container away from watercourses |  |  |  |  |
| 35 | Noise assessment completed and hearing protection provided if required |  |  |  |  |
| 36 | Pump discharge does not risk flooding or contamination of watercourse |  |  |  |  |
| 37 | Environmental permit / consent in place where required (e.g. EA discharge consent) |  |  |  |  |
| 38 | Silt management measures in place at discharge point (e.g. settlement tank, silt fence) |  |  |  |  |

# Defects Identified

| No. | Defect Description | Action Taken | Rectified By | Date |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |

# Inspection Outcome

| Equipment Fit for Use: | Yes / No (delete as appropriate) |
| --- | --- |
| If No, state reason: | Enter details |

# Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Inspector / Operator |  |  |  |
| Supervisor / Foreman |  |  |  |

# Notes / Additional Comments

Regulatory References: PUWER 1998, Electricity at Work Regulations 1989, Environmental Permitting (England and Wales) Regulations 2016, CIRIA C532 (Control of water pollution from construction sites), HSE INDG163.

# Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| 01 |  | Initial issue |  |  |