Scaffold Risk Assessment

| Document Ref: | Enter document reference |
| --- | --- |
| Revision: | Enter revision number |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract No.: | Enter contract number |
| Prepared By: | Enter name and role |
| Approved By: | Enter name and role |

Risk Rating: L = Likelihood (1–5), S = Severity (1–5), R = Risk (L x S). Low (1–6), Medium (8–12), High (15–25). All controls must reduce risk to ALARP.

# Risk Assessment

| No. | Hazard | Who at Risk | Consequence | Control Measures | L | S | Risk |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Falls from height during erection / dismantling | Scaffold erectors, labourers | Fatal / Major injury | Competent CISRS-trained erectors only. SG4:22 procedures followed. Full body harness worn during leading edge work. Collective edge protection installed as work progresses. Advance guardrails used. Rescue plan in place. | L | S | L |
| 2 | Falls from height during scaffold use | All scaffold users | Fatal / Major injury | Full guardrails (950 mm min.), mid-rails, and toe boards on all working platforms. Platforms fully boarded. Green tag inspection regime. Scaffold safety briefing delivered to all users. | L | S | L |
| 3 | Scaffold collapse / structural failure | All site personnel, public | Fatal / Multiple fatalities | Design to TG20:21 or bespoke design by competent engineer. Independent design check for Cat 2/3. Adequate ties at correct centres. Regular 7-day inspections. Loading class displayed and enforced. | L | S | L |
| 4 | Falling objects / materials from scaffold | Persons below scaffold | Serious / Fatal injury | Toe boards on all platforms. Brick guards and debris netting installed. Tool lanyards used. Exclusion zone at base. No throwing of materials. Loading bays used for material transfer. | L | M | L |
| 5 | Manual handling of scaffold components | Scaffold erectors | Musculoskeletal injury | Mechanical handling where possible (hoist, crane). Team lifts for heavy components. Rotation of tasks. Correct lifting techniques briefed. | M | M | M |
| 6 | Electrocution from overhead power lines | Scaffold erectors, users | Fatal injury | Minimum clearance distances maintained per HSE GS6. Goal posts installed. Barrier tape. Scaffold height restricted where required. Power company consulted. | L | S | L |
| 7 | Adverse weather (wind, ice, lightning) | All scaffold users | Falls, collapse | Wind speed monitoring. Scaffold not used above manufacturer wind limit (typically 17 m/s). Sheeted scaffolds have lower limits. Ice gritting of platforms. Lightning procedure. | M | S | M |
| 8 | Unauthorised alteration of scaffold | All scaffold users | Collapse, falls | Scaffold alteration request procedure enforced. Red tag applied during alterations. Site rules poster displayed. Disciplinary procedure for breaches. | L | S | L |
| 9 | Slips, trips and falls on scaffold platforms | Scaffold users | Sprains, fractures | Good housekeeping enforced. Platforms kept clear. Anti-slip measures in wet/icy conditions. Adequate lighting. Boot cleaning provisions. | M | M | M |
| 10 | Contact with hazardous substances | Scaffold erectors | Skin / respiratory irritation | PPE provided (gloves for treated timber). COSHH assessment for any coatings. Welfare facilities available for hand washing. | L | L | L |
| 11 | Proximity to vehicle movements | Scaffold erectors, users | Struck by vehicle, collapse | Vehicle exclusion zone around scaffold base. Stop blocks. Banksman for reversing vehicles. High-visibility clothing. Physical protection (Armco, concrete barriers) where needed. | L | S | L |
| 12 | Public interface / pedestrians | Members of public | Falls, struck by objects | Pedestrian protection fans / covered walkways. Exclusion zone. Debris netting. Warning signage. Night lighting. Considerate Constructors compliance. | L | S | L |

# Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Assessor |  |  |  |
| Reviewed By (H&S) |  |  |  |
| Approved By |  |  |  |

Regulatory References: Work at Height Regulations 2005, Management of Health & Safety at Work Regulations 1999, CDM 2015, NASC TG20:21, SG4:22, BS EN 12811.

# Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| 01 |  | Initial issue |  |  |