Temporary Retention Structure Inspection

| Document Ref: | Enter document reference |
| --- | --- |
| Revision: | Enter revision number |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract No.: | Enter contract number |
| Prepared By: | Enter name and role |
| Approved By: | Enter name and role |

# Structure Details

| Structure Ref / ID: | Enter reference | TW Register Ref: | Enter TW number |
| --- | --- | --- | --- |
| Type of Structure: | e.g. Sheet pile wall, king post wall, gabion wall, soil nails | Design Reference: | Enter design ref |
| Retained Height: | Enter in metres | Structure Length: | Enter in metres |
| Design Surcharge: | Enter kN/m² | Weather Conditions: | Enter conditions |
| Inspector Name: | Enter name | Inspection Date: | DD / MM / YYYY |

# Inspection Checklist

Mark each item Yes (satisfactory), No (defective), or N/A. Report any item marked No immediately to the TWC.

| No. | Inspection Item | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| Structural Condition |  |  |  |  |  |
| 1 | Structure free from visible cracking, distortion, or displacement |  |  |  |  |
| 2 | All structural connections intact and showing no signs of overstress |  |  |  |  |
| 3 | Waling beams correctly aligned and securely connected |  |  |  |  |
| 4 | Struts / props correctly positioned, plumb, and adequately braced |  |  |  |  |
| 5 | Tie-back anchors (if used) showing no signs of failure or yield |  |  |  |  |
| 6 | Base and foundation conditions stable with no undermining or scour |  |  |  |  |
| 7 | No signs of rotation, tilting, or lateral movement |  |  |  |  |

| Retained Ground and Drainage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 8 | Retained soil face stable with no signs of bulging, slumping, or loss of material |  |  |  |  |
| 9 | Surcharge loading within design limits – no unauthorised plant or materials within exclusion zone |  |  |  |  |
| 10 | Drainage provisions (weep holes, filter drains) functioning and not blocked |  |  |  |  |
| 11 | No hydrostatic pressure build-up behind structure |  |  |  |  |

| Monitoring |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 12 | Ground and wall movement monitoring readings within trigger levels |  |  |  |  |
| 13 | Adjacent structures and services showing no signs of distress |  |  |  |  |
| 14 | Monitoring instruments (inclinometers, settlement points, piezometers) functioning |  |  |  |  |

| Safety and Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 15 | Edge protection and barriers in place and secure |  |  |  |  |
| 16 | Warning signage displayed (loading restrictions, exclusion zones) |  |  |  |  |
| 17 | Safe access maintained around the structure |  |  |  |  |

| General |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 18 | Dewatering system (if applicable) operational and maintaining required drawdown |  |  |  |  |
| 19 | Structure and surroundings inspected after adverse weather events |  |  |  |  |
| 20 | All inspection and monitoring records up to date |  |  |  |  |

# Monitoring Readings Summary

| Point Ref | Type | Previous | Current | Change | Trigger |
| --- | --- | --- | --- | --- | --- |
| MP-1 |  |  |  |  |  |
| MP-2 |  |  |  |  |  |
| MP-3 |  |  |  |  |  |
| MP-4 |  |  |  |  |  |

# Defects Identified

| No. | Defect | Action Required | By Whom | Target Date |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |

# Inspection Outcome

| Structure Safe for Continued Use: | Yes / No / Conditional |
| --- | --- |
| If Conditional, state conditions: | Enter details |
| Next Inspection Due: | DD / MM / YYYY |

# Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Inspector |  |  |  |
| Temporary Works Coordinator |  |  |  |

# Notes / Additional Comments

Regulatory References: BS 5975:2019, CDM 2015, BS 8002 (Earth Retaining Structures), CIRIA C580 (Embedded Retaining Walls), Eurocode 7.

# Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| 01 |  | Initial issue |  |  |