Dewatering System Design Request

| Document Ref: | Enter document reference |
| --- | --- |
| Revision: | Enter revision number |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract No.: | Enter contract number |
| Prepared By: | Enter name and role |
| Approved By: | Enter name and role |

This form requests dewatering system design from the temporary works designer or specialist subcontractor. Complete all sections with available data and attach ground investigation reports, site plans, and any relevant environmental permits.

# Section 1: General Information

| TW Register Ref: | Enter TW number | TW Category: | 1 / 2 / 3 |
| --- | --- | --- | --- |
| Purpose of Dewatering: | e.g. Excavation dewatering, shaft sinking, trench works, foundation construction |  |  |
| Location on Site: | Enter specific area, grid references |  |  |

# Section 2: Excavation Details

| Excavation Dimensions: | L x W (m) | Depth Below GL: | Enter in metres |
| --- | --- | --- | --- |
| Existing Ground Level: | Enter mAOD | Formation Level Required: | Enter mAOD |
| Target Drawdown Level: | Enter mAOD (min. 0.5 m below formation) |  |  |
| Excavation Support Type: | e.g. Sheet piles, battered slopes, trench boxes, secant piles, or open cut |  |  |

# Section 3: Hydrogeological Conditions

| Groundwater Level: | Enter mAOD or depth BGL – include seasonal variation if known |
| --- | --- |
| Soil Type(s): | Describe strata sequence: e.g. Topsoil / Alluvial clay / River gravel / London Clay |
| Permeability (k): | Enter if known (m/s) for each relevant stratum, or state Unknown |
| Aquifer Type: | Unconfined / Confined / Perched / Multiple – describe |
| Tidal / River Influence: | Yes / No – If Yes, describe proximity and tidal range |
| Artesian Conditions: | Yes / No / Unknown |
| GI Report Reference: | Enter report reference and date |
| Pumping Test Data: | Available / Not available – attach results if available |

# Section 4: Environmental and Regulatory

| EA Abstraction Licence: | Required / In place / Not required – enter licence ref if applicable |
| --- | --- |
| Discharge Consent: | Required / In place / Not required – enter consent ref |
| Discharge Location: | e.g. Surface water drain, watercourse, soakaway, foul sewer, tanker |
| Water Quality Concerns: | Contamination, suspended solids, pH, hydrocarbons – describe or state None |
| Treatment Required: | Settlement, filtration, pH adjustment, oil separation – describe or state Designer to determine |
| Nearby Receptors: | Adjacent buildings, services, trees, watercourses, protected habitats |

# Section 5: Programme and Duration

| Design Required By: | DD / MM / YYYY | Installation Start: | DD / MM / YYYY |
| --- | --- | --- | --- |
| Dewatering Duration: | Enter estimated duration (weeks / months) |  |  |
| 24/7 Pumping Required: | Yes / No | Standby Pumps Required: | Yes / No |

# Section 6: Preferred System

| Preferred Method: | e.g. Sump pumping, wellpoint, deep well, ejector, or Designer to propose |
| --- | --- |
| Power Supply Available: | Mains / Generator – describe capacity and location |

# Section 7: Sketch / Plan

Attach or insert a sketch showing excavation extent, proposed pump/well positions, discharge route, monitoring points, and nearby receptors.

# Attached Documents

| No. | Document Title | Reference | Revision | Date |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

# Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Brief Prepared By |  |  |  |
| Temporary Works Coordinator |  |  |  |
| Issued to Designer / Specialist |  |  |  |

# Notes / Additional Comments

Regulatory References: BS 5975:2019, CDM 2015, Environmental Permitting (England and Wales) Regulations 2016, CIRIA C750 (Groundwater Control), CIRIA C532 (Control of Water Pollution from Construction Sites).

# Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| 01 |  | Initial issue |  |  |