Trench Box Inspection Checklist

| Document Ref: | Enter document reference |
| --- | --- |
| Revision: | Enter revision number |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract No.: | Enter contract number |
| Prepared By: | Enter name and role |
| Approved By: | Enter name and role |

# Equipment Details

| Trench Box Type: | e.g. Lightweight / Standard / Heavy Duty | Manufacturer: | Enter manufacturer |
| --- | --- | --- | --- |
| Serial Number(s): | Enter panel serial numbers | Rated Depth: | Enter rated depth in metres |
| Actual Trench Depth: | Enter depth in metres | Trench Width: | Enter width in metres |
| Ground Type: | e.g. Clay / Sand / Gravel / Made ground | Water Present: | Yes / No |
| Inspector Name: | Enter name | Inspection Date: | DD / MM / YYYY |

# Inspection Checklist

Mark each item Yes (satisfactory), No (defective), or N/A. Do not allow entry into the trench if any safety-critical item is marked No.

| No. | Inspection Item | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| Panels |  |  |  |  |  |
| 1 | Side panels free from cracks, dents, holes, and excessive corrosion |  |  |  |  |
| 2 | Panel welds intact with no cracking or distortion |  |  |  |  |
| 3 | Panel edges straight and undamaged |  |  |  |  |
| 4 | Panel identification markings legible (serial number, SWL, manufacturer) |  |  |  |  |
| 5 | Panels matched as a pair (same size, type, and manufacturer) |  |  |  |  |
| 6 | Panels within rated depth for trench conditions |  |  |  |  |

| Struts and Spreaders |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 7 | Struts / spreaders correct type and size for panels being used |  |  |  |  |
| 8 | Strut pins / locking devices present, secure, and undamaged |  |  |  |  |
| 9 | Strut adjustment mechanism operating freely |  |  |  |  |
| 10 | Struts set at correct spacing per manufacturer’s guidance |  |  |  |  |
| 11 | No signs of bending, distortion, or damage to struts |  |  |  |  |
| 12 | Strut end connections seated correctly in panel sockets |  |  |  |  |

| Installation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 13 | Trench box installed in accordance with manufacturer’s instructions |  |  |  |  |
| 14 | Panels driven to minimum embedment depth below formation level |  |  |  |  |
| 15 | Trench box extending at least 150 mm above ground level |  |  |  |  |
| 16 | Gap between trench box and excavation face minimised and backfilled where required |  |  |  |  |
| 17 | No surcharge loading within 1 m of trench edge (or as per design) |  |  |  |  |
| 18 | Trench box level and plumb with panels parallel |  |  |  |  |

| Access and Safety |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 19 | Safe means of access / egress provided (ladder extending 1 m above ground) |  |  |  |  |
| 20 | Ladder secured and positioned within 6 m of any working position |  |  |  |  |
| 21 | Edge protection provided where persons could fall into the trench |  |  |  |  |
| 22 | Barriers and warning signs in place around the excavation |  |  |  |  |
| 23 | Stop blocks provided to prevent vehicles approaching the trench edge |  |  |  |  |
| 24 | Services identified and marked (CAT and Genny scan completed) |  |  |  |  |

| Environmental and General |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 25 | Dewatering arrangements adequate and functioning |  |  |  |  |
| 26 | Ground conditions consistent with those assumed in the design / selection |  |  |  |  |
| 27 | No signs of ground movement, cracking, or instability around excavation |  |  |  |  |
| 28 | Daily excavation inspection completed and recorded (CDM Reg. 22) |  |  |  |  |
| 29 | Permit to dig / break ground in place and valid |  |  |  |  |

# Defects Identified

| No. | Defect | Action Required | By Whom | Target Date |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |

# Inspection Outcome

| Trench Box Fit for Use: | Yes / No |
| --- | --- |
| If No, state reason: | Enter details |

# Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Inspector |  |  |  |
| Supervisor / Foreman |  |  |  |
| Temporary Works Coordinator |  |  |  |

# Notes / Additional Comments

Regulatory References: CDM 2015 Regulation 22, BS 5975:2019, HSE Guidance CIS47 (Excavation Safety), PUWER 1998.

# Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| 01 |  | Initial issue |  |  |