Asbestos Management Plan

Health & Safety – Asbestos

| Document Ref: | Enter document reference (e.g. AMP-001) |
| --- | --- |
| Revision: | Enter revision number |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract No: | Enter contract number |
| Site Address: | Enter full site address |
| Client: | Enter client name |
| Principal Contractor: | Enter PC name |
| Prepared By: | Enter name, role and qualifications |
| Approved By: | Enter name and role |

# Approval

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Prepared By |  |  |  |
| Reviewed By |  |  |  |
| Approved By |  |  |  |
| Client Accepted By |  |  |  |

# 1. Introduction and Purpose

This Asbestos Management Plan has been prepared in accordance with the Control of Asbestos Regulations 2012 (CAR 2012), particularly Regulation 4 (Duty to manage asbestos in non-domestic premises) and Regulations 5–15 (work with asbestos). It sets out the arrangements for managing asbestos-containing materials (ACMs) identified on this project to protect workers and others from exposure to asbestos fibres.

The plan covers: identification and recording of ACMs, risk assessment, management of ACMs in situ, procedures for work that may disturb ACMs, emergency procedures, training requirements, and waste disposal arrangements.

# 2. Legal Framework

• Control of Asbestos Regulations 2012 (CAR 2012)

• HSE Approved Code of Practice L143 – Managing and working with asbestos

• HSE Guidance HSG264 – Asbestos: The survey guide

• HSE Guidance HSG248 – Asbestos: The analysts’ guide

• Hazardous Waste Regulations 2005 (as amended)

• CDM 2015 – Pre-construction information must include asbestos survey data

• Control limit: 0.1 fibres per cm³ of air averaged over a continuous 4-hour period (CAR 2012 Regulation 2)

# 3. Project Asbestos Information

## 3.1 Asbestos Survey Details

| Survey Type: | Management Survey / Refurbishment and Demolition (R&D) Survey / Both |
| --- | --- |
| Survey Date: | DD / MM / YYYY |
| Survey Company: | Enter surveying company name |
| UKAS Accreditation No: | Enter UKAS accreditation number |
| Surveyor Name: | Enter surveyor name and qualifications (P402 or equivalent) |
| Survey Report Reference: | Enter report reference number |
| Survey Limitations: | Describe any areas not accessed or limitations noted in the survey report |

## 3.2 Summary of Findings

Summarise the asbestos survey findings. Reference the full survey report for detailed results.

Asbestos identified on site? □ Yes □ No □ Presumed (not sampled) If Yes, number of ACM locations identified: ________

## 3.3 Asbestos-Containing Material Register

Record all identified or presumed ACMs. This register must be kept up to date and accessible to all persons who may be affected.

| ID | Location | Material Type | Asbestos Type | Condition | Risk Score | Action | Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ACM-01 |  |  |  |  |  |  |  |
| ACM-02 |  |  |  |  |  |  |  |
| ACM-03 |  |  |  |  |  |  |  |
| ACM-04 |  |  |  |  |  |  |  |
| ACM-05 |  |  |  |  |  |  |  |
| ACM-06 |  |  |  |  |  |  |  |
| ACM-07 |  |  |  |  |  |  |  |
| ACM-08 |  |  |  |  |  |  |  |
| ACM-09 |  |  |  |  |  |  |  |
| ACM-10 |  |  |  |  |  |  |  |
| ACM-11 |  |  |  |  |  |  |  |
| ACM-12 |  |  |  |  |  |  |  |
| ACM-13 |  |  |  |  |  |  |  |
| ACM-14 |  |  |  |  |  |  |  |
| ACM-15 |  |  |  |  |  |  |  |

Material Type: e.g. Insulating board (AIB), cement products, textured coatings, gaskets, rope seals, floor tiles, roof sheets

Asbestos Type: Chrysotile (white), Amosite (brown), Crocidolite (blue), Presumed

Condition: Good / Fair / Poor / Damaged

Action: Monitor / Encapsulate / Enclose / Remove

# 4. Risk Assessment

Each ACM must be risk-assessed to determine the management action required. The risk assessment considers the material’s condition, likelihood of disturbance and the potential for fibre release.

## 4.1 Material Assessment

Score the material based on product type (1–3), damage/deterioration (0–3), surface treatment (0–2) and asbestos type (1–3). Total score determines the material risk.

## 4.2 Priority Assessment

Score the priority based on occupant activity (0–3), likelihood of disturbance (0–3), human exposure potential (0–3) and maintenance activity (0–3). Total score determines the priority for action.

## 4.3 Combined Risk Matrix

| Risk Category | Score Range | Management Action |
| --- | --- | --- |
| Low Risk | Combined score ≤10 | Monitor – Inspect periodically. Label and record. No immediate action required. |
| Medium Risk | Combined score 11–18 | Manage – Consider encapsulation or enclosure. Increase monitoring. Plan for removal when practicable. |
| High Risk | Combined score >18 | Remove – Arrange removal by a licensed asbestos removal contractor as a priority. |

# 5. Management of ACMs In Situ

Where ACMs are to remain in place (managed in situ), the following controls must be applied:

• ACM locations must be clearly labelled with asbestos warning labels

• ACM locations must be recorded on the asbestos register and marked on site drawings

• The register must be made available to all contractors and persons who may work in areas containing ACMs

• Regular condition inspections must be carried out (minimum 6-monthly, or as determined by the risk assessment)

• Any damage to ACMs must be reported immediately and the area made safe

• A permit-to-work system must be used for any work that may disturb ACMs

• All workers must receive asbestos awareness training (CAR 2012 Regulation 10)

# 6. Work with Asbestos – Categories

| Category | Description | Examples |
| --- | --- | --- |
| Non-Licensed Work (NNLW) | Short-duration, low-risk work with certain ACMs. Does not require a licensed contractor. Must be notified to the HSE. | Removal of textured coatings (Artex), asbestos cement sheets (non-power tools), floor tiles, small gaskets. Maximum 1-hour exposure in 7 days. |
| Notifiable Non-Licensed Work (NNLW) | Work that exceeds non-licensed thresholds but is not licensable. Must notify HSE 14 days before work starts. | Larger-scale asbestos cement removal, encapsulation of AIB, certain maintenance activities. Requires medical surveillance. |
| Licensed Work | Removal of asbestos insulation, sprayed coatings, insulating board (AIB), or any work where exposure may exceed the control limit. Must be carried out by an HSE-licensed contractor. | Removal of pipe lagging, sprayed coatings, AIB, boiler insulation. 14-day notification to HSE. Full enclosure, negative pressure, air monitoring. |

# 7. Emergency Procedures – Suspected Asbestos Disturbance

If asbestos is suspected to have been disturbed or unknown material is found that may contain asbestos, the following procedure must be followed IMMEDIATELY:

1. STOP all work in the area immediately

2. EVACUATE all persons from the affected area. Do NOT attempt to clean up.

3. SEAL OFF the area. Close doors and windows. Prevent access. Post warning signs.

4. NOTIFY the site manager, H&S manager and asbestos coordinator immediately.

5. DECONTAMINATE – Anyone who has been in contact with the suspected material should wash hands and face. Contaminated clothing should be double-bagged for disposal.

6. DO NOT re-enter the area until it has been inspected and cleared by a competent person (UKAS-accredited analyst).

7. ARRANGE for the material to be sampled and analysed by a UKAS-accredited laboratory.

8. RECORD the incident. Complete an incident report form. Notify the HSE if required.

# 8. Training Requirements

| Training | Who Requires It | Applicable To |
| --- | --- | --- |
| Asbestos Awareness (Cat A) | All workers who may encounter ACMs during their normal work. Annual refresher. | All site operatives, supervisors, maintenance staff |
| Non-Licensed Work Training (Cat B) | Workers carrying out non-licensed work with asbestos. | Operatives removing asbestos cement, floor tiles, textured coatings |
| Licensed Removal Training | Operatives working for licensed asbestos removal contractors. | Licensed removal contractors’ operatives only |
| Asbestos Analyst (P403/P404) | Analysts carrying out air monitoring and 4-stage clearance. | UKAS-accredited laboratory analysts |
| Asbestos Surveyor (P402) | Surveyors carrying out management and R&D surveys. | UKAS-accredited survey companies |

# 9. Asbestos Waste Disposal

All asbestos waste must be disposed of in accordance with the Hazardous Waste Regulations 2005 (as amended) and the Environmental Permitting Regulations 2016.

| Licensed Waste Carrier: | Enter waste carrier name and licence number |
| --- | --- |
| Receiving Disposal Site: | Enter permitted disposal site name and permit number |
| Consignment Note Ref: | Enter hazardous waste consignment note reference |
| Waste Description Code: | e.g. 17 06 01* (insulation with asbestos) or 17 06 05* (asbestos cement) |

# 10. ACM Condition Inspection Log

| Date | ACM ID | Location | Condition Observed | Action Taken | Next Review | Initials |
| --- | --- | --- | --- | --- | --- | --- |

# 11. Plan Review

This plan must be reviewed and updated whenever: a new asbestos survey is carried out; ACMs are removed or encapsulated; the condition of any ACM changes; there is an asbestos-related incident; new areas are opened up for refurbishment or demolition. As a minimum, review every 12 months.

| Review Date | Reviewed By | Changes Made | Next Review |
| --- | --- | --- | --- |

# Sign-Off

| Prepared By: |  | Date: |  |
| --- | --- | --- | --- |
| Signature: |  | Qualifications: |  |

| Approved By: |  | Date: |  |
| --- | --- | --- | --- |
| Signature: |  | Role: |  |

# Notes / Additional Comments

Enter any additional notes or comments here.

# Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |