LPG Storage and Use Risk Assessment

Health & Safety – Fire Safety

This assessment covers the storage, handling and use of Liquefied Petroleum Gas (LPG – propane and butane) on construction sites. It addresses compliance with the Dangerous Substances and Explosive Atmospheres Regulations 2002 (DSEAR), the Regulatory Reform (Fire Safety) Order 2005, and HSE guidance on the safe use of LPG. LPG is heavier than air and will collect in low-lying areas, drains and enclosed spaces, creating an explosion risk.

| Document Ref: | Enter document reference |
| --- | --- |
| Revision: | Enter revision number |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract No: | Enter contract number |
| Assessed By: | Enter name and role |
| Approved By: | Enter name and role |

# LPG Inventory

Record all LPG cylinders and bulk storage on site.

| No. | Gas Type / Appliance | Cylinder Size | Quantity | Location | Full/Empty | Responsible Person |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |

# LPG Storage Checklist

| No. | Storage Check | Yes | No | N/A | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | LPG cylinder storage area is located outdoors in a well-ventilated position |  |  |  |  |
| 2 | Storage area is at least 1 metre from any building opening (door, window, air intake) |  |  |  |  |
| 3 | Storage area is at least 2 metres from any drain, opening to a cellar or below-ground area |  |  |  |  |
| 4 | Storage area is at least 3 metres from any source of ignition |  |  |  |  |
| 5 | Storage area is on firm, level ground (not on a slope where cylinders could roll) |  |  |  |  |
| 6 | Storage area is protected from direct sunlight and excessive heat (shaded where necessary) |  |  |  |  |
| 7 | Storage area is protected from vehicle impact (bollards or crash barriers in place) |  |  |  |  |
| 8 | All cylinders are stored upright (valve uppermost) and secured to prevent toppling (chained or caged) |  |  |  |  |
| 9 | Full and empty cylinders are segregated and clearly marked |  |  |  |  |
| 10 | No more than the minimum quantity of LPG is stored on site for current operational needs |  |  |  |  |
| 11 | LPG cylinders are not stored inside temporary accommodation, welfare units or enclosed spaces |  |  |  |  |
| 12 | LPG is not stored near oxygen cylinders (minimum 3-metre separation or fire-resistant barrier) |  |  |  |  |
| 13 | Cylinder storage area is clearly signed with ‘LPG – Highly Flammable – No Smoking – No Naked Flames’ |  |  |  |  |
| 14 | Fire extinguisher (dry powder) is available within 2 metres of the storage area |  |  |  |  |
| 15 | Storage area is included on the site fire plan and known to the fire service |  |  |  |  |
| 16 | Cylinders are not stored below electrical cables or near electrical equipment |  |  |  |  |
| 17 | Access to the storage area is restricted to authorised personnel only |  |  |  |  |
| 18 | Cylinder valves are closed and dust caps fitted when not in use |  |  |  |  |
| 19 | An inventory of LPG cylinders on site is maintained and kept up to date |  |  |  |  |

# LPG Use and Handling Checklist

| No. | Use / Handling Check | Yes | No | N/A | Initials |
| --- | --- | --- | --- | --- | --- |
| 20 | Only competent and trained operatives are authorised to connect, disconnect and use LPG equipment |  |  |  |  |
| 21 | Correct regulator is fitted for the gas type and appliance (check BS EN 16129 compliance) |  |  |  |  |
| 22 | Hoses are in good condition – no cracking, perishing, cuts or kinks |  |  |  |  |
| 23 | Hoses are the correct type for LPG use (orange BS 3212 hose or equivalent) |  |  |  |  |
| 24 | Hose connections are secured with proper clips (not jubilee clips on push-fit connections) |  |  |  |  |
| 25 | Leak testing is carried out on all connections before use (soapy water test – never a naked flame) |  |  |  |  |
| 26 | Flashback arrestors are fitted where oxy-fuel cutting/welding equipment is used |  |  |  |  |
| 27 | LPG heaters are used only in well-ventilated areas – never inside enclosed or poorly ventilated spaces |  |  |  |  |
| 28 | LPG heaters are positioned on stable, non-combustible surfaces at least 1 metre from combustible materials |  |  |  |  |
| 29 | LPG-powered equipment is turned off and cylinders disconnected at the end of each shift |  |  |  |  |
| 30 | Cylinders are never heated, struck or dropped |  |  |  |  |
| 31 | Cylinders are transported upright in a secured position (trolley or vehicle with ventilation) |  |  |  |  |
| 32 | Gas leak emergency procedure is known by all operatives using LPG |  |  |  |  |
| 33 | No smoking within 3 metres of any LPG cylinder or equipment in use |  |  |  |  |

# Minimum Separation Distances

| Separation Requirement | Distance |
| --- | --- |
| LPG cylinders to building openings (doors, windows, air intakes) | 1 metre minimum |
| LPG cylinders to drains, gullies, below-ground openings | 2 metres minimum |
| LPG cylinders to sources of ignition | 3 metres minimum |
| LPG cylinders to oxygen cylinders | 3 metres (or fire-resistant partition) |
| LPG cylinders to site boundary | 1 metre minimum |
| LPG cylinders to temporary accommodation units | 3 metres minimum (no LPG heaters inside cabins) |
| Bulk LPG tank to buildings and boundaries | Refer to LP Gas Association CoP 1 – depends on tank size |

# LPG Emergency Procedure

In the event of an LPG gas leak, fire or explosion:

If a gas leak is detected: extinguish all sources of ignition immediately

Shut off the gas supply at the cylinder valve if safe to do so

Do NOT operate electrical switches (could create a spark)

Evacuate the area – move upwind and at least 25 metres from the leak

Call 999 if the leak cannot be stopped or if fire has started

Notify the site manager immediately

Do not re-enter the area until declared safe by the fire service or competent person

If a cylinder is involved in a fire: evacuate at least 100 metres. Do NOT attempt to fight a fire involving a leaking LPG cylinder. Cool surrounding cylinders with water from a safe distance only if the fire service advises.

# Overall Risk Rating

□ LOW – All controls in place and adequate. □ MEDIUM – Improvements required. Address within agreed timescales. □ HIGH – Significant deficiencies. Immediate action. Consider suspending LPG use until resolved.

# Action Plan

| No. | Action Required | Responsible Person | Target Date | Priority | Status |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |

# Assessor Sign-Off

| Assessed By: |  | Date: |  |
| --- | --- | --- | --- |
| Signature: |  | Review Date: |  |

# Manager Approval

| Approved By: |  | Date: |  |
| --- | --- | --- | --- |
| Signature: |  | Role: |  |

# Notes / Additional Comments

Enter any additional notes or comments here.

# Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |