Pre-Christmas Shutdown Checklist

Health & Safety – Inspection Checklists

| Document Ref: | Enter document reference |
| --- | --- |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract No: | Enter contract number |
| Last Working Day: | DD / MM / YYYY |
| Return to Work Date: | DD / MM / YYYY |
| Shutdown Duration: | Enter number of days |
| Completed By: | Enter name and role |

# Emergency Contacts During Shutdown

| Role / Contact | Name | Phone Number |
| --- | --- | --- |
| Site Manager |  |  |
| Project Manager |  |  |
| Security Company |  |  |
| Security Patrol Contact |  |  |
| Alarm Monitoring Company |  |  |
| Insurance Company |  |  |
| Police (non-emergency) |  |  |
| Fire Service (non-emergency) |  |  |
| Water Leak / Burst |  |  |
| Electricity Emergency (105) |  |  |

# Site Security

| No. | Shutdown Check | Done | N/A | Action | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | Perimeter fencing and hoarding inspected for damage – all breaches repaired |  |  |  |  |
| 2 | All gates and access points secured with heavy-duty padlocks / chains |  |  |  |  |
| 3 | Combination codes or keys issued only to authorised personnel with a need for access during shutdown |  |  |  |  |
| 4 | CCTV system checked, recording and operational for the full shutdown period |  |  |  |  |
| 5 | Intruder alarm tested and set – monitoring company notified of shutdown dates |  |  |  |  |
| 6 | Security patrol arrangements confirmed for the shutdown period (company, frequency, contact details) |  |  |  |  |
| 7 | Security company emergency contact details displayed at the gatehouse and provided to the site manager |  |  |  |  |
| 8 | Valuable materials, tools and equipment secured in locked containers or removed from site |  |  |  |  |
| 9 | Fuel and diesel stores locked, bunded and secure |  |  |  |  |
| 10 | Plant keys removed from all machines and secured in the key safe |  |  |  |  |
| 11 | Signage displayed: ‘SITE CLOSED – SECURITY PATROLS IN OPERATION’ |  |  |  |  |

# Fire Safety

| No. | Shutdown Check | Done | N/A | Action | Initials |
| --- | --- | --- | --- | --- | --- |
| 12 | All hot works ceased and fire watch completed (minimum 60 minutes post-work) |  |  |  |  |
| 13 | LPG cylinders disconnected, valves closed, caps fitted, and stored in the designated storage area |  |  |  |  |
| 14 | All temporary heaters switched off, unplugged and allowed to cool |  |  |  |  |
| 15 | Combustible waste and rubbish cleared from all areas – skips emptied or collected before shutdown |  |  |  |  |
| 16 | Fire escape routes clear and fire doors unobstructed |  |  |  |  |
| 17 | Fire extinguishers checked and accessible |  |  |  |  |
| 18 | Fire alarm panel checked – no outstanding faults. Alarm monitoring company notified. |  |  |  |  |
| 19 | Smoking materials fully extinguished and disposed of safely |  |  |  |  |
| 20 | No flammable materials left near electrical panels or heat sources |  |  |  |  |
| 21 | Christmas decorations (if any) are fire-retardant and not draped over heaters or electrical equipment |  |  |  |  |

# Electrical and Services

| No. | Shutdown Check | Done | N/A | Action | Initials |
| --- | --- | --- | --- | --- | --- |
| 22 | All non-essential electrical supplies isolated at the distribution board |  |  |  |  |
| 23 | Essential supplies identified and left on (frost protection heaters, security lighting, CCTV, alarms, sump pumps) |  |  |  |  |
| 24 | Frost protection heaters checked and functioning – thermostats set correctly |  |  |  |  |
| 25 | Temporary electrical installations inspected – no exposed cables, damaged sockets or trip hazards |  |  |  |  |
| 26 | Water supplies drained down or trace-heated where freeze risk exists (exposed pipework, external taps) |  |  |  |  |
| 27 | Dewatering pumps checked, fuel levels confirmed, automatic operation verified (where applicable) |  |  |  |  |
| 28 | Sump pumps operational to prevent flooding during heavy rainfall over the holiday period |  |  |  |  |
| 29 | Generator fuel levels checked – sufficient for the shutdown period (if providing essential supplies) |  |  |  |  |

# Plant and Equipment

| No. | Shutdown Check | Done | N/A | Action | Initials |
| --- | --- | --- | --- | --- | --- |
| 30 | All plant shut down, parked in designated areas, handbrakes applied, stabilisers lowered |  |  |  |  |
| 31 | Tower cranes left in free slew (weathervane) with jibs clear of obstructions |  |  |  |  |
| 32 | Mobile cranes – booms lowered, outriggers retracted, parked on firm ground |  |  |  |  |
| 33 | MEWPs – platforms lowered, parked, isolated and secured |  |  |  |  |
| 34 | Plant keys removed and secured in the locked key safe / office safe |  |  |  |  |
| 35 | Fuel tanks locked and bunded – no drip trays overflowing |  |  |  |  |
| 36 | AdBlue and hydraulic fluid containers secured |  |  |  |  |
| 37 | Pre-use check records up to date – any defects recorded and reported before shutdown |  |  |  |  |

# Scaffolding and Temporary Works

| No. | Shutdown Check | Done | N/A | Action | Initials |
| --- | --- | --- | --- | --- | --- |
| 38 | All scaffolding inspected – ties, base plates, standards, bracing, boards secure |  |  |  |  |
| 39 | Scaffold tags updated – next inspection date noted (schedule inspection for first day back) |  |  |  |  |
| 40 | Loose scaffold fittings, boards and materials secured or removed from platforms |  |  |  |  |
| 41 | Temporary works (propping, shoring, formwork) inspected and confirmed stable for the shutdown duration |  |  |  |  |
| 42 | Temporary access towers secured – access ladders removed or access hatches locked |  |  |  |  |
| 43 | Netting and sheeting secured to withstand winter storms |  |  |  |  |
| 44 | Edge protection and barriers checked and secure at all open edges, voids and excavations |  |  |  |  |

# Excavations and Earthworks

| No. | Shutdown Check | Done | N/A | Action | Initials |
| --- | --- | --- | --- | --- | --- |
| 45 | All excavations either backfilled or securely fenced with rigid barriers (not just tape) |  |  |  |  |
| 46 | Edge protection around open excavations checked and adequate (1100mm guardrail with toeboard) |  |  |  |  |
| 47 | Excavation support (shoring, sheet piling) inspected and stable |  |  |  |  |
| 48 | Dewatering arrangements operational (pumps running, discharge managed) |  |  |  |  |
| 49 | Spoil heaps stable and not encroaching on excavation edges |  |  |  |  |
| 50 | Warning signage displayed at all open excavations |  |  |  |  |

# Environmental

| No. | Shutdown Check | Done | N/A | Action | Initials |
| --- | --- | --- | --- | --- | --- |
| 51 | Fuel and chemical bunds inspected – no leaks, adequate freeboard for rainfall |  |  |  |  |
| 52 | Spill kits checked and restocked at all fuel and chemical storage areas |  |  |  |  |
| 53 | Silt fences and drainage controls inspected and functional |  |  |  |  |
| 54 | Waste skips covered or emptied to prevent rainwater accumulation and windblown waste |  |  |  |  |
| 55 | No pollutants able to enter drains or watercourses during heavy rainfall |  |  |  |  |
| 56 | Ecological exclusion zones maintained (fencing, signage intact) |  |  |  |  |

# Welfare and Administration

| No. | Shutdown Check | Done | N/A | Action | Initials |
| --- | --- | --- | --- | --- | --- |
| 57 | Welfare cabins locked and secured – all perishable food removed from fridges |  |  |  |  |
| 58 | Fridges and freezers cleaned out and doors left ajar (if switched off) |  |  |  |  |
| 59 | Toilets flushed, cleaned, waste tanks emptied (for portable units) |  |  |  |  |
| 60 | Water supply to welfare drained or protected from freezing (trace heating or lagging) |  |  |  |  |
| 61 | Heating in welfare set to frost-protection mode to prevent pipe bursts |  |  |  |  |
| 62 | Windows and doors closed and locked on all cabins and offices |  |  |  |  |
| 63 | Important documents, drawings and laptops secured or removed from site |  |  |  |  |
| 64 | Post / deliveries – suppliers notified of shutdown dates. Post redirection if required. |  |  |  |  |
| 65 | All subcontractors and suppliers notified of shutdown dates and return-to-work date |  |  |  |  |
| 66 | Client and neighbours notified of shutdown arrangements and emergency contact details |  |  |  |  |

# Return to Work – Re-Opening Actions

The following must be completed on the first day back before work recommences:

| No. | Re-Opening Action | Done | Initials |
| --- | --- | --- | --- |
| 1 | Full site walkabout and inspection carried out by the site manager |  |  |
| 2 | Security check – confirm no break-ins, theft, vandalism or damage during shutdown |  |  |
| 3 | Scaffolding inspection by a competent person before any use (new green tag required) |  |  |
| 4 | Temporary works inspection before any reliance on propping, shoring, formwork |  |  |
| 5 | Excavation inspection before any re-entry |  |  |
| 6 | All plant pre-use checks completed before operation |  |  |
| 7 | Electrical systems re-energised and checked – no water ingress, trip hazards |  |  |
| 8 | Water supplies turned on, flushed through, and checked for leaks |  |  |
| 9 | Welfare facilities cleaned, restocked and operational |  |  |
| 10 | Return-to-work briefing / toolbox talk delivered to all personnel |  |  |

# Shutdown Completed By

I confirm that this checklist has been completed and the site is secure for the Christmas shutdown period.

| Name: |  | Date: |  |
| --- | --- | --- | --- |
| Signature: |  | Time Left Site: |  |

# Project Manager Approval

| Name: |  | Date: |  |
| --- | --- | --- | --- |
| Signature: |  | Role: |  |

# Notes / Additional Comments

Enter any additional notes.

# Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |