Temporary Works Inspection Checklist

Health & Safety – Inspection Checklists

This checklist is used by the Temporary Works Coordinator (TWC) or delegated Temporary Works Supervisor (TWS) to inspect temporary works installations on site. It covers propping, formwork, falsework, excavation support, access structures and crane pads. Inspections must be carried out in accordance with BS 5975 and the site TW procedure. Any defect must be reported immediately and remedied before the TW is loaded or continues in use.

| Document Ref: | Enter document reference |
| --- | --- |
| TW Register Ref: | Enter the temporary works register reference number |
| Date of Inspection: | DD / MM / YYYY |
| Time: | HH:MM |
| Project Name: | Enter project name |
| Contract No: | Enter contract number |
| Inspected By: | Enter name and role (TWC / TWS) |

# Temporary Works Details

| TW Description: | e.g. Propping to slab Level 3, formwork to RC wall W7, trench box at chainage 120 |
| --- | --- |
| TW Type: | e.g. Propping, formwork, falsework, shoring, sheet piling, trench box, crane pad, access road |
| Location: | Enter exact location on site |
| TW Design Ref: | Enter design drawing / calculation reference |
| TW Designer: | Enter name and company of the TW designer |
| Design Checker: | Enter name and company of the independent checker (Category 2/3) |
| Installed By: | Enter name and company of the installer |
| Date Installed: | DD / MM / YYYY |
| TW Category: | 1 (low risk) / 2 (medium) / 3 (high risk) as per BS 5975 |
| Inspection Trigger: | Initial / Routine / Post-event (describe: e.g. after storm, impact, loading) |

# General / Administrative

| No. | Inspection Item | Sat. | Unsat. | N/A | Action / Comment |
| --- | --- | --- | --- | --- | --- |
| 1 | The temporary works item is registered on the TW register with a unique reference number |  |  |  |  |
| 2 | A temporary works design brief has been issued by the TWC to the TW designer |  |  |  |  |
| 3 | The TW design has been completed by a competent designer and checked by an independent checker |  |  |  |  |
| 4 | The TW design has been approved by the TWC before installation commenced |  |  |  |  |
| 5 | Design drawings and calculations are available on site and have been issued to the installer |  |  |  |  |
| 6 | The installer has been briefed on the design requirements and any installation tolerances |  |  |  |  |
| 7 | A method statement for the installation of the temporary works has been produced and briefed |  |  |  |  |
| 8 | A permit to load has been issued (where applicable) confirming the TW is ready to receive load |  |  |  |  |

# Propping and Shoring

| No. | Inspection Item | Sat. | Unsat. | N/A | Action / Comment |
| --- | --- | --- | --- | --- | --- |
| 9 | Props are the correct type, size and capacity as specified in the TW design |  |  |  |  |
| 10 | Props are set at the correct centres and positions as shown on the TW design drawing |  |  |  |  |
| 11 | All props are plumb (vertical) and not leaning or displaced |  |  |  |  |
| 12 | Props have adequate bearing at both top and bottom (sole plates, head plates, forkheads as required) |  |  |  |  |
| 13 | Sole plates are on firm, level bearing and are of adequate size to distribute the load |  |  |  |  |
| 14 | Props are pinned, wedged or locked at the specified extension length – no over-extension |  |  |  |  |
| 15 | Cross-bracing or lacing is installed as specified in the design to prevent buckling |  |  |  |  |
| 16 | No props have been removed, repositioned or altered without authorisation from the TWC |  |  |  |  |
| 17 | Warning signage is displayed: ‘TEMPORARY WORKS – DO NOT ALTER OR REMOVE’ |  |  |  |  |
| 18 | Props are protected from vehicle and plant impact (barriers, bollards) |  |  |  |  |

# Formwork and Falsework

| No. | Inspection Item | Sat. | Unsat. | N/A | Action / Comment |
| --- | --- | --- | --- | --- | --- |
| 19 | Formwork panels are correctly assembled, aligned and secured to the design specification |  |  |  |  |
| 20 | Falsework supports are at the correct centres and heights as per the design |  |  |  |  |
| 21 | All connections, pins, wedges and ties are fully engaged and secure |  |  |  |  |
| 22 | Base plates are on firm, level bearing with sole plates of adequate size |  |  |  |  |
| 23 | The formwork is adequately braced to resist lateral forces (wind, concrete pressure, vibration) |  |  |  |  |
| 24 | Kicker formwork or construction joints are clean and correctly positioned |  |  |  |  |
| 25 | Stop ends are secure and sealed to prevent concrete loss |  |  |  |  |
| 26 | Access and egress from the formwork / falsework is safe (ladders, stair towers) |  |  |  |  |
| 27 | The falsework has been checked for plumb, level and alignment before loading |  |  |  |  |
| 28 | The permit to load / pour has been issued by the TWC or delegated person |  |  |  |  |

# Excavation Support (Sheet Piling, Trench Boxes, Shoring)

| No. | Inspection Item | Sat. | Unsat. | N/A | Action / Comment |
| --- | --- | --- | --- | --- | --- |
| 29 | Excavation support is installed as specified in the TW design |  |  |  |  |
| 30 | Sheet piles are driven to the specified depth and interlocks are engaged |  |  |  |  |
| 31 | Trench boxes are correctly assembled, pinned and seated at the base of the excavation |  |  |  |  |
| 32 | Walers, struts and frames are at the correct levels and are fully tightened / loaded |  |  |  |  |
| 33 | There are no gaps between the support system and the excavation face |  |  |  |  |
| 34 | Groundwater is being managed (dewatering, pumping) and is not undermining the support |  |  |  |  |
| 35 | No surcharge loading (spoil, materials, plant) is within the exclusion zone at the top of the excavation |  |  |  |  |
| 36 | The excavation support has been inspected at the start of each shift and after adverse weather |  |  |  |  |

# Temporary Access Structures and Platforms

| No. | Inspection Item | Sat. | Unsat. | N/A | Action / Comment |
| --- | --- | --- | --- | --- | --- |
| 37 | Temporary access roads, ramps and platforms are constructed as per the TW design |  |  |  |  |
| 38 | Load limits are clearly displayed and are not being exceeded |  |  |  |  |
| 39 | Edge protection is in place on all exposed edges |  |  |  |  |
| 40 | Surfaces are non-slip and maintained in good condition |  |  |  |  |
| 41 | Drainage is adequate to prevent water accumulation and softening of the sub-base |  |  |  |  |
| 42 | The structure is stable and there are no signs of settlement, movement or distress |  |  |  |  |

# Crane Pads, Hardstandings and Outrigger Bases

| No. | Inspection Item | Sat. | Unsat. | N/A | Action / Comment |
| --- | --- | --- | --- | --- | --- |
| 43 | Crane pad / hardstanding has been constructed to the TW design specification |  |  |  |  |
| 44 | Ground bearing capacity has been confirmed and is adequate for the applied load |  |  |  |  |
| 45 | The pad is level and of the correct dimensions and thickness as specified |  |  |  |  |
| 46 | Outrigger pads / spreader mats are correctly positioned and of adequate size |  |  |  |  |
| 47 | No evidence of settlement, cracking or rutting in the hardstanding |  |  |  |  |
| 48 | Drainage around the crane pad is functional and not undermining the bearing capacity |  |  |  |  |

# Actions Required

| No. | Action Required | Responsible | Target Date | Priority | Status |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |

# Overall Inspection Result

□ SATISFACTORY – TW installation is in accordance with the design and may continue in use / receive load. □ MINOR DEFECTS – TW may remain in use but defects must be rectified within the agreed timescale. □ UNSATISFACTORY – TW must NOT be loaded or must be taken out of service immediately. TWC to be notified. Remedial action required before use.

# Permit to Load (if applicable)

If this is an initial inspection before first loading, complete the permit to load section below.

□ PERMIT TO LOAD GRANTED – The temporary works have been inspected and are in accordance with the approved design. Loading may proceed. □ PERMIT TO LOAD REFUSED – Defects identified. Do NOT load. Rectify and re-inspect.

# TWC / TWS Sign-Off

| Inspected By: |  | Date: |  |
| --- | --- | --- | --- |
| Signature: |  | Role: | e.g. TWC / TWS |
| Next Inspection Due: |  | Inspection Frequency: | e.g. Daily / Weekly / Post-event |

# Notes / Additional Comments

Enter any additional notes.

# Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |