Excavator Used as Crane Assessment

Health & Safety – Lifting Operations

Using an excavator as a crane is NOT the preferred method of lifting. A purpose-built crane should always be used where reasonably practicable. Where an excavator must be used for lifting, this assessment must be completed by the Appointed Person in accordance with LOLER 1998, PUWER 1998 and BS 7121-3.

| Document Ref: | Enter document reference |
| --- | --- |
| Lift Plan Ref: | Enter associated lift plan reference |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract No: | Enter contract number |
| Assessed By (AP): | Enter Appointed Person name and qualifications |

# Justification for Using Excavator as Crane

| Why is a crane not available? | Describe why a purpose-built crane cannot be used (access, cost is NOT a valid justification on its own) |
| --- | --- |
| Alternative methods considered: | e.g. Mobile crane, telehandler, HIAB, manual handling, mechanical aids |
| Why are alternatives not practicable? | Describe why each alternative was rejected |

# Excavator Details

| Make / Model: | Enter make and model |
| --- | --- |
| Serial Number: | Enter serial number |
| Operating Weight (tonnes): | Enter operating weight |
| Maximum Lifting Capacity: | Enter rated lifting capacity in crane mode (tonnes at closest radius) |
| Lifting Capacity at Planned Radius: | Enter capacity at the worst-case radius for this lift (tonnes) |
| Crane Mode Available?: | Yes / No – if yes, describe (rated capacity limiter, load moment indicator) |
| LOLER Certificate No: | Enter thorough examination certificate number (must cover lifting duties) |
| LOLER Certificate Expiry: | DD / MM / YYYY |
| Lifting Point Type: | e.g. Purpose-built crane hook, quick-hitch with lifting eye, dipper pin |
| Safety Latch on Hook: | Yes / No |
| Owner / Hirer: | Enter owner or hire company |

# Operator Details

| Operator Name: | Enter name |
| --- | --- |
| CPCS / NPORS Card No: | Enter card number |
| Card Category: | e.g. CPCS A59 (360 Excavator) – confirm lifting endorsement / assessment |
| Card Expiry: | DD / MM / YYYY |
| Lifting Competence Assessment: | Has the operator been specifically assessed for lifting duties? Yes / No. Reference: |
| Experience: | Describe relevant lifting experience with excavators |

# Lift Details

| Load Description: | Describe the load (type, dimensions, material) |
| --- | --- |
| Load Weight (confirmed): | Enter confirmed weight (tonnes). Source: weighbridge / manufacturer / calculation |
| Pick-Up Location: | Enter exact position |
| Set-Down Location: | Enter exact position |
| Maximum Radius: | Enter maximum radius during the lift (metres) |
| Capacity at Max Radius: | Enter excavator lifting capacity at the max radius (tonnes) |
| % Utilisation: | (Load ÷ Capacity) × 100 = ___% (must not exceed 80%) |
| Lift Height: | Enter maximum height the load will reach (metres) |
| Travel With Load?: | Yes / No – if yes, describe controls (generally not recommended) |

# Lifting Accessories

| No. | Description | ID / Serial | SWL (t) | LOLER Due | Pre-Use OK? | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |

# Pre-Lift Checklist

| No. | Pre-Lift Check | Yes | No | N/A | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | A formal justification has been documented as to why a purpose-built crane cannot be used for this lift |  |  |  |  |
| 2 | The excavator manufacturer confirms the machine is suitable for use as a crane (check operator’s manual) |  |  |  |  |
| 3 | A lifting mode is available on the excavator’s operating system (if fitted with a crane function, it must be engaged) |  |  |  |  |
| 4 | The excavator has a current LOLER thorough examination certificate specifically for lifting operations (within 12 months) |  |  |  |  |
| 5 | The lifting capacity chart for the excavator in crane mode is available and has been checked for this lift |  |  |  |  |
| 6 | The load weight has been confirmed (not estimated) and does not exceed the excavator’s rated lifting capacity at the planned radius |  |  |  |  |
| 7 | The percentage utilisation does not exceed 80% of the rated lifting capacity at the maximum radius |  |  |  |  |
| 8 | A designated lifting point is available on the excavator (crane hook on the dipper arm, or approved quick-hitch attachment) |  |  |  |  |
| 9 | The lifting point / hook has a safety catch or latch to prevent accidental disconnection |  |  |  |  |
| 10 | All lifting accessories (slings, shackles, chains) have current LOLER certificates (within 6 months) |  |  |  |  |
| 11 | The operator holds a valid CPCS or NPORS competence card for the excavator AND has been assessed as competent for lifting duties |  |  |  |  |
| 12 | A slinger / signaller is appointed and holds a valid CPCS A62 card (or equivalent) |  |  |  |  |
| 13 | A lift plan has been prepared by the Appointed Person for this lifting operation |  |  |  |  |
| 14 | The ground conditions where the excavator will be positioned have been assessed and are adequate for the applied load |  |  |  |  |
| 15 | The excavator is on firm, level ground and is stable for the lifting operation |  |  |  |  |
| 16 | The excavator tracks are fully extended (tracked machines) to provide maximum stability |  |  |  |  |
| 17 | An exclusion zone has been established and barriered around the lifting operation |  |  |  |  |
| 18 | All non-essential persons have been cleared from the exclusion zone |  |  |  |  |
| 19 | Communication between the operator and slinger has been tested (radio or hand signals) |  |  |  |  |
| 20 | No persons will be lifted or suspended from the excavator – this is PROHIBITED |  |  |  |  |
| 21 | The load will not be swung freely or dragged along the ground – only vertical lifting and controlled slewing |  |  |  |  |
| 22 | The lift will not involve travelling with a suspended load unless the machine is specifically rated for this and a risk assessment has been completed |  |  |  |  |
| 23 | Wind speed has been checked and is within safe limits for the lift |  |  |  |  |
| 24 | Overhead obstructions (OHLs, structures, scaffolding) have been checked and clearances confirmed |  |  |  |  |
| 25 | The emergency / abort procedure has been communicated to the lift team |  |  |  |  |

# Prohibited Activities

Lifting or suspending persons from the excavator is PROHIBITED

Dragging loads along the ground using the excavator is PROHIBITED

Exceeding the rated lifting capacity at any radius is PROHIBITED

Lifting with the excavator on a slope unless specifically assessed and approved is PROHIBITED

Using the bucket as a lifting point (unless a purpose-designed lifting eye is incorporated) is PROHIBITED

# Appointed Person Approval

I confirm that the use of this excavator as a crane has been assessed and is acceptable for this specific lifting operation. All controls are in place and the lift may proceed.

| AP Name: |  | Date: |  |
| --- | --- | --- | --- |
| Signature: |  | Qualifications: |  |

# Site Manager Approval

| Name: |  | Date: |  |
| --- | --- | --- | --- |
| Signature: |  | Role: |  |

# Notes / Additional Comments

Enter any additional notes.

# Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |