Lifting Briefing and Permit Record

Health & Safety – Lifting Operations

This record combines the pre-lift briefing and lifting permit into a single document. It must be completed by the Appointed Person or Crane Supervisor before every lifting operation. The briefing must be delivered to ALL members of the lift team. No lift may proceed until this form is signed by the Appointed Person and accepted by the crane operator.

| Permit / Briefing Ref: | Enter unique reference (e.g. LBP-001) |
| --- | --- |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract No: | Enter contract number |
| Lift Plan Ref: | Enter associated lift plan reference |

# Lift Summary

| Load Description: | Describe the load |
| --- | --- |
| Load Weight (tonnes): | Enter confirmed weight |
| Crane Type: | e.g. Mobile crane, tower crane, crawler crane, telehandler |
| Crane Make / Model: | Enter details |
| Boom / Jib Config: | Enter configuration and length |
| Max Radius (m): | Enter maximum radius during the lift |
| Capacity at Max Radius (t): | Enter rated capacity at worst-case radius |
| % Utilisation: | Enter percentage |
| Lift Category: | Routine / Complex / Critical / Tandem |
| Pick-Up Location: | Enter location |
| Set-Down Location: | Enter location |

# Lift Team

| Role | Name | Card No. | Briefed? |
| --- | --- | --- | --- |
| Appointed Person |  |  |  |
| Crane Supervisor |  |  |  |
| Crane Operator |  |  |  |
| Slinger / Signaller 1 |  |  |  |
| Slinger / Signaller 2 |  |  |  |
| Banksman |  |  |  |
| Tag Line Operative 1 |  |  |  |
| Tag Line Operative 2 |  |  |  |

# Weather Check

| Wind Speed: |  | Max Permitted: |  |
| --- | --- | --- | --- |
| Visibility: |  | Forecast: |  |

# Pre-Lift Briefing Content

The Appointed Person or Crane Supervisor must cover ALL of the following points with the lift team. Tick each item as it is briefed.

| No. | Briefing Point | Covered | Initials |
| --- | --- | --- | --- |
| 1 | Description of the load (type, weight, dimensions, centre of gravity, lifting points) |  |  |
| 2 | Crane type, configuration and capacity at the planned radius |  |  |
| 3 | Rigging arrangement (sling type, number of legs, angle, attachment method) |  |  |
| 4 | Lift sequence (pick-up, lift, slew, travel if applicable, set-down) |  |  |
| 5 | Roles and responsibilities of each lift team member confirmed |  |  |
| 6 | Communication method agreed (radio, hand signals) and tested |  |  |
| 7 | Signal person identified – only ONE person gives signals to the crane operator |  |  |
| 8 | Exclusion zone boundaries and barriering confirmed |  |  |
| 9 | Overhead obstructions identified (power lines, structures, other cranes) |  |  |
| 10 | Ground conditions at pick-up and set-down locations confirmed suitable |  |  |
| 11 | Weather conditions checked – wind speed, visibility, forecast |  |  |
| 12 | Maximum wind speed limit for this lift communicated |  |  |
| 13 | Tag line arrangements confirmed (number, length, positions) |  |  |
| 14 | Emergency / abort procedure explained (what to do if something goes wrong) |  |  |
| 15 | Any special requirements or constraints for this lift discussed |  |  |
| 16 | Questions from the lift team addressed before commencing |  |  |

# Site-Specific Hazards for This Lift

Describe any site-specific hazards relevant to this lift: overhead lines, adjacent structures, public proximity, live services, simultaneous operations, etc.

# Permit to Lift – Authorisation

I confirm that the lift plan has been reviewed, the briefing has been delivered to all lift team members, all pre-lift checks are satisfactory, and the lifting operation may proceed.

| AP / Crane Supervisor: |  | Date: |  |
| --- | --- | --- | --- |
| Signature: |  | Time: |  |

# Accepted By (Crane Operator)

I confirm that I have been briefed, understand the lift plan, and am satisfied that the lift can be carried out safely.

| Operator: |  | Date: |  |
| --- | --- | --- | --- |
| Signature: |  | CPCS Card No: |  |

# Lift Completion

| Lift Completed Successfully: | Yes / No |
| --- | --- |
| Time Completed: | HH:MM |
| Issues / Observations: | Describe any issues, near misses or observations during the lift |
| Lifting Accessories Returned: | Yes / No |
| Exclusion Zone Removed: | Yes / No |

| Closed By: |  | Date / Time: |  |
| --- | --- | --- | --- |
| Signature: |  | Role: |  |

# Notes / Additional Comments

Enter any additional notes.

# Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |