Slinging and Rigging Selection Guide

Health & Safety – Lifting Operations

| Document Ref: | Enter document reference |
| --- | --- |
| Revision: | Enter revision number |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract No: | Enter contract number |
| Prepared By: | Enter name and role |
| Approved By: | Enter name and role |

# 1. Purpose and Scope

This guide assists slingers, riggers, appointed persons and crane supervisors in selecting the correct lifting accessories for each lift. It covers common sling types, shackles, eyebolts and other rigging hardware used in construction. It must be used in conjunction with the site lifting plan, LOLER 1998 and BS 8437.

All lifting accessories must have a current thorough examination certificate (LOLER Regulation 9) and must be inspected before each use by a competent person.

# 2. Sling Angle De-Rating

The capacity of a multi-leg sling reduces as the sling angle decreases from vertical. The included angle between the sling legs must always be considered when selecting slings.

CRITICAL: Never use slings at an included angle greater than 120°. The minimum sling angle from horizontal should be 45° (included angle 90°) unless specifically calculated by a competent person.

| Included Angle Between Legs | Angle from Horizontal | Mode Factor (2-leg) | Mode Factor (3/4-leg) | Capacity Retained |
| --- | --- | --- | --- | --- |
| 0° (vertical) | 90° | 2.0 | 2.1 | 100% |
| 30° | 75° | 1.93 | 2.0 | 97% |
| 60° | 60° | 1.73 | 1.5 | 87% |
| 90° | 45° | 1.41 | 1.06 | 71% |
| 120° (MAXIMUM) | 30° | 1.0 | 0.75 | 50% |

# 3. Sling Type Selection Matrix

Select the appropriate sling type based on the load characteristics, environment and lifting method.

| Sling Type | Characteristics | Best For | Avoid / Caution | LOLER Exam | Key Inspection Point |
| --- | --- | --- | --- | --- | --- |
| Wire Rope Slings | High SWL, durable, heat resistant up to 200°C | Heavy steel, precast, plant | Sharp edges (use corner protectors) | 6 months | Can kink – inspect for broken wires |
| Chain Slings (Grade 80/100) | Very durable, heat resistant up to 200°C (Grade 80), resistant to abrasion | Hot loads, rough surfaces, harsh environments | Heavy – manual handling risk | 6 months | Check for elongation >5% |
| Polyester Webbing Slings | Lightweight, non-marking, flexible | Finished goods, glazing, painted surfaces, pipe | Avoid sharp edges, chemicals, UV | 6 months | Cut easily – use corner protectors |
| Polyester Round Slings | Lightweight, flexible, equal strength all round | Pipes, columns, awkward shapes | Avoid sharp edges, heat, chemicals | 6 months | Inspect inner core if outer damaged |
| High-Performance Fibre Slings | Very high SWL-to-weight ratio | Heavy lifts where weight is critical | Avoid sharp edges, UV | 6 months | Specialist inspection required |

# 4. Shackle Selection

| Shackle Type | Use For | Notes |
| --- | --- | --- |
| Bow Shackle (Omega) | Multi-directional loading, sling connections, wide jaw for multiple slings | Most common. Use where sling angle may vary. |
| Dee Shackle (D-shape) | In-line loading only, connecting to fixed lugs or eyebolts | Only for straight pull – never side-loaded. |
| Wide Body / Super Shackle | Heavy-duty multi-sling connections, large bearing area | For heavy lifts with multiple slings. |

Shackle Rules:

• Pin must always pass through the sling eye, NOT through the load

• Pin must be fully tightened (hand-tight plus 1/4 turn)

• Never replace a shackle pin with a bolt

• Never side-load a D-shackle

• SWL/WLL must be clearly stamped and legible

# 5. Eyebolt Guidance

| Type | Use | Critical Notes |
| --- | --- | --- |
| Collar Eyebolt (BS 4278) | Vertical lifting only (straight pull in line with the bolt axis) | NEVER side-load. If angular pull is required, use a swivel hoist ring. |
| Dynamo Eyebolt | Vertical and angular loading permitted | De-rate at angles: 45° = 50% SWL, 90° = 25% SWL. |
| Swivel Hoist Ring | Multi-directional loading, angular pull | Most versatile. Rated for angular loads. Specify correct bolt size and grade. |

# 6. Sling Colour Coding System

Many sites use a colour coding system to identify slings by their thorough examination date. Replace this with your site-specific colour code.

| Period | Colour | Tag / Band | Exam Due By | Status |
| --- | --- | --- | --- | --- |
| Jan–Jun | Green | Green | June 30 |  |
| Jul–Dec | Blue | Blue | December 31 |  |

Any sling without a valid colour code or with an expired examination certificate must be immediately quarantined and removed from service.

# 7. Pre-Use Inspection Checklist

Complete this checklist before using any lifting accessory. If any item fails, quarantine the equipment and report to the lifting coordinator.

| No. | Pre-Use Check | Pass | Fail | N/A | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | SWL / WLL is clearly marked on the equipment and is adequate for the load |  |  |  |  |
| 2 | Identification tag / unique ID number is present and legible |  |  |  |  |
| 3 | Current LOLER thorough examination certificate is in date (within 6 months for lifting accessories) |  |  |  |  |
| 4 | No visible signs of damage, wear, corrosion or distortion |  |  |  |  |
| 5 | No broken wires, kinks, crushing or birdcaging in wire rope slings |  |  |  |  |
| 6 | No cuts, abrasions, chemical damage or UV degradation in synthetic slings |  |  |  |  |
| 7 | No elongation or deformation in chain slings (links stretched, twisted or worn) |  |  |  |  |
| 8 | Safety latches on hooks are operational and undamaged |  |  |  |  |
| 9 | Master links, coupling links and end fittings are in good condition |  |  |  |  |
| 10 | Shackle pins are tight and correctly oriented (pin through the sling eye, not the load) |  |  |  |  |
| 11 | Equipment is clean and free from contaminants (oil, chemicals, grit) |  |  |  |  |
| 12 | Equipment has not been subjected to overloading or shock loading |  |  |  |  |
| 13 | Equipment has not been repaired or modified without authorisation |  |  |  |  |
| 14 | Protective sleeves and corner protectors are available and in good condition |  |  |  |  |

# 8. Rigging Selection Record

Complete this section when selecting lifting accessories for a specific lift. This forms part of the lift plan documentation.

| Lift Plan Reference: | Enter lift plan reference number |
| --- | --- |
| Load Description: | Describe the load to be lifted |
| Load Weight (tonnes): | Enter confirmed weight of the load |
| Lifting Points: | Describe the lifting points on the load |
| Number of Sling Legs: | 1 / 2 / 3 / 4 |
| Estimated Sling Angle: | Enter the included angle between sling legs |
| Required SWL (calculated): | Enter the minimum SWL required after de-rating for angle |

| No. | Equipment Type | ID / Serial | SWL / WLL | Exam Due | Pre-Use OK? | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |

# 9. Rejection Criteria Summary

Remove from service immediately if any of the following are found:

| Equipment | Reject If |
| --- | --- |
| Wire Rope Slings | Broken wires exceeding the limit in BS EN 13414; kinks; crushing; birdcaging; corrosion; heat damage; distorted ferrules or thimbles |
| Chain Slings | Elongation >5% on any link; bent or twisted links; wear >10% of cross section; cracks or gouges; heat discolouration |
| Webbing Slings | Cuts through any load-bearing stitching; chemical damage; UV degradation (powdery surface); burns or melting; legible label missing |
| Round Slings | Outer cover damaged exposing inner yarns; chemical damage; knots; abrasion; label unreadable |
| Shackles | Pin bent or worn; body distorted; thread damaged; SWL marking illegible; crown worn or cracked |
| All Equipment | LOLER certificate expired; no identification tag; evidence of unauthorised repair; evidence of overloading |

# Sign-Off

| Selected By: |  | Date: |  |
| --- | --- | --- | --- |
| Signature: |  | Competence: | e.g. CPCS A62 Slinger/Signaller |

| Checked By (AP): |  | Date: |  |
| --- | --- | --- | --- |
| Signature: |  | Role: |  |

# Notes / Additional Comments

Enter any additional notes or comments here.

# Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |