Ground Penetration Permit

Health & Safety – Permits to Work

WARNING: No ground penetration (excavation, piling, drilling, driving posts, anchor installation or any other intrusive activity) is permitted without this permit being issued and displayed at the point of work.

This permit is issued in accordance with HSE guidance HSG47 ‘Avoiding danger from underground services’ and site-specific safe digging procedures. It must be completed before any ground penetration activity commences.

| Permit No: | Enter unique permit number (e.g. GPP-001) |
| --- | --- |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract No: | Enter contract number |
| Issued By: | Enter name, role and company of permit issuer |

# Permit Validity

| Valid From: | DD / MM / YYYY HH:MM | Valid Until: | DD / MM / YYYY HH:MM |
| --- | --- | --- | --- |
| Shift: | Day / Night | Max Duration: | This permit is valid for one shift only |

# Work Details

| Description of Work: | Describe the ground penetration activity (e.g. trench excavation, trial pit, piling, post driving) |
| --- | --- |
| Location: | Enter exact location (grid ref, chainage, drawing ref, GPS coordinates) |
| Maximum Depth (m): | Enter maximum planned depth of penetration |
| Method of Excavation: | e.g. Hand dig, mechanical excavator, vacuum excavation, augering |
| Contractor / Operative: | Enter name and company of person carrying out the work |
| Supervisor: | Enter name and company of person supervising the work |
| Relevant Drawing Refs: | Enter drawing references showing the work area and known services |

# Buried Services Identified

Record all services identified from utility records, site drawings and detection surveys. Mark their approximate position on the attached sketch.

| No. | Service Type | Colour Code | Approx. Depth | Detected By | Live? | Hand Dig Zone? |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Electricity (HV) | Red |  |  |  |  |
| 2 | Electricity (LV) | Red |  |  |  |  |
| 3 | Gas (HP) | Yellow |  |  |  |  |
| 4 | Gas (MP/LP) | Yellow |  |  |  |  |
| 5 | Water | Blue |  |  |  |  |
| 6 | Telecommunications | Grey / Green |  |  |  |  |
| 7 | Fibre Optic | Grey / Green |  |  |  |  |
| 8 | Drainage / Sewer |  |  |  |  |  |
| 9 | Other (specify) |  |  |  |  |  |

# Service Detection Survey Details

| CAT Operative Name: | Enter name |
| --- | --- |
| CAT Competence Card: | Enter card type and number (e.g. CSCS, EUSR) |
| CAT Make / Model: | Enter make and model |
| CAT Serial No: | Enter serial number |
| CAT Calibration Date: | DD / MM / YYYY |
| Genny Make / Model: | Enter make and model |
| Genny Serial No: | Enter serial number |
| Modes Used: | Power / Radio / Genny (all three must be used) |
| GPR Survey Completed: | Yes / No – if yes, enter operator and equipment details |
| Survey Date: | DD / MM / YYYY |
| Survey Drawing / Sketch Ref: | Enter reference of the marked-up survey drawing (attach copy) |

# Pre-Dig Safety Checklist

All items must be confirmed before ground penetration commences. Any item marked ‘No’ must be resolved before work can proceed.

| No. | Pre-Dig Check | Yes | No | N/A | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | Statutory utility records (asset plans) have been obtained from all utility providers for the work area |  |  |  |  |
| 2 | Private service records and site drawings have been reviewed for known services |  |  |  |  |
| 3 | As-built records and previous excavation records have been checked |  |  |  |  |
| 4 | A desk study / site reconnaissance has been completed to identify above-ground indicators (valve covers, marker posts, junction boxes, vent pipes, manholes) |  |  |  |  |
| 5 | CAT (Cable Avoidance Tool) and Genny (Signal Generator) survey has been completed by a competent operative |  |  |  |  |
| 6 | CAT operative holds a valid competence card (CSCS, EUSR or equivalent) |  |  |  |  |
| 7 | CAT and Genny equipment is within calibration date and in good condition |  |  |  |  |
| 8 | GPR (Ground Penetrating Radar) survey has been completed where required (congested services, critical areas) |  |  |  |  |
| 9 | All detected services have been clearly marked on the ground surface using paint / markers |  |  |  |  |
| 10 | Service detection results have been recorded on a drawing / sketch (attached to this permit) |  |  |  |  |
| 11 | Hand dig zone has been established around all detected and suspected services (minimum 500mm either side) |  |  |  |  |
| 12 | No mechanical excavation is permitted within the hand dig zone without specific additional controls |  |  |  |  |
| 13 | Trial holes / hand dug investigation pits have been completed where required to confirm service positions |  |  |  |  |
| 14 | All identified services have been confirmed as live or dead (where possible) |  |  |  |  |
| 15 | Service strike emergency procedures have been communicated to all operatives |  |  |  |  |
| 16 | Operatives are trained in safe digging practices (HSG47 guidance) |  |  |  |  |
| 17 | Appropriate hand tools are available (insulated spades, non-metallic hand tools for near cables) |  |  |  |  |
| 18 | Emergency contact numbers for utility providers are available on site |  |  |  |  |
| 19 | Method statement for the ground penetration works has been briefed to all operatives |  |  |  |  |
| 20 | Exclusion zone around the excavation has been established (barriers, signage) |  |  |  |  |

# Service Strike Emergency Procedure

In the event of a service strike, the following actions must be taken immediately:

ELECTRICITY: Stop work immediately. Do NOT touch any exposed cable. Keep everyone at least 5 metres away. Call 105 (electricity network operator) and site emergency number. Do NOT attempt to disconnect.

GAS: Stop work immediately. Eliminate all sources of ignition (engines, cigarettes, electrical switches). Evacuate the area. Call National Gas Emergency 0800 111 999 and site emergency number.

WATER: Stop work. Attempt to isolate if safe to do so. Contact the water company emergency number. Manage flooding and prevent contaminated water entering watercourses.

TELECOMS / FIBRE: Stop work. Do not touch exposed cables (fibre optic can cause eye injury). Contact the telecoms provider. Report to site management.

ALL STRIKES: Report to the site manager and H&S manager immediately. Complete an incident report. Preserve the scene. Do not backfill until authorised.

| Utility | Emergency Number | Site Contact |
| --- | --- | --- |
| Electricity (DNO) | 105 |  |
| Gas (GDN) | 0800 111 999 |  |
| Water |  |  |
| Telecoms |  |  |

# Permit Authorisation

I confirm that all pre-dig checks have been completed, services have been detected and marked, hand dig zones established, and all operatives have been briefed. Ground penetration may proceed in accordance with this permit.

| Permit Issuer: |  | Date: |  |
| --- | --- | --- | --- |
| Signature: |  | Time: |  |

# Permit Accepted By

I confirm that I have read and understood this permit, the service locations, the hand dig zones and the emergency procedures. I will comply with all conditions.

| Operative Name: |  | Date: |  |
| --- | --- | --- | --- |
| Signature: |  | Company: |  |

# Permit Close-Out

| Date / Time Closed: | DD / MM / YYYY HH:MM |
| --- | --- |
| Work Completed: | Yes / No – if No, describe status |
| Services Encountered: | Yes / No – if yes, describe (type, depth, condition) |
| Any Service Strikes: | Yes / No – if yes, complete incident report |
| Excavation Secured: | Yes / No (backfilled, barriers, covers) |

| Closed By: |  | Date: |  |
| --- | --- | --- | --- |
| Signature: |  | Role: |  |

# Service Location Sketch

Sketch the work area showing detected service locations, hand dig zones, excavation extent and access routes. Attach a separate drawing if required.

Sketch area

# Notes / Additional Comments

Enter any additional notes or comments here.

# Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |