Waterproofing Membrane Inspection Record

Inspection record for waterproofing membrane installations. For use on below-ground structures, basements, tunnels, roofs, and other applications where watertight integrity is required. Completed in accordance with BS 8102:2022 (Protection of below-ground structures against water ingress), BS 8215, and the membrane manufacturer installation requirements.

Project Details

| Document Ref: | Enter document reference |
| --- | --- |
| Revision: | Enter revision number |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| Location / Structure: | Enter specific structure or area |
| Waterproofing Contractor: | Enter subcontractor name |
| Prepared By: | Name / role |
| Approved By: | Name / role |

Membrane Material Details

| Membrane Product: | Enter manufacturer and product name |
| --- | --- |
| Membrane Type: | e.g. torch-applied bituminous, self-adhesive, liquid-applied, PVC, HDPE, bentonite |
| Specification Reference: | Enter spec clause number |
| Membrane Thickness (mm): | Enter nominal thickness |
| Batch Number(s): | Enter batch numbers used |
| Area Applied (m²): | Enter total area covered |
| Primer / Adhesive Used: | Enter product name and batch |
| Protection Layer: | e.g. protection board, geotextile, screed |

Environmental Conditions Check

| No. | Inspection Item | Y / N / NA | Comments |
| --- | --- | --- | --- |
| 26 | Ambient temperature within manufacturer application range at time of installation |  |  |
| 27 | Substrate temperature within manufacturer application range |  |  |
| 28 | No rain, standing water, frost, or condensation on substrate at time of application |  |  |
| 29 | Wind speed acceptable for torch-applied or spray-applied membranes |  |  |
| 30 | Adequate ventilation provided for solvent-based or hot-applied products |  |  |

Substrate Preparation Inspection

| No. | Inspection Item | Y / N / NA | Comments |
| --- | --- | --- | --- |
| 1 | Substrate is clean, dry, and free from loose material, dust, oil, and grease |  |  |
| 2 | Substrate surface is smooth and free from sharp protrusions, honeycombing, and voids |  |  |
| 3 | All fillets / chamfers formed at internal angles (min 50mm triangular fillet or 75mm radius cove) |  |  |
| 4 | Concrete surface has been cured for minimum period per membrane manufacturer requirements |  |  |
| 5 | Substrate moisture content tested and within acceptable limits for adhesion |  |  |
| 6 | All construction joints, movement joints, and cracks have been identified and treated |  |  |
| 7 | Surface preparation method applied as specified (e.g. grit blast, scabble, grind, primer coat) |  |  |
| 8 | Primer / adhesive applied at correct coverage rate and allowed to tack off / cure as specified |  |  |
| 9 | Any levelling screed or protection board installed where required by specification |  |  |
| 10 | All penetrations (pipes, ducts, fixings) identified and prepped for detailing |  |  |

Membrane Installation Inspection

| No. | Inspection Item | Y / N / NA | Comments |
| --- | --- | --- | --- |
| 11 | Membrane product matches approved specification and material approval submission |  |  |
| 12 | Membrane batch numbers recorded and traceable to delivery tickets |  |  |
| 13 | Membrane stored correctly prior to use (dry, flat, within temperature range per manufacturer) |  |  |
| 14 | Membrane applied in correct direction (e.g. positive or negative side as specified) |  |  |
| 15 | Membrane laid with minimum overlap as per manufacturer requirements (typically 100-150mm) |  |  |
| 16 | All laps / joints sealed / welded in accordance with manufacturer instructions |  |  |
| 17 | Membrane fully adhered / mechanically fixed / loose-laid with ballast as per specification |  |  |
| 18 | Details at penetrations formed correctly using pre-formed boots, collars, or site-fabricated details |  |  |
| 19 | Movement joints and construction joints detailed with appropriate proprietary details |  |  |
| 20 | Membrane turned into termination details (chases, mechanical bars, sealant) correctly |  |  |
| 21 | No damage, punctures, tears, or fish-mouths observed in membrane surface |  |  |
| 22 | Membrane protected from UV exposure, traffic, and other trades as required |  |  |
| 23 | Protection board / fleece layer installed where specified before backfilling or covering |  |  |
| 24 | Flood / leak test or spark test carried out where required by specification |  |  |
| 25 | All waterstops and injection tubes installed at construction joints where specified |  |  |

Inspection Result

| Overall Result: | Pass / Fail / Conditional — delete as appropriate |
| --- | --- |
| Remedial Actions Required: | Detail any remedial actions needed before covering / backfilling |
| Permission to Cover: | Yes / No — structure may be covered / backfilled |

Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Waterproofing Installer / Foreman |  |  | DD / MM / YYYY |
| Membrane Manufacturer Inspector (if reqd) |  |  | DD / MM / YYYY |
| Site Engineer (Main Contractor) |  |  | DD / MM / YYYY |
| Design Engineer / Waterproofing Designer |  |  | DD / MM / YYYY |

Notes / Additional Comments

Enter any additional notes, photographic references, or sketch references here.

Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| A |  | Initial issue |  |  |