Structural Steel Erection Checklist

Comprehensive checklist for the erection, inspection, and sign-off of structural steelwork. For use by site engineers and steelwork supervisors to verify compliance with the National Structural Steelwork Specification (NSSS, 7th Edition), BS EN 1090-2 (Execution of steel structures), LOLER 1998, and the project specification.

Project Details

| Document Ref: | Enter document reference |
| --- | --- |
| Revision: | Enter revision number |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| Erection Zone / Area: | Enter zone, gridlines, or phase reference |
| Steelwork Contractor: | Enter steelwork erector name |
| Steelwork Fabricator: | Enter fabricator name and CE marking ref |
| Execution Class: | e.g. EXC2, EXC3 per BS EN 1090-2 |
| Prepared By: | Name / role |
| Approved By: | Name / role |

Section 1 — Pre-Erection Checks

| No. | Inspection Item | Yes | No / NA | Comments / Action |
| --- | --- | --- | --- | --- |
| 1 | Approved steelwork erection method statement and risk assessment in place |  |  |  |
| 2 | Lifting plan approved for all lifts (routine and non-routine) in accordance with LOLER 1998 |  |  |  |
| 3 | Crane / lifting equipment inspected and current thorough examination certificates available |  |  |  |
| 4 | All steelwork members checked against delivery schedule and GA drawings before offloading |  |  |  |
| 5 | Members visually inspected for transport damage, distortion, or coating defects on arrival |  |  |  |
| 6 | Steel identification marks (piece marks) legible and match erection drawings |  |  |  |
| 7 | Foundation / base plate holding-down bolts checked for position, level, and projection |  |  |  |
| 8 | Foundation / base plate levels and positions surveyed and within tolerance |  |  |  |
| 9 | Grout pockets / packs prepared and levelling shims available |  |  |  |
| 10 | Temporary works design approved (bracing, propping, guy ropes) and in place |  |  |  |
| 11 | Exclusion zones established and banksmen / signallers briefed |  |  |  |
| 12 | Weather conditions assessed — wind speed within safe limits for erection |  |  |  |

Section 2 — During Erection Checks

| No. | Inspection Item | Yes | No / NA | Comments / Action |
| --- | --- | --- | --- | --- |
| 13 | Columns plumbed and aligned within tolerance before releasing crane |  |  |  |
| 14 | Column base plate connections made with correct number and grade of bolts |  |  |  |
| 15 | Holding-down bolt nuts tightened to specified torque and witnessed |  |  |  |
| 16 | Primary beams connected with correct bolts to design specification |  |  |  |
| 17 | Secondary beams and purlins fixed in correct sequence per erection method statement |  |  |  |
| 18 | All bolted connections made with correct bolt grade, diameter, and length (BS EN 14399 / BS EN 15048) |  |  |  |
| 19 | HSFG bolts tightened using approved method (torque, turn-of-nut, or DTI washers) |  |  |  |
| 20 | Bolt tightening sequence followed for multi-bolt connections |  |  |  |
| 21 | Packing plates and shims installed where specified |  |  |  |
| 22 | Temporary erection cleats and bracing installed at each stage before progressing |  |  |  |
| 23 | Column splices aligned and bolted / welded as per connection design |  |  |  |
| 24 | Moment connections assembled in correct sequence with all components fitted |  |  |  |
| 25 | Steelwork alignment and plumb checked by survey at each stage |  |  |  |
| 26 | Beam levels checked and within tolerance (typical +/- 5mm or per NSSS) |  |  |  |
| 27 | Overall frame stability achieved before removing temporary bracing |  |  |  |

Section 3 — Post-Erection and Completion Checks

| No. | Inspection Item | Yes | No / NA | Comments / Action |
| --- | --- | --- | --- | --- |
| 28 | All permanent bracing installed and connections completed |  |  |  |
| 29 | All connections fully tightened — no temporary or partially tightened bolts remaining |  |  |  |
| 30 | Bolt inspections completed: visual check, torque check or DTI gap check recorded |  |  |  |
| 31 | Site welds completed and inspected by qualified welding inspector (BS EN ISO 5817) |  |  |  |
| 32 | NDT of site welds carried out where specified (UT, MPI, DPI) and results recorded |  |  |  |
| 33 | Grout / bedding beneath base plates placed and cured (non-shrink grout to spec) |  |  |  |
| 34 | Steelwork alignment survey completed and recorded (as-built vs design comparison) |  |  |  |
| 35 | Surface preparation and paint / fire protection touch-up carried out at connections |  |  |  |
| 36 | Fire protection system installed to connections and splices where required |  |  |  |
| 37 | Temporary erection aids (cleats, lugs, brackets) removed if specified |  |  |  |
| 38 | Snag list completed and all items closed out |  |  |  |
| 39 | As-built mark-ups provided to design team and records filed |  |  |  |

Section 4 — Health and Safety Checks

| No. | Inspection Item | Yes | No / NA | Comments / Action |
| --- | --- | --- | --- | --- |
| 40 | Operatives hold valid CSCS cards and steelwork-specific competency |  |  |  |
| 41 | All operatives briefed on erection sequence, exclusion zones, and emergency procedures |  |  |  |
| 42 | Fall protection in place: safety nets, air bags, harnesses with lanyards to suitable anchors |  |  |  |
| 43 | Edge protection installed at all open edges and floor openings |  |  |  |
| 44 | Access equipment (MEWPs, scaffolding, ladders) inspected and suitable for the task |  |  |  |
| 45 | Tools and equipment secured against falling (tethered tools, bolt bags, magnetic trays) |  |  |  |
| 46 | Communication plan in place between crane operator, banksman, and erection team |  |  |  |
| 47 | Fire risk assessment completed — hot works permit in place for site welding / burning |  |  |  |

Inspection Result

| Overall Result: | Pass / Fail / Conditional |
| --- | --- |
| Outstanding Actions: | Detail any items requiring rectification before handover |
| NCR Raised: | Yes / No — if yes, enter NCR reference number |

Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Steelwork Erection Foreman |  |  | DD / MM / YYYY |
| Steelwork Inspector (Fabricator QA) |  |  | DD / MM / YYYY |
| Site Engineer (Main Contractor) |  |  | DD / MM / YYYY |
| Structural Engineer / Designer (if reqd) |  |  | DD / MM / YYYY |

Notes / Additional Comments

Enter any additional notes, survey references, or photographic records here.

Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| A |  | Initial issue |  |  |