Concrete Pre-Pour Checklist

Pre-pour inspection checklist to be completed before any concrete pour commences. Confirms formwork, reinforcement, cast-in items, and logistics are ready. The hold point inspection — concrete shall not be placed until this checklist is signed off. In accordance with BS EN 13670 (Execution of concrete structures) and the project ITP.

Details

| Pour Reference: | Enter pour number |
| --- | --- |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| Element: | Enter structural element (e.g. slab L02 Zone A, foundation F14, wall W3) |
| Drawing Reference: | Enter structural drawing number and revision |
| Concrete Mix: | Enter BS 8500 designation |
| Estimated Volume (m³): | Enter planned pour volume |
| Pour Start Time (Planned): | HH:MM |
| Inspected By: | Enter site engineer / inspector name |

Checklist

| No. | Check Item | Y / N / NA | Comments |
| --- | --- | --- | --- |
| 1 | Pour reference and drawing number confirmed on site |  |  |
| 2 | Formwork / shutters erected to correct dimensions, line, and level per design |  |  |
| 3 | Formwork adequately supported — propping / falsework design followed and TWC permit to load issued |  |  |
| 4 | Formwork clean, free from debris, and release agent applied |  |  |
| 5 | Reinforcement installed per bar bending schedule and structural drawing |  |  |
| 6 | Reinforcement spacers and chairs in position — correct cover achieved |  |  |
| 7 | Reinforcement tied securely — no loose bars, laps checked, continuity confirmed |  |  |
| 8 | Cast-in items installed (holding-down bolts, ferrules, starter bars, ducts, inserts) |  |  |
| 9 | Construction joints prepared — clean, roughened, bonding agent applied if specified |  |  |
| 10 | Waterstops and hydrophilic strips installed at joints (if specified) |  |  |
| 11 | Box-outs and voids formed correctly per drawing |  |  |
| 12 | Services, sleeves, and penetrations installed and coordinates checked |  |  |
| 13 | Concrete cover checked with cover meter (if existing reinforcement) |  |  |
| 14 | Pre-pour inspection by site engineer / structural engineer completed |  |  |
| 15 | Formwork and reinforcement inspection signed off — hold point released |  |  |
| 16 | Concrete mix designation confirmed — order placed with ready-mix plant |  |  |
| 17 | Delivery route confirmed — access for wagons and pump checked |  |  |
| 18 | Concrete pump set up (if applicable) — pipeline secure, boom reach confirmed |  |  |
| 19 | Vibrators available and tested — correct type for the section |  |  |
| 20 | Finishing tools and equipment available (float, tamp, screed rail) |  |  |
| 21 | Curing materials available on site (polythene, curing compound, wet hessian) |  |  |
| 22 | Test cube moulds available — cube sets prepared per testing schedule |  |  |
| 23 | Weather forecast checked — no heavy rain, frost, or extreme heat expected |  |  |
| 24 | Operatives briefed on pour sequence, access, and emergency procedures |  |  |
| 25 | Adjacent areas protected from concrete splash and spillage |  |  |

Notes / Additional Comments

Enter any additional notes here.

Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Site Engineer |  |  | DD / MM / YYYY |
| Structural Engineer (hold point witness) |  |  | DD / MM / YYYY |
| Concrete Foreman |  |  | DD / MM / YYYY |
| Formwork Supervisor |  |  | DD / MM / YYYY |

Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| A |  | Initial issue |  |  |