Post-Tensioning Record Sheet

Record of post-tensioning operations for prestressed concrete structures. Documents strand/tendon details, stressing sequence, elongation measurements, lock-off forces, and grouting. In accordance with BS EN 13670, BS EN 1992-1-1 (Eurocode 2), and the post-tensioning system supplier specifications.

Project Details

| Document Ref: | Enter document reference |
| --- | --- |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| Element / Pour: | Enter structural element |
| PT System Supplier: | Enter post-tensioning system name and supplier |

Tendon Details

| Tendon Reference: | Enter tendon ID from PT drawings |
| --- | --- |
| Tendon Type: | e.g. bonded / unbonded, internal / external |
| Strand Type: | e.g. 7-wire, 15.7mm dia, 1860 grade, low relaxation |
| Number of Strands: | Enter number per tendon |
| Duct Type: | e.g. corrugated steel, HDPE, flat duct |
| Duct Size (mm): | Enter internal diameter |
| Stressing End: | Live end / Dead end / Both ends |

Stressing Record

| Stressing Date: | DD / MM / YYYY |
| --- | --- |
| Concrete Strength at Stressing (N/mm²): | Enter — must meet minimum per design (typically 25-35 N/mm²) |
| Cube Reference: | Enter cube test reference confirming strength |
| Jack Type: | Enter jack make, model, and serial number |
| Jack Calibration Date: | DD / MM / YYYY |
| Jack Calibration Certificate Ref: | Enter certificate reference |
| Design Jacking Force (kN): | Enter per strand and per tendon |
| Actual Jacking Force (kN): | Enter measured force |
| Design Elongation (mm): | Enter calculated elongation |
| Measured Elongation (mm): | Enter actual elongation |
| Elongation Tolerance: | Enter +/- % tolerance (typically +/- 5% per EC2) |
| Elongation Within Tolerance: | Yes / No — if No, stop and investigate |
| Lock-Off Force (kN): | Enter force after wedge seating |
| Wedge Draw-In (mm): | Enter measured draw-in |

Grouting (Bonded Tendons)

| Grouting Date: | DD / MM / YYYY |
| --- | --- |
| Grout Product: | Enter grout name and manufacturer |
| Grout w/c Ratio: | Enter water/cement ratio |
| Grout Flow Time (seconds): | Enter Marsh cone flow time |
| Grout Injection Pressure (bar): | Enter maximum pressure |
| Grout Emerged from Vent / Far End: | Yes — confirm full duct grouted |
| Vent Ports Sealed: | Yes — enter sequence of closure |
| Grout Cube Made: | Yes / No — enter reference |

Notes / Additional Comments

Enter any additional notes or supporting documentation references here.

Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| PT Stressing Engineer |  |  | DD / MM / YYYY |
| Site Engineer |  |  | DD / MM / YYYY |
| Structural Engineer |  |  | DD / MM / YYYY |
| PT System Supplier Rep |  |  | DD / MM / YYYY |

Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| A |  | Initial issue |  |  |