Pipe Bedding and Surround Checklist

Inspection checklist for pipe bedding, surround, and initial backfill on drainage and pipeline installations. To be completed per pipe run or section and retained as a permanent quality record. In accordance with BS EN 1610 (Installation and testing of drains and sewers), Sewers for Adoption / Design and Construction Guidance, and the project specification.

Project Details

| Document Ref: | Enter document reference |
| --- | --- |
| Revision: | Enter revision number |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| Drainage / Pipeline Contractor: | Enter contractor name |
| Prepared By: | Name / role |
| Approved By: | Name / role |

Pipeline / Drain Details

| Pipeline / Drain Reference: | Enter pipeline or drain run reference |
| --- | --- |
| Section (From — To): | Enter manhole or chainage references |
| Drawing Reference: | Enter installation or cross-section drawing reference |
| Pipe Material: | e.g. vitrified clay, uPVC, concrete, DI, PE, GRP |
| Pipe Diameter (mm): | Enter nominal diameter |
| Pipe Class / Strength Class: | e.g. DN/ID 150 Class 120, SN8, SN16 |
| Bedding Class: | e.g. Class S, Class B, Class N, Class F per BS EN 1610 |
| Surround Type: | e.g. granular, concrete, pea gravel, as-dug (if approved) |

Bedding and Surround Details

| Bedding Material: | e.g. 10mm single-size gravel, pea gravel, granular Type 1, sand, concrete |
| --- | --- |
| Bedding Thickness Below Invert (mm): | Enter design bedding depth below pipe invert (typically 100-150mm) |
| Actual Bedding Thickness (mm): | Enter measured thickness |
| Bedding Width: | Enter width — typically trench width or as specified |
| Surround Material: | e.g. same as bedding, concrete, selected fill |
| Surround Depth Above Crown (mm): | Enter design surround depth above pipe crown (typically 150-300mm) |
| Actual Surround Depth (mm): | Enter measured depth |
| Concrete Surround Grade (if applicable): | e.g. C20/25, GEN 1 |
| Selected Backfill Material: | Enter material type for initial backfill above surround zone |
| Compaction Method: | Enter compaction equipment used |
| Compaction Layer Thickness (mm): | Enter maximum layer thickness (typically 150mm or 300mm per spec) |

Installation Inspection Checklist

| No. | Inspection Item | Y / N / NA | Comments |
| --- | --- | --- | --- |
| 1 | Trench formation inspected and approved — free from hard spots, soft spots, and standing water |  |  |
| 2 | Trench width adequate for pipe diameter and bedding class (min width per BS EN 1610) |  |  |
| 3 | Trench sides stable and adequately supported where necessary (trench support in place) |  |  |
| 4 | Bedding material approved and matches specification |  |  |
| 5 | Bedding placed evenly across trench width and at correct depth below pipe invert |  |  |
| 6 | Bedding compacted to specification (hand tamping for granular, vibrated for concrete) |  |  |
| 7 | Pipe laid on bedding with uniform support along full length — no point loading |  |  |
| 8 | Pipe joints made correctly and not stressed by uneven bedding |  |  |
| 9 | Haunch support material placed and compacted evenly on both sides of pipe simultaneously |  |  |
| 10 | Surround material placed symmetrically around pipe to avoid displacement |  |  |
| 11 | Surround material hand-compacted in layers — no mechanical compaction directly above pipe crown |  |  |
| 12 | Correct depth of surround achieved above pipe crown before mechanical compaction begins |  |  |
| 13 | No large stones, sharp objects, or frozen material in contact with pipe |  |  |
| 14 | Warning / marker tape placed at correct height above pipe crown (typically 300mm) |  |  |
| 15 | Initial backfill placed in layers and compacted to specification above surround zone |  |  |
| 16 | No construction traffic permitted over pipe until minimum cover depth achieved |  |  |
| 17 | Air test or water test of completed pipe run carried out and passed per BS EN 1610 |  |  |
| 18 | Photographic record taken at bedding, surround, and initial backfill stages |  |  |
| 19 | All inspection points recorded on as-built drawings |  |  |

Inspection Result

| Overall Result: | Pass / Fail / Conditional |
| --- | --- |
| Permission to Continue Backfill: | Yes / No |
| Remedial Actions: | Detail any remedial works needed |

Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Drainage Gang Foreman |  |  | DD / MM / YYYY |
| Site Engineer (Main Contractor) |  |  | DD / MM / YYYY |
| Adopting Authority Inspector (if applicable) |  |  | DD / MM / YYYY |
| Client Representative |  |  | DD / MM / YYYY |

Notes / Additional Comments

Enter any additional notes, compaction test references, or photographic references here.

Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| A |  | Initial issue |  |  |