| FORMWORK AND FALSEWORK INSPECTION RECORD |  |
| --- | --- |
| Document Ref: | Enter document reference |
| Revision: | Enter revision number |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| Temporary Works Ref: | Enter TW register reference number |
| Prepared By: | Name, position and company |
| Approved By: | Name, position and company |

This record documents the inspection of formwork and falsework before a concrete pour. All falsework must be designed, erected and inspected in accordance with BS 5975 (Code of Practice for Temporary Works). A Permit to Load must be issued by the Temporary Works Coordinator before any concrete is placed. This inspection is a Hold Point.

1. Element and Design Details

| Element / Pour Reference: | e.g. Suspended slab, wall, beam soffit, column, bridge deck |
| --- | --- |
| Location / Grid Reference: |  |
| Structural Drawing Ref: |  |
| Formwork Design Ref: | Enter formwork / falsework design drawing and calculation ref |
| Design Check Certificate Ref: | Enter TWC design check certificate number (BS 5975) |
| Formwork System: | Traditional timber / System formwork (PERI, Doka, ULMA) / Steel / GRP / Other |
| Falsework System: | Props / Towers / Birdcage / None (ground slab) |
| Max Concrete Pour Height (m): |  |
| Max Concrete Pressure (kN/m²): |  |
| Pour Rate (m/hr): |  |
| Striking Time (minimum): | Enter per specification (e.g. 3 days soffits with backprops, 12 hrs walls) |
| TWC (Temp Works Coordinator): | Name |
| TW Designer: | Name and company |

2. Formwork Inspection

| No. | Formwork Inspection | OK | N/A | Comments / Initials |
| --- | --- | --- | --- | --- |
| 1 | Formwork erected in accordance with formwork design drawings |  |  |  |
| 2 | Formwork face clean, free from debris and treated with release agent |  |  |  |
| 3 | Formwork panels in good condition (no excessive wear, damage or distortion) |  |  |  |
| 4 | Formwork joints tight (no gaps that would allow grout loss) |  |  |  |
| 5 | Formwork plumb / level / line checked and within tolerance |  |  |  |
| 6 | Correct camber applied to soffits (if specified in design) |  |  |  |
| 7 | Kicker / construction joint preparation correct (straight, clean, roughened) |  |  |  |
| 8 | Tie bolts correct type, spacing and torque per formwork design |  |  |  |
| 9 | Walers and soldiers correctly positioned and secured |  |  |  |
| 10 | Formwork stop ends installed at correct positions |  |  |  |
| 11 | Openings, box-outs and cast-in formwork positioned per drawing |  |  |  |
| 12 | Chamfer strips and drip details fitted where specified |  |  |  |
| 13 | Formwork clean-out openings provided (for deep pours) |  |  |  |

3. Falsework Inspection

| No. | Falsework Inspection | OK | N/A | Comments / Initials |
| --- | --- | --- | --- | --- |
| 14 | Falsework erected on firm, level bearing surface (ground conditions adequate) |  |  |  |
| 15 | Sole plates and base plates correctly positioned and sized |  |  |  |
| 16 | Props / towers vertical and within tolerance |  |  |  |
| 17 | Props at correct spacing per falsework design |  |  |  |
| 18 | All props pinned / locked at correct height |  |  |  |
| 19 | All bracing installed per design (plan and elevation bracing) |  |  |  |
| 20 | Ledgers and transoms correctly positioned and secured |  |  |  |
| 21 | Bearer beams and joists correct size and spacing per design |  |  |  |
| 22 | No overloading of any single prop or tower (load distribution checked) |  |  |  |
| 23 | Falsework stable under expected wind loads (ties and bracing adequate) |  |  |  |
| 24 | Backpropping installed to lower floors (if striking formwork above) |  |  |  |
| 25 | Safe access and egress to all parts of falsework |  |  |  |
| 26 | No unauthorised modifications to falsework system |  |  |  |
| 27 | Permit to load obtained from TWC before concrete placement |  |  |  |

4. Safety and Access

| No. | Safety and Access | OK | N/A | Comments / Initials |
| --- | --- | --- | --- | --- |
| 28 | Working platforms at perimeter provided with edge protection |  |  |  |
| 29 | Safe access to work at height provided (ladders, stairs, scaffold) |  |  |  |
| 30 | Materials cannot fall through formwork gaps to levels below |  |  |  |
| 31 | Exclusion zone below falsework established and signed |  |  |  |
| 32 | Emergency procedure for falsework collapse communicated |  |  |  |
| 33 | Concrete pour plan agreed (pour sequence, rate, maximum height per lift) |  |  |  |
| 34 | Monitoring arrangements in place during pour (visual, survey, instrumentation) |  |  |  |

5. Permit to Load

| Permit to Load Issued: | Yes / No |
| --- | --- |
| Permit to Load Date: | DD / MM / YYYY |
| Issued By (TWC): | Name and signature |
| Maximum Pour Rate Permitted: | Enter m/hr |
| Maximum Pour Height Per Lift: | Enter m |
| Monitoring Required During Pour: | Yes / No (describe method) |

6. Inspection Result

| Inspection Result: | APPROVED / NOT APPROVED (remedial work required) |
| --- | --- |
| Defects Found: | Yes / No (describe) |
| Re-Inspection Required: | Yes / No |

7. Sign-Off

This is a HOLD POINT. Concrete must not be placed until the TWC has issued the Permit to Load and all parties have signed below.

| Role | Print Name | Signature | Date |
| --- | --- | --- | --- |
| Formwork / Falsework Supervisor |  |  |  |
| Site Engineer |  |  |  |
| Temporary Works Coordinator |  |  |  |
| Quality Manager |  |  |  |
| Client / Engineer's Representative |  |  |  |

8. Notes / Additional Comments

Enter any additional notes

9. Document Control

| Rev | Date | Description | Author | Approved |
| --- | --- | --- | --- | --- |

References: BS 5975 (Temporary Works), NSCS, CIRIA C580 (Formwork Guide), BS EN 12812 (Falsework), CDM 2015, Temporary Works Register, ITP