Curtain Wall Inspection Checklist

Building Envelope — Curtain Walling Installation Inspection per BS EN 13830 and CWCT Standards

| Inspection Details |  |  |  |
| --- | --- | --- | --- |
| Document Ref: |  | Inspection Date: | DD/MM/YYYY |
| Project Name: |  | Contract No: |  |
| Location / Elevation: | e.g., north elevation, levels 1–5 |  |  |
| Curtain Wall System: | e.g., stick system, unitised, structural glazing |  |  |
| Manufacturer / System: |  | System Ref: |  |
| Facade Contractor: |  | Main Contractor: |  |
| Inspector: |  | Drawing Ref: |  |
| Weather: |  | Temperature (°C): |  |

Section A: Brackets, Fixings, and Supporting Structure

| A: Brackets and Structure |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Primary structure (concrete / steel) checked for position, level, and plumb within tolerance |  |  |  |  |
| 2 | Cast-in channels or fixing positions verified against bracket layout drawing |  |  |  |  |
| 3 | Bracket type and size correct per structural design (thermal break brackets where specified) |  |  |  |  |
| 4 | Brackets installed at correct centres and heights per layout drawing |  |  |  |  |
| 5 | Bracket fixings correct type, grade, and quantity (e.g., M12 Grade 8.8 through bolts) |  |  |  |  |
| 6 | Bracket torque checked and recorded (calibrated torque wrench used) |  |  |  |  |
| 7 | Bracket alignment checked — plumb, level, and within specified tolerance |  |  |  |  |
| 8 | Shims and packing correctly installed at brackets (stainless steel or approved material) |  |  |  |  |
| 9 | Thermal break between bracket and primary structure provided where required |  |  |  |  |
| 10 | Dead load and live load transfer path verified per structural design |  |  |  |  |

Section B: Mullions and Transoms

| B: Framing Members |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Mullion sections correct type, profile, and finish (anodised, PPC — colour checked against sample) |  |  |  |  |
| 2 | Mullions installed plumb and at correct centres (within ±2mm or as specified) |  |  |  |  |
| 3 | Mullion splice joints correctly assembled with spigots / sleeves |  |  |  |  |
| 4 | Mullion expansion joints provided at specified intervals for thermal movement |  |  |  |  |
| 5 | Transoms installed level and at correct heights |  |  |  |  |
| 6 | Transom-to-mullion connections secure and correctly assembled |  |  |  |  |
| 7 | Drainage slots / weep holes provided in transoms as per system design |  |  |  |  |
| 8 | Screw / rivet fixings correct type and at correct centres |  |  |  |  |
| 9 | No visible damage, scratches, or dents on exposed aluminium profiles |  |  |  |  |
| 10 | Protective film intact on profiles (to be removed only at handover) |  |  |  |  |

Section C: Glazing and Infill Panels

| C: Glazing Installation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Glass units correct type, thickness, and configuration (e.g., DGU 6/16/6, toughened, Low-E) |  |  |  |  |
| 2 | Glass units comply with specification (BS EN 12150, BS EN 14449, BS EN 1279 as applicable) |  |  |  |  |
| 3 | Setting blocks positioned correctly (quarter points or per system manual) |  |  |  |  |
| 4 | Edge clearance maintained between glass and frame (minimum 5mm or as specified) |  |  |  |  |
| 5 | Glazing gaskets correct profile, material (EPDM), and fully seated in channels |  |  |  |  |
| 6 | Pressure plates / caps installed and screwed at correct centres |  |  |  |  |
| 7 | Spandrel panels and opaque infill panels correct specification and correctly installed |  |  |  |  |
| 8 | Insulated panels meet thermal performance requirements (U-value as designed) |  |  |  |  |
| 9 | No chips, cracks, scratches, or delamination visible in glass or panel units |  |  |  |  |
| 10 | Glass markings visible for identification (toughened, laminated kite mark) |  |  |  |  |

Section D: Weatherproofing and Sealing

| D: Weatherproofing |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Perimeter sealant applied between curtain wall frame and primary structure |  |  |  |  |
| 2 | Sealant type correct per specification (silicone, polyurethane — UV stable, compatible) |  |  |  |  |
| 3 | Sealant joints correct width and depth (backing rod installed where required) |  |  |  |  |
| 4 | Sealant tooled to concave profile with clean edges (no smearing or gaps) |  |  |  |  |
| 5 | Internal vapour barrier / air seal continuous and properly lapped |  |  |  |  |
| 6 | Fire barrier / cavity barrier installed at each floor slab edge (intumescent where specified) |  |  |  |  |
| 7 | Fire barrier installation matches approved detail and fire test evidence |  |  |  |  |
| 8 | Drainage channels clear and functional — water test completed |  |  |  |  |
| 9 | No visible gaps, openings, or discontinuities in the weatherproofing envelope |  |  |  |  |
| 10 | Flashing and cover trims correctly installed at head, cill, and jamb conditions |  |  |  |  |

Section E: Thermal and Acoustic Performance

| E: Performance Requirements |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Thermal break profiles used throughout (no cold bridging at mullions / transoms) |  |  |  |  |
| 2 | Insulation correctly installed behind spandrel panels and at perimeter conditions |  |  |  |  |
| 3 | U-value calculation verified against design specification and Part L requirements |  |  |  |  |
| 4 | Acoustic performance of glazing units meets specification (Rw dB rating) |  |  |  |  |
| 5 | Acoustic seals provided at opening vents and interfaces |  |  |  |  |
| 6 | Condensation risk assessment reviewed — no risk of interstitial condensation |  |  |  |  |

Section F: Opening Vents and Louvres (if applicable)

| F: Opening Vents |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Opening vents correct type and size per specification (top-hung, side-hung, parallel opening) |  |  |  |  |
| 2 | Vent hardware (hinges, restrictors, actuators) correctly installed and operational |  |  |  |  |
| 3 | Safety restrictors fitted to limit opening where required (falls prevention) |  |  |  |  |
| 4 | Weatherstripping and gaskets correctly installed on opening vents |  |  |  |  |
| 5 | Vents operate smoothly without obstruction or binding |  |  |  |  |
| 6 | Louvres installed with correct blade angle and insect mesh (if specified) |  |  |  |  |
| 7 | Motorised vents / actuators tested and connected to BMS (if applicable) |  |  |  |  |

Section G: General Quality and Compliance

| G: General |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Overall alignment of curtain wall facade checked (plumb, level, flatness) |  |  |  |  |
| 2 | Colour consistency of frames, panels, and glass across the elevation |  |  |  |  |
| 3 | All fixings stainless steel or corrosion-resistant as specified |  |  |  |  |
| 4 | Bi-metallic corrosion prevention measures in place (isolation gaskets / coatings) |  |  |  |  |
| 5 | Lightning protection / earthing bonding connected to curtain wall frame (if required) |  |  |  |  |
| 6 | Cleaning and maintenance access strategy confirmed (BMU, cradle, abseiling — compatible with system) |  |  |  |  |
| 7 | Hose test or site water spray test completed per CWCT TN 41 or BS EN 13051 |  |  |  |  |
| 8 | All test results recorded and filed |  |  |  |  |
| 9 | Manufacturer's installation manual available on site and followed |  |  |  |  |

Non-Conformances / Defects

| No. | Section / Item | Description | Corrective Action | Closed Out |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |

Inspection Sign-Off

By signing below, we confirm the curtain wall installation has been inspected against the approved drawings, system manufacturer's requirements, BS EN 13830, and CWCT standards.

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Facade Inspector |  |  | DD/MM/YYYY |
| Facade Contractor Supervisor |  |  | DD/MM/YYYY |
| Main Contractor PM |  |  | DD/MM/YYYY |
| Client / Architect Representative |  |  | DD/MM/YYYY |

Notes / Additional Comments

Record additional observations, hose test results, photographs, or follow-up items.

Document Control

| Rev | Date | Description | Author | Approved |
| --- | --- | --- | --- | --- |
| 0 |  | Initial issue |  |  |
| 1 |  |  |  |  |