First Fix Inspection Checklist

Carpentry and Joinery — First Fix Quality Inspection prior to Boarding and Closing Up

| Inspection Details |  |  |  |
| --- | --- | --- | --- |
| Document Ref: |  | Inspection Date: | DD/MM/YYYY |
| Project Name: |  | Contract No: |  |
| Location / Plot No: | e.g., Plot 12, Block B, Unit 5 |  |  |
| Level / Storey: | e.g., ground floor, first floor | Drawing Ref: |  |
| Timber Specification: | e.g., C16, C24 graded softwood | Treatment: | e.g., tanalised, untreated |
| Contractor / Subcontractor: |  | Inspector: |  |
| Inspection Stage: | Pre-plasterboard / pre-close-up | Weather: |  |

Section A: Floor Structure

| A: Floor Joists and Structure |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Floor joists correct size, grade, and centres as per structural design |  |  |  |  |
| 2 | Joist hangers correct type, size, and properly fixed (BS EN 845-1) |  |  |  |  |
| 3 | Joists bearing correctly onto walls or steel beams (minimum bearing as specified) |  |  |  |  |
| 4 | Joists level and true (checked with spirit level / laser) |  |  |  |  |
| 5 | Noggings installed between joists at correct centres (maximum 1200mm or as designed) |  |  |  |  |
| 6 | Strutting provided at mid-span for joists over 2.5m span (herringbone or solid) |  |  |  |  |
| 7 | Trimming joists and trimmers correctly sized and fixed around openings (stairwells, hearths) |  |  |  |  |
| 8 | Openings in floor structure correctly framed and supported per structural drawings |  |  |  |  |
| 9 | All joists and structural timbers dry and free from damage, twist, or excessive bow |  |  |  |  |
| 10 | Engineered joists / I-beams installed per manufacturer's instructions (no site modifications) |  |  |  |  |
| 11 | Restraint straps fitted between floor structure and masonry walls (Building Regulations Part A) |  |  |  |  |

Section B: Stud Partitions

| B: Internal Stud Walls |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Stud partitions positioned as per floor plan layout drawings |  |  |  |  |
| 2 | Sole plates fixed to floor with correct fixings at maximum 600mm centres |  |  |  |  |
| 3 | Head plates fixed to ceiling joists or noggings above |  |  |  |  |
| 4 | Studs correct size (typically 75x50mm or 100x50mm) at correct centres (typically 400mm or 600mm) |  |  |  |  |
| 5 | Noggings installed at correct heights for fixtures (basins, radiators, kitchen units, TV brackets) |  |  |  |  |
| 6 | Additional noggings / blocking for heavy items clearly positioned per specification |  |  |  |  |
| 7 | Stud walls plumb, square, and correctly braced during construction |  |  |  |  |
| 8 | Door openings correctly framed with head, cripple studs, and adequate lintel support |  |  |  |  |
| 9 | Service zones provided or marked for plumber and electrician (notching / drilling zones observed) |  |  |  |  |
| 10 | Fire-rated partitions constructed to correct specification (double stud, mineral wool, etc.) |  |  |  |  |

Section C: Roof Structure (if applicable)

| C: Roof Timbers and Trusses |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Roof trusses / rafters delivered undamaged and stored correctly on site |  |  |  |  |
| 2 | Trusses installed at correct centres (typically 600mm) and to correct falls |  |  |  |  |
| 3 | Truss bracing installed per engineer's bracing layout (longitudinal, diagonal, chevron) |  |  |  |  |
| 4 | Truss clips / fixings correct type and properly nailed / screwed (minimum nail pattern observed) |  |  |  |  |
| 5 | Ridge board / ridge beam correctly installed and level |  |  |  |  |
| 6 | Hip and valley rafters correctly cut and fixed |  |  |  |  |
| 7 | Purlins and binders correctly positioned and fixed |  |  |  |  |
| 8 | Tank platforms and water tank supports correctly installed (if applicable) |  |  |  |  |
| 9 | Truss modifications prohibited without structural engineer's approval |  |  |  |  |
| 10 | Restraint straps installed at gable walls and ceiling level per Building Regulations Part A |  |  |  |  |

Section D: Staircase Installation

| D: Stairs |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Staircase correct type and size as per drawing (straight flight, winder, half-landing) |  |  |  |  |
| 2 | Going and rise dimensions comply with Building Regulations Part K (minimum 220mm going, maximum 220mm rise for private stairs) |  |  |  |  |
| 3 | Staircase securely fixed to trimmer at top and kicker plate at bottom |  |  |  |  |
| 4 | String boards correctly notched or routed for treads and risers |  |  |  |  |
| 5 | Newel posts securely fixed (bolted through floor joists or trimmer) |  |  |  |  |
| 6 | Temporary handrail / guarding installed for safety during construction |  |  |  |  |
| 7 | Headroom above staircase compliant (minimum 2.0m measured vertically per Part K) |  |  |  |  |
| 8 | Staircase protected from damage during construction phase (covered / boarded) |  |  |  |  |

Section E: Window and Door Frames

| E: Frames and Openings |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | External door frames plumb, level, and correctly positioned in opening |  |  |  |  |
| 2 | Door frames fixed with appropriate fixings at maximum 600mm centres |  |  |  |  |
| 3 | Window frames / sub-frames installed plumb, level, and square |  |  |  |  |
| 4 | DPC / cavity closer / DPM correctly dressed around window and door openings |  |  |  |  |
| 5 | Lintels above openings verified (structural check — correct size and bearing) |  |  |  |  |
| 6 | Fire door frames correct specification for fire rating (FD30, FD60 etc.) |  |  |  |  |
| 7 | Frames not twisted, damaged, or distorted during fixing |  |  |  |  |
| 8 | Adequate packing used (no over-packing causing distortion) |  |  |  |  |

Section F: Services Coordination (prior to close-up)

| F: Services First Fix |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | All first fix plumbing installed and pressure tested (water, heating, waste) |  |  |  |  |
| 2 | All first fix electrical cables installed and tested (circuits, back boxes, consumer unit) |  |  |  |  |
| 3 | Notching and drilling of joists within permitted zones (BS 8103-3 / Building Regulations) |  |  |  |  |
| 4 | No notches or holes in structural members outside safe zones without engineer approval |  |  |  |  |
| 5 | Mechanical ventilation ductwork installed (MVHR, extract fans) and sealed |  |  |  |  |
| 6 | Fire stopping and intumescent seals installed at all service penetrations through fire-rated elements |  |  |  |  |
| 7 | Insulation installed in external walls (partial fill / full fill as specified) |  |  |  |  |
| 8 | Vapour control layer (VCL) installed correctly on warm side of insulation (if required) |  |  |  |  |
| 9 | All services labelled and recorded for as-built documentation |  |  |  |  |
| 10 | Gas pipes installed, tested, and capped ready for second fix |  |  |  |  |

Section G: General Quality and Building Regulations

| G: General Compliance |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Timber preservative treatment correct for location (DPC level, external, wet areas) |  |  |  |  |
| 2 | All structural timber graded and marked (C16, C24 etc. with certification mark) |  |  |  |  |
| 3 | No excessive moisture in timbers (checked with moisture meter — below 20%) |  |  |  |  |
| 4 | Joist notching and drilling compliant with approved safe zones |  |  |  |  |
| 5 | Building control pre-plaster inspection booked / completed |  |  |  |  |
| 6 | NHBC / warranty provider inspection booked / completed (if applicable) |  |  |  |  |
| 7 | Photographs taken of all hidden work prior to boarding (services, insulation, fire stopping) |  |  |  |  |
| 8 | All snags from previous inspections closed out before proceeding to boarding |  |  |  |  |

Non-Conformances / Defects

| No. | Section / Item | Description | Corrective Action | Closed Out |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |

Inspection Sign-Off

By signing below, we confirm that the first fix carpentry and associated works have been inspected prior to close-up and boarding. All items must be satisfactorily completed before plasterboard installation commences.

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Site Inspector / Clerk of Works |  |  | DD/MM/YYYY |
| Carpentry Foreman |  |  | DD/MM/YYYY |
| Site Manager |  |  | DD/MM/YYYY |
| Building Control Officer |  |  | DD/MM/YYYY |

Overall First Fix Status

| First Fix Status | PASS / FAIL / REMEDIAL WORKS REQUIRED |
| --- | --- |

Notes / Additional Comments

Record additional observations, photograph references, or items requiring follow-up.

Document Control

| Rev | Date | Description | Author | Approved |
| --- | --- | --- | --- | --- |
| 0 |  | Initial issue |  |  |
| 1 |  |  |  |  |