Brickwork Inspection Checklist

Masonry Construction — Quality Inspection per BS EN 1996 / BS 5628 and Project Specification

| Inspection Details |  |  |  |
| --- | --- | --- | --- |
| Document Ref: |  | Inspection Date: | DD/MM/YYYY |
| Project Name: |  | Contract No: |  |
| Location / Block: | e.g., Block A, external elevation north |  |  |
| Level / Storey: | e.g., ground floor to first floor | Drawing Ref: |  |
| Brick Type / Manufacturer: |  | Brick Standard: | e.g., BS EN 771-1 |
| Mortar Mix / Designation: | e.g., M4, 1:1:6 (OPC:lime:sand) | Mortar Standard: | e.g., BS EN 998-2 |
| Contractor: |  | Inspector: |  |
| Weather Conditions: |  | Temperature (°C): |  |

Section A: Materials and Storage

| A: Materials and Storage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Bricks delivered to site match the specification (type, colour, texture, manufacturer) |  |  |  |  |
| 2 | Brick delivery notes checked and filed (including batch references) |  |  |  |  |
| 3 | Bricks stored on level, clean, hard-standing away from site contamination |  |  |  |  |
| 4 | Bricks adequately protected from weather (covered to prevent saturation) |  |  |  |  |
| 5 | Mortar materials (cement, lime, sand, admixtures) stored correctly and within shelf life |  |  |  |  |
| 6 | Sand clean, well-graded, and free from deleterious materials |  |  |  |  |
| 7 | Pre-mixed / silo mortar delivery notes checked (mix designation confirmed) |  |  |  |  |
| 8 | Mortar colour consistent and approved against sample panel |  |  |  |  |
| 9 | Wall ties correct type, material, and length for cavity width (BS EN 845-1) |  |  |  |  |
| 10 | Insulation type and thickness as specified (full-fill / partial-fill cavity insulation) |  |  |  |  |

Section B: Setting Out and DPC

| B: Setting Out and DPC |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Setting out checked against approved drawings (plan dimensions and openings) |  |  |  |  |
| 2 | Brick gauge rod used and checked (correct course heights for brick + joint) |  |  |  |  |
| 3 | DPC correctly positioned at minimum 150mm above finished ground level |  |  |  |  |
| 4 | DPC material to specification (BS 6515 / BS EN 14967) and full width of wall |  |  |  |  |
| 5 | DPC lapped correctly at joints (minimum 100mm) with sealed or welted joints |  |  |  |  |
| 6 | DPC extends through full thickness of leaf and is not bridged by mortar droppings |  |  |  |  |
| 7 | Ground floor slab / beam level correct relative to DPC level |  |  |  |  |
| 8 | Cavity tray DPCs correctly installed at all abutments, lintels, and steps in cavity |  |  |  |  |

Section C: Construction Quality

| C: General Brickwork Construction |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Bond pattern correct and consistent (stretcher bond, Flemish bond, etc. as specified) |  |  |  |  |
| 2 | Perpend joints aligned vertically (or correctly staggered per bond type) |  |  |  |  |
| 3 | Bed joints consistent thickness (nominally 10mm, within tolerance) |  |  |  |  |
| 4 | Perpend joints consistent thickness and fully filled |  |  |  |  |
| 5 | Brickwork plumb (checked with spirit level — within tolerance of 10mm in 3m height) |  |  |  |  |
| 6 | Brickwork level (checked with spirit level — courses level within tolerance) |  |  |  |  |
| 7 | Brickwork line (face of wall straight within tolerance — checked with string line) |  |  |  |  |
| 8 | No frog-down bricks (frogs laid upward and filled with mortar unless specified otherwise) |  |  |  |  |
| 9 | Cut bricks neatly cut and positioned correctly (no small pieces less than half-brick at reveals) |  |  |  |  |
| 10 | Brickwork sample panel approved and referenced during construction |  |  |  |  |
| 11 | Mortar joints finished to specified profile (flush, bucket handle, weatherstruck, recessed) |  |  |  |  |
| 12 | Jointing / pointing consistent and completed before mortar sets |  |  |  |  |
| 13 | No mortar smears, staining, or efflorescence on face brickwork |  |  |  |  |
| 14 | Brickwork adequately protected during construction (covered overnight and in rain) |  |  |  |  |

Section D: Cavity Construction

| D: Cavity Requirements |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Cavity width correct as specified (minimum 50mm residual clear cavity for partial-fill) |  |  |  |  |
| 2 | Cavity clean and free from mortar droppings (cavity battens used during construction) |  |  |  |  |
| 3 | Wall ties installed at correct centres (maximum 900mm horizontal, 450mm vertical, or as designed) |  |  |  |  |
| 4 | Additional wall ties at openings, movement joints, and edges (300mm from reveals) |  |  |  |  |
| 5 | Wall ties correct type and length for cavity width |  |  |  |  |
| 6 | Wall ties bedded minimum 50mm into each leaf with slight fall towards outer leaf |  |  |  |  |
| 7 | Cavity insulation correctly installed and retained (no gaps, sagging, or compression) |  |  |  |  |
| 8 | Insulation retaining clips / discs fitted where required |  |  |  |  |
| 9 | Cavity barriers and cavity closers installed at all required locations (fire stopping) |  |  |  |  |
| 10 | Weep holes provided above DPC trays and lintels (every 3rd perpend joint or as specified) |  |  |  |  |

Section E: Openings, Lintels, and Movement Joints

| E: Openings and Structural Elements |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Lintels correct type, size, and bearing length (minimum 150mm per BS EN 845-2) |  |  |  |  |
| 2 | Lintel level, plumb, and correctly bedded on mortar |  |  |  |  |
| 3 | Lintel DPC / cavity tray correctly installed with stop ends and weep holes |  |  |  |  |
| 4 | Window and door reveals built plumb and to correct dimensions |  |  |  |  |
| 5 | Reveals built with correct bond and ties (frame cramps at 300mm centres) |  |  |  |  |
| 6 | Cills correctly installed with drip detail and end dams |  |  |  |  |
| 7 | Movement joints positioned as per structural engineer's design and specification |  |  |  |  |
| 8 | Movement joints filled with compressible filler and sealed with appropriate sealant |  |  |  |  |
| 9 | Movement joint ties (sliding ties) correctly installed across movement joints |  |  |  |  |
| 10 | Soldier courses / feature brickwork correctly constructed per approved details |  |  |  |  |

Section F: Below Ground and Substructure Brickwork

| F: Substructure |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Below-ground brickwork uses engineering bricks or specified frost-resistant bricks |  |  |  |  |
| 2 | Mortar designation correct for below-ground use (minimum M6 / Class ii) |  |  |  |  |
| 3 | Tanking / waterproofing membrane applied correctly where specified |  |  |  |  |
| 4 | Cavity filled with concrete or lean mix below ground level as specified |  |  |  |  |
| 5 | Weep holes provided above ground level to drain cavity |  |  |  |  |

Non-Conformances / Defects

| No. | Section / Item | Description of Defect | Corrective Action | Closed Out |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

Inspection Sign-Off

By signing below, the inspector confirms that the brickwork has been inspected against the project specification, approved sample panel, and this checklist.

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Site Inspector / Clerk of Works |  |  | DD/MM/YYYY |
| Bricklaying Foreman |  |  | DD/MM/YYYY |
| Site Manager |  |  | DD/MM/YYYY |
| Client Representative |  |  | DD/MM/YYYY |

Notes / Additional Comments

Record any additional observations, photographs, or sample panel references.

Document Control

| Rev | Date | Description | Author | Approved |
| --- | --- | --- | --- | --- |
| 0 |  | Initial issue |  |  |
| 1 |  |  |  |  |