Concrete Pour Record

Concrete Placement — Pour Record, Delivery Log, and Test Cube Schedule per BS EN 206 / BS 8500

| Pour Details |  |  |  |
| --- | --- | --- | --- |
| Document Ref: |  | Pour Date: | DD/MM/YYYY |
| Project Name: |  | Contract No: |  |
| Pour Reference: | e.g., Pour P-025 | Element: | e.g., base slab, wall, column, beam |
| Location / Gridlines: | e.g., Block A, gridlines 1-5 / A-D |  |  |
| Structural Drawing Ref: |  | Drawing Revision: |  |
| Contractor: |  | Client: |  |
| Site Engineer: |  | Concrete Foreman: |  |

1. Concrete Specification

| Concrete Grade: | e.g., C32/40, C40/50 | Specification Standard: | BS 8500-1 / BS EN 206 |
| --- | --- | --- | --- |
| Designated / Designed Mix: | e.g., RC32/40 XC3/4, GEN 1 | Exposure Class: | e.g., XC3/4, XD2, XS1, XF1 |
| Maximum w/c Ratio: | e.g., 0.50 | Minimum Cement Content (kg/m3): | e.g., 340 |
| Cement Type: | e.g., CEM I, CEM II/B-V, CEM III/A | Max Aggregate Size (mm): | e.g., 20 |
| Consistence Class: | e.g., S3 (100-150mm slump), SF2 (SCC) | Admixtures: | e.g., retarder, superplasticiser, air entrainer |
| Concrete Supplier: |  | Batching Plant: |  |
| Planned Volume (m3): |  | Actual Volume (m3): |  |

2. Concrete Delivery Log

Record each delivery ticket. Check slump / consistence on arrival. Note any rejected loads.

| Load No. | Ticket No. | Arrive Time | Vol. (m3) | Batch Time | Slump (mm) | Temp (°C) | Water Added | Accept / Reject | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  | Y/N litres |  |  |
| 2 |  |  |  |  |  |  | Y/N litres |  |  |
| 3 |  |  |  |  |  |  | Y/N litres |  |  |
| 4 |  |  |  |  |  |  | Y/N litres |  |  |
| 5 |  |  |  |  |  |  | Y/N litres |  |  |
| 6 |  |  |  |  |  |  | Y/N litres |  |  |
| 7 |  |  |  |  |  |  | Y/N litres |  |  |
| 8 |  |  |  |  |  |  | Y/N litres |  |  |
| 9 |  |  |  |  |  |  | Y/N litres |  |  |
| 10 |  |  |  |  |  |  | Y/N litres |  |  |
| 11 |  |  |  |  |  |  | Y/N litres |  |  |
| 12 |  |  |  |  |  |  | Y/N litres |  |  |
| 13 |  |  |  |  |  |  | Y/N litres |  |  |
| 14 |  |  |  |  |  |  | Y/N litres |  |  |
| 15 |  |  |  |  |  |  | Y/N litres |  |  |

3. Test Cube / Cylinder Schedule

Record all test specimens taken during the pour. Minimum 1 set per 50m3 or per day's production (or as specified). Each set typically comprises 3 cubes (7-day and 28-day testing).

| Set No. | Sample Ref | Load / Ticket | Time Taken | Slump at Sampling (mm) | Cube Size (mm) | 7-Day Test (Y/N) | 28-Day Test (Y/N) | Lab / Testing Co. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | HH:MM |  | 150 / 100 |  |  |  |
| 2 |  |  | HH:MM |  | 150 / 100 |  |  |  |
| 3 |  |  | HH:MM |  | 150 / 100 |  |  |  |
| 4 |  |  | HH:MM |  | 150 / 100 |  |  |  |
| 5 |  |  | HH:MM |  | 150 / 100 |  |  |  |
| 6 |  |  | HH:MM |  | 150 / 100 |  |  |  |

4. Pre-Pour Checks

| Pre-Pour Verification |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. | Item | Y | N | Comments |
| 1 | Formwork inspection completed and signed off (separate formwork checklist) |  |  |  |
| 2 | Reinforcement inspection completed and signed off (separate rebar checklist) |  |  |  |
| 3 | Formwork clean, oiled, and free from debris and standing water |  |  |  |
| 4 | Construction joint preparation complete (roughened, clean, dampened) |  |  |  |
| 5 | All cast-in items, box-outs, and waterstops checked and secure |  |  |  |
| 6 | Concrete specification confirmed with supplier (grade, consistence, admixtures) |  |  |  |
| 7 | Placement method confirmed (pump, skip, chute) and equipment in position |  |  |  |
| 8 | Vibrators available, tested, and adequate for the pour (minimum 1 spare) |  |  |  |
| 9 | Curing materials available on site (curing compound, polythene, hessian) |  |  |  |
| 10 | Weather forecast reviewed — precautions in place for hot / cold / wet weather |  |  |  |
| 11 | Pour sequence and method statement briefed to all operatives |  |  |  |
| 12 | Emergency concrete disposal area identified (for rejected loads) |  |  |  |

5. During Pour Observations

| Pour Monitoring |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. | Item | Y | N | Comments |
| 1 | Concrete placed within 2 hours of batching (or within the specified time per admixture) |  |  |  |
| 2 | No additional water added on site without supplier approval (documented if approved) |  |  |  |
| 3 | Concrete placed in layers not exceeding 500mm (or as specified) for adequate compaction |  |  |  |
| 4 | Vibration applied systematically — no over-vibration or under-vibration |  |  |  |
| 5 | No visible segregation, bleeding, or honeycombing during placement |  |  |  |
| 6 | Pour rate within the formwork design capacity |  |  |  |
| 7 | Formwork monitored for movement, leakage, or deflection during pour |  |  |  |
| 8 | Construction joints formed at approved locations only |  |  |  |
| 9 | Top surface finished to specified level and tolerance (screeded, floated, tamped as required) |  |  |  |
| 10 | Concrete temperature monitored (min. 5°C at placement for normal concrete) |  |  |  |

6. Post-Pour and Curing

| Post-Pour Checks |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. | Item | Y | N | Comments |
| 1 | Curing commenced immediately after finishing (within specified time) |  |  |  |
| 2 | Curing method applied as specified (curing compound, polythene sheet, wet hessian, ponding) |  |  |  |
| 3 | Curing period minimum 3 days (7 days for water-retaining structures or as specified) |  |  |  |
| 4 | Formwork striking time confirmed by engineer (minimum strength achieved) |  |  |  |
| 5 | Props and re-props maintained as per striking sequence / backpropping schedule |  |  |  |
| 6 | Surface defects recorded (honeycombing, blowholes, cracks) — repaired per approved method |  |  |  |
| 7 | Top surface protected from foot traffic and loading during early curing |  |  |  |
| 8 | Cold weather precautions in place if ambient temperature below 5°C (insulation, heated enclosures) |  |  |  |

7. Pour Summary

| Pour Start Time: | HH:MM | Pour End Time: | HH:MM |
| --- | --- | --- | --- |
| Total Duration (hrs): |  | Total Loads Received: |  |
| Planned Volume (m3): |  | Actual Volume (m3): |  |
| Loads Rejected: | number and reason | Wastage (%) |  |
| Cube Sets Taken: |  | Ambient Temp Range (°C): | min–max |
| Weather During Pour: |  | Wind Conditions: |  |
| Curing Method: |  | Formwork Strike Date: | DD/MM/YYYY (minimum) |

8. Non-Conformances / Issues

| No. | Item | Description | Action Taken | Closed Out |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

9. Pour Sign-Off

By signing below, we confirm that the concrete pour was carried out in accordance with the approved specification, method statement, and this record. All delivery tickets have been collected and filed.

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Concrete Foreman |  |  | DD/MM/YYYY |
| Site Engineer |  |  | DD/MM/YYYY |
| Structural Engineer / Clerk of Works |  |  | DD/MM/YYYY |
| Client Representative |  |  | DD/MM/YYYY |

10. Notes / Additional Comments

Record additional observations, delivery ticket file reference, cube test results when available, or photographs.

11. Document Control

| Rev | Date | Description | Author | Approved |
| --- | --- | --- | --- | --- |
| 0 |  | Initial issue |  |  |
| 1 |  |  |  |  |