Formwork Inspection Checklist

Concrete and Formwork — Pre-Pour Formwork and Falsework Inspection per BS 5975 and Project Specification

| Inspection Details |  |  |  |
| --- | --- | --- | --- |
| Document Ref: |  | Inspection Date: | DD/MM/YYYY |
| Project Name: |  | Contract No: |  |
| Location / Pour Ref: | e.g., Base B1, Wall W3, Slab S2, Column C4 |  |  |
| Structural Drawing Ref: |  | Drawing Revision: |  |
| Formwork System: | e.g., traditional timber, proprietary panel, table form, slip form |  |  |
| Formwork Design Ref: |  | TW Coordinator: |  |
| TW Permit to Load No: |  | Permit Issued By: |  |
| Contractor: |  | Inspector: |  |
| Planned Pour Date: | DD/MM/YYYY | Concrete Grade: | e.g., C32/40, C40/50 |
| Pour Volume (m3): |  | Max Pour Rate (m/hr): | as per formwork design |

Section A: Foundations, Bearing, and Base Plates

| A: Foundations and Bearing |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Ground beneath falsework / props adequate to support imposed loads (bearing capacity verified) |  |  |  |  |
| 2 | Sole plates or base plates placed on firm, level bearing surface |  |  |  |  |
| 3 | Sole plates of adequate size to distribute load without exceeding ground bearing capacity |  |  |  |  |
| 4 | Base plates not placed on frozen, waterlogged, or recently backfilled ground without engineer approval |  |  |  |  |
| 5 | Wedges or screw jacks used for levelling — not packing with loose materials |  |  |  |  |
| 6 | Drainage provided to prevent water undermining falsework foundations |  |  |  |  |

Section B: Falsework and Propping

| B: Falsework and Props |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Falsework / propping system matches the approved design drawing |  |  |  |  |
| 2 | Props correct type, size, and rated capacity for the applied load |  |  |  |  |
| 3 | Props plumb (within 1:200 or as specified) and correctly braced |  |  |  |  |
| 4 | Prop extensions within permitted range (not over-extended beyond safe limit) |  |  |  |  |
| 5 | Adjustable forks / U-heads correctly fitted and pin locations correct |  |  |  |  |
| 6 | Horizontal and diagonal bracing installed as per falsework design |  |  |  |  |
| 7 | Ledger and transom connections secure (scaffold tube: correct couplers and tightened) |  |  |  |  |
| 8 | Proprietary system components undamaged, clean, and correctly assembled per manufacturer |  |  |  |  |
| 9 | No unauthorised modifications to the falsework system |  |  |  |  |
| 10 | Loads from formwork transferred concentrically through props (no eccentric loading) |  |  |  |  |
| 11 | Adjacent completed structure not overloaded by falsework / propping reactions |  |  |  |  |

Section C: Formwork Panels and Sheathing

| C: Formwork Panels |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Formwork panels correct size and type for the element being cast |  |  |  |  |
| 2 | Panels clean and free from debris, old concrete, and nails |  |  |  |  |
| 3 | Panel surfaces treated with approved release agent (mould oil applied evenly) |  |  |  |  |
| 4 | Joints between panels tight — no gaps that would allow grout loss |  |  |  |  |
| 5 | Kickers cast to correct line and level (for wall and column pours) |  |  |  |  |
| 6 | Formwork aligned to correct line, level, and dimensions (checked against drawing) |  |  |  |  |
| 7 | Internal dimensions of formwork match structural drawing tolerances |  |  |  |  |
| 8 | Chamfer strips and feature moulds correctly positioned (if specified) |  |  |  |  |
| 9 | Tie bolts / form ties installed at correct centres and tightened |  |  |  |  |
| 10 | Tie bolt cones or sleeves in place for water-retaining structures (if applicable) |  |  |  |  |
| 11 | Waler beams and soldiers correctly positioned and secured |  |  |  |  |
| 12 | No bowing, bulging, or distortion visible in formwork panels |  |  |  |  |

Section D: Cast-In Items and Openings

| D: Cast-In Items |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | All cast-in items positioned per the structural and M&E drawings |  |  |  |  |
| 2 | Holding-down bolts correctly positioned in template (checked for alignment and projection) |  |  |  |  |
| 3 | Cast-in channels, plates, and fixings secure and at correct level |  |  |  |  |
| 4 | Box-outs for openings, ducts, and penetrations correctly sized and positioned |  |  |  |  |
| 5 | Box-outs secured to prevent movement during concrete placement |  |  |  |  |
| 6 | Waterstops correctly positioned and supported at construction joints |  |  |  |  |
| 7 | Conduits, sleeves, and pipes through formwork sealed to prevent grout loss |  |  |  |  |
| 8 | Embedment items checked and approved by the structural engineer / designer |  |  |  |  |

Section E: Access and Safety

| E: Access and Safety |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Safe access provided to all working levels (ladders, stair towers, platforms) |  |  |  |  |
| 2 | Edge protection in place around open edges and voids (minimum 950mm guardrail + toe board) |  |  |  |  |
| 3 | Working platforms adequate width and load capacity for operatives and equipment |  |  |  |  |
| 4 | Area beneath formwork clear of personnel during loading / pouring (exclusion zone) |  |  |  |  |
| 5 | Temporary works permit to load issued and displayed at the pour location |  |  |  |  |
| 6 | Weather conditions checked — wind speed, temperature, and forecast reviewed |  |  |  |  |
| 7 | Concrete pump or skip placement route confirmed (no overloading of falsework) |  |  |  |  |
| 8 | Emergency procedures briefed to all operatives (formwork collapse, concrete spill) |  |  |  |  |
| 9 | Lighting adequate for inspection and pouring (if night works) |  |  |  |  |

Section F: Final Pre-Pour Verification

| F: Final Pre-Pour Checks |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | All reinforcement installed, inspected, and signed off (separate rebar inspection record) |  |  |  |  |
| 2 | Cover to reinforcement verified using cover meter or spacers checked |  |  |  |  |
| 3 | Formwork dimensions final-checked against structural drawing (key dimensions recorded) |  |  |  |  |
| 4 | Formwork stable, secure, and ready to receive concrete at the specified pour rate |  |  |  |  |
| 5 | All temporary bracing, ties, and connections fully tightened |  |  |  |  |
| 6 | Formwork surface clean — no standing water, ice, debris, or loose material |  |  |  |  |
| 7 | Construction joint preparation complete (surface roughened, clean, dampened if required) |  |  |  |  |
| 8 | Vibrator access points confirmed — formwork accommodates compaction method |  |  |  |  |
| 9 | Pour sequence and method statement briefed to concrete gang |  |  |  |  |
| 10 | Concrete supply confirmed — grade, volume, delivery schedule, and mix design approved |  |  |  |  |

Non-Conformances / Defects

| No. | Section / Item | Description | Corrective Action | Closed Out |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |

Overall Formwork Inspection Result

| Formwork Status | APPROVED FOR POUR / NOT APPROVED / REMEDIAL WORKS REQUIRED |
| --- | --- |

Inspection Sign-Off

By signing below, we confirm that the formwork and falsework have been inspected against the approved design, BS 5975, and this checklist. The formwork is / is not approved for concrete placement.

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Formwork Foreman |  |  | DD/MM/YYYY |
| TW Coordinator / Supervisor |  |  | DD/MM/YYYY |
| Site Engineer |  |  | DD/MM/YYYY |
| Structural Engineer / Client Rep |  |  | DD/MM/YYYY |

Notes / Additional Comments

Record any additional observations, key dimension checks, photographs, or conditions.

Document Control

| Rev | Date | Description | Author | Approved |
| --- | --- | --- | --- | --- |
| 0 |  | Initial issue |  |  |
| 1 |  |  |  |  |