Pre-Pour Reinforcement Inspection Checklist

Concrete Reinforcement — Pre-Pour Quality Inspection per BS 4449, BS 8666, and Eurocode 2 (BS EN 1992-1-1)

| Inspection Details |  |  |  |
| --- | --- | --- | --- |
| Document Ref: |  | Inspection Date: | DD/MM/YYYY |
| Project Name: |  | Contract No: |  |
| Pour Reference: | e.g., Pour P-012, Base B3, Wall W7 |  |  |
| Structural Element: | e.g., foundation, slab, wall, column, beam | Level / Location: | e.g., ground floor, gridline A-C / 1-4 |
| Reinforcement Drawing Ref: |  | Drawing Revision: |  |
| Bar Schedule Ref: | per BS 8666 | Schedule Revision: |  |
| Concrete Grade: | e.g., C32/40, C40/50 | Exposure Class: | e.g., XC3/4, XD2, XS1 |
| Specified Cover (mm): | per structural drawing / spec | Fixing Tolerance (mm): | per Eurocode 2 / spec |
| Contractor: |  | Inspector: |  |
| Planned Pour Date: | DD/MM/YYYY | Pour Volume (m3): |  |

Section A: Reinforcement Materials and Delivery

| A: Materials Verification |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Reinforcement delivered with mill certificates and test certificates (BS 4449 / BS EN 10080) |  |  |  |  |
| 2 | Bar marks checked against the bar bending schedule (BS 8666) |  |  |  |  |
| 3 | Bar sizes (diameters) correct as specified on the reinforcement drawing |  |  |  |  |
| 4 | Bar grade correct (e.g., B500B, B500C) and clearly marked / tagged |  |  |  |  |
| 5 | Reinforcement clean and free from excessive rust, oil, grease, paint, or loose scale |  |  |  |  |
| 6 | Pre-fabricated cages and bent bars match the approved bending schedule shapes |  |  |  |  |
| 7 | Mesh reinforcement (if used) correct type, size, and sheet dimensions |  |  |  |  |
| 8 | Reinforcement stored off the ground on bearers, protected from contamination |  |  |  |  |

Section B: Bar Positioning and Arrangement

| B: Positioning |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Reinforcement positioned in accordance with the approved reinforcement drawing |  |  |  |  |
| 2 | Correct number of bars in each layer and direction |  |  |  |  |
| 3 | Bar spacing correct in both directions (measured and within tolerance) |  |  |  |  |
| 4 | Lap lengths correct as specified on the drawing / detailing standard |  |  |  |  |
| 5 | Lap locations staggered as per drawing (not all laps at the same cross-section) |  |  |  |  |
| 6 | Anchorage lengths correct at supports and end conditions |  |  |  |  |
| 7 | Starter bars / crank bars correctly positioned and projecting to correct length |  |  |  |  |
| 8 | Continuity reinforcement at construction joints correctly positioned |  |  |  |  |
| 9 | U-bars and hairpin bars at edges correctly positioned |  |  |  |  |
| 10 | Reinforcement layers correctly ordered (bottom / top, inner / outer as per drawing) |  |  |  |  |
| 11 | Links / stirrups correct size, spacing, shape, and orientation |  |  |  |  |

Section C: Concrete Cover

| C: Cover Verification |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Minimum specified cover achieved to all exposed faces (checked with cover meter or spacer measurement) |  |  |  |  |
| 2 | Spacers / chairs correct type for exposure class (concrete, plastic, or stainless steel as specified) |  |  |  |  |
| 3 | Spacer density adequate (typically 4–5 per m2 for soffit, 2–3 per m2 for vertical faces) |  |  |  |  |
| 4 | Spacers correctly positioned and stable — not dislodged during fixing |  |  |  |  |
| 5 | Side cover to vertical faces maintained using appropriate side spacers or wheel spacers |  |  |  |  |
| 6 | Cover to top reinforcement maintained using bar chairs at correct spacing |  |  |  |  |
| 7 | No reinforcement in direct contact with formwork surface |  |  |  |  |
| 8 | Cover at construction joints checked (starter bars properly spaced from formwork) |  |  |  |  |
| 9 | Cover verified at congested areas (beam-column junctions, pile caps) — adequate for vibrator access |  |  |  |  |

Section D: Fixing, Tying, and Stability

| D: Fixing Quality |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | All bar intersections tied with approved tying wire (soft annealed steel wire, min. 1.6mm) |  |  |  |  |
| 2 | Tying pattern adequate for stability (at least every other intersection) |  |  |  |  |
| 3 | Reinforcement cage stable and rigid — will not displace during concrete placement |  |  |  |  |
| 4 | Reinforcement adequately supported to prevent sagging or displacement |  |  |  |  |
| 5 | Mechanical couplers (if used) correctly installed per manufacturer's instructions |  |  |  |  |
| 6 | Coupler type approved and test certificates available |  |  |  |  |
| 7 | Welded connections (if permitted) completed per approved WPS and inspected |  |  |  |  |
| 8 | No cutting or bending of reinforcement on site without engineer approval |  |  |  |  |
| 9 | Anti-crack reinforcement and distribution steel correctly placed |  |  |  |  |

Section E: Embedded Items and Services

| E: Embedments |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Cast-in items (holding-down bolts, plates, channels) correctly positioned relative to reinforcement |  |  |  |  |
| 2 | Adequate clearance between reinforcement and cast-in items for concrete flow |  |  |  |  |
| 3 | Conduits, sleeves, and service penetrations positioned and secured |  |  |  |  |
| 4 | Waterstops correctly positioned at construction joints and lapped / welded |  |  |  |  |
| 5 | Box-outs for openings do not conflict with reinforcement arrangement |  |  |  |  |
| 6 | All clashes between reinforcement and services resolved before pour |  |  |  |  |

Section F: Final Pre-Pour Checks

| F: Final Verification |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Inspection Item | Y | N | N/A | Comments |
| 1 | Reinforcement drawing signed off by the checking engineer (design checked) |  |  |  |  |
| 2 | All reinforcement in place — no missing bars identified |  |  |  |  |
| 3 | Formwork clean and free from debris, standing water, and ice |  |  |  |  |
| 4 | Construction joint surface prepared (roughened, clean, dampened if required) |  |  |  |  |
| 5 | Kicker or previous pour surface clean and free from laitance |  |  |  |  |
| 6 | Concrete cover confirmed with cover meter at representative locations |  |  |  |  |
| 7 | Photographs taken of reinforcement prior to pour (record for as-built documentation) |  |  |  |  |
| 8 | Pour method statement and concrete supply confirmed |  |  |  |  |
| 9 | Access for vibrators confirmed — adequate gaps between bars for compaction |  |  |  |  |
| 10 | Weather forecast reviewed — no adverse conditions expected during pour and curing |  |  |  |  |

Cover Verification Readings (Representative Checks)

Record cover meter readings at representative locations. Minimum cover must not be less than the specified nominal cover minus the permitted tolerance (cdev).

| Pt | Location / Face | Specified Cover (mm) | Measured Cover (mm) | Min. Permissible (mm) | Pass / Fail | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |

Non-Conformances / Defects

| No. | Section / Item | Description | Corrective Action | Closed Out |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |

Overall Reinforcement Inspection Result

| Reinforcement Status | APPROVED FOR POUR / NOT APPROVED / REMEDIAL WORKS REQUIRED |
| --- | --- |

Inspection Sign-Off

By signing below, we confirm that the reinforcement has been inspected against the approved drawings, bar bending schedule, and this checklist. The reinforcement is / is not approved for concrete placement.

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Steel Fixer Foreman |  |  | DD/MM/YYYY |
| Site Engineer |  |  | DD/MM/YYYY |
| Structural Engineer / Clerk of Works |  |  | DD/MM/YYYY |
| Client Representative |  |  | DD/MM/YYYY |

Notes / Additional Comments

Record additional observations, photograph references, or mill certificate numbers.

Document Control

| Rev | Date | Description | Author | Approved |
| --- | --- | --- | --- | --- |
| 0 |  | Initial issue |  |  |
| 1 |  |  |  |  |