Invasive Species Record Form

It is an offence under the Wildlife and Countryside Act 1981 to cause invasive non-native species listed on Schedule 9 to grow in the wild.

For recording and managing invasive non-native species (INNS) on construction sites

| Document Reference: | INNS- |
| --- | --- |
| Revision: | 0 |
| Date: | DD/MM/YYYY |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| Site Address: | Enter full site address |
| Ecologist / Specialist: | Name and company |
| Recorded By: | Enter name and role |

1. Species Identification

| Species Identified: | Common name and Latin name |
| --- | --- |
| Identified By: | Name and qualification (ecologist, PCA surveyor, etc.) |
| Date of Identification: | DD/MM/YYYY |
| Location on Site: | Describe exact location; mark on site plan and provide grid reference |
| Approximate Extent: | Area in m² and/or linear extent |
| Growth Stage: | e.g., Active growth, dormant, flowering, seeding, dead canes present |
| Proximity to Watercourse: | Distance to nearest watercourse, drain, or water body |
| Proximity to Site Boundary: | Distance to site boundary / neighbouring property |
| Photographic Record: | Photo reference numbers |

2. Common Invasive Species Reference (tick if present)

| No. | Species | Key Identification Features | Present |
| --- | --- | --- | --- |
| 1 | Japanese Knotweed (Fallopia japonica) | Tall stems with distinctive zigzag pattern, shield-shaped leaves, clusters of small white flowers in late summer. Rhizome system extends up to 7m laterally and 3m deep. |  |
| 2 | Giant Hogweed (Heracleum mantegazzianum) | Very large plant (3–5m tall), thick bristly stems with purple blotches, large umbrella-shaped white flower heads. SAP CAUSES SEVERE BURNS — do not touch. |  |
| 3 | Himalayan Balsam (Impatiens glandulifera) | Tall annual (1–2m), reddish-green stems, pink-purple helmet-shaped flowers, exploding seed pods. Commonly found near watercourses. |  |
| 4 | Signal Crayfish (Pacifastacus leniusculus) | Large freshwater crayfish with red undersides to claws. Found in rivers and streams. Displaces native white-clawed crayfish. |  |
| 5 | Floating Pennywort (Hydrocotyle ranunculoides) | Aquatic plant forming dense mats on water surface. Round kidney-shaped leaves on long stalks. |  |
| 6 | New Zealand Pigmyweed (Crassula helmsii) | Dense mat-forming aquatic/marginal plant with small succulent leaves. Extremely invasive in ponds and ditches. |  |

3. Management and Control Measures

| No. | Control Measure | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Ecological survey completed identifying invasive species presence, extent, and species type |  |  |  |  |
| 2 | Invasive species management plan prepared by a qualified ecologist or specialist contractor |  |  |  |  |
| 3 | All invasive species locations clearly marked on site plan and demarcated with exclusion fencing on the ground |  |  |  |  |
| 4 | Workers briefed on identification of invasive species present on site (photographs provided) |  |  |  |  |
| 5 | Biosecurity measures in place: designated clean zone and contamination zone established |  |  |  |  |
| 6 | Boot washing / disinfection stations provided at zone boundaries |  |  |  |  |
| 7 | Vehicles, plant, and equipment cleaned before leaving contaminated areas |  |  |  |  |
| 8 | No soil, vegetation, or water from contaminated areas moved to clean areas without treatment |  |  |  |  |
| 9 | Contaminated soil and plant material classified as controlled waste under Environmental Protection Act 1990 |  |  |  |  |
| 10 | Licensed waste carrier and permitted facility identified for disposal of contaminated material |  |  |  |  |
| 11 | Waste Transfer Notes reference invasive species contamination (EWC code 20 02 01 or as advised) |  |  |  |  |
| 12 | Herbicide treatment programme planned and executed by qualified operatives (PA1/PA6 certified) |  |  |  |  |
| 13 | Root barrier or membrane installed where specified to prevent rhizome spread (Japanese Knotweed) |  |  |  |  |
| 14 | Monitoring programme established to check for regrowth for minimum 3 growing seasons post-treatment |  |  |  |  |
| 15 | Environment Agency notified where invasive species are near controlled waters |  |  |  |  |
| 16 | No invasive plant material composted on site or disposed of via standard green waste routes |  |  |  |  |

4. Treatment Record

| No. | Date | Treatment Method | Product / Herbicide | Operator | Weather | Notes / Observations |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |

5. Post-Treatment Monitoring

| Date | Season / Year | Observations (Regrowth?) | Action Required | Inspected By |
| --- | --- | --- | --- | --- |

6. Contaminated Material Disposal

| Material Type: | e.g., Contaminated soil, plant material, rhizome fragments |
| --- | --- |
| Estimated Quantity: | tonnes / m³ |
| Waste Carrier: | Name and registration number |
| Disposal Facility: | Name, address, and permit number (must accept INNS waste) |
| WTN / Consignment Note Ref: |  |
| Disposal Date: | DD/MM/YYYY |

Notes / Additional Comments

Enter any additional comments. Attach site plan, photographs, and ecological survey extracts.

Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Recorded By |  |  | DD/MM/YYYY |
| Ecologist / Specialist |  |  | DD/MM/YYYY |
| Site Manager |  |  | DD/MM/YYYY |
| Environmental Manager |  |  | DD/MM/YYYY |

Document Control

| Rev. | Date | Author | Description |
| --- | --- | --- | --- |
| 0 | DD/MM/YYYY |  | Initial issue |