Water Pollution Prevention Checklist

Causing or knowingly permitting a water discharge activity or groundwater activity without a permit is a criminal offence under the Environmental Permitting (England and Wales) Regulations 2016.

In accordance with the Environmental Permitting Regulations 2016, Water Resources Act 1991, and Environment Agency guidance (GPP5, GPP21, GPP22)

| Document Reference: | WPP-CHK-001 |
| --- | --- |
| Revision: | 0 |
| Inspection Date: | DD/MM/YYYY |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| Nearest Watercourse: | Name and distance from site boundary |
| Discharge Consent Ref: | EA permit reference (if applicable) |
| Inspector Name: | Enter name and role |

Section A: Site Drainage and Treatment

| No. | Drainage Control | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Site drainage plan prepared showing all surface water drains, foul drains, and watercourse locations |  |  |  |  |
| 2 | All drainage on site identified and marked (surface water drains colour-coded blue, foul drains red) |  |  |  |  |
| 3 | Drain covers and gullies protected with drain mats, neoprene covers, or sandbags during works |  |  |  |  |
| 4 | No site runoff entering surface water drains without treatment (settlement, filtration) |  |  |  |  |
| 5 | Temporary settlement lagoons, silt traps, or treatment systems installed and operational |  |  |  |  |
| 6 | Settlement facilities sized appropriately for the contributing catchment area and flow rates |  |  |  |  |
| 7 | Outfall from treatment system monitored for turbidity and visual quality |  |  |  |  |
| 8 | Discharge consent obtained from Environment Agency where required (Environmental Permitting Regulations 2016) |  |  |  |  |
| 9 | Silt fencing installed along watercourse banks and around exposed soil areas |  |  |  |  |
| 10 | Drainage channels and ditches maintained to prevent blockage and overflow |  |  |  |  |

Section B: Silt and Sediment Control

| No. | Silt / Sediment Control | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Topsoil and subsoil stockpiles located away from watercourses (minimum 10m buffer zone) |  |  |  |  |
| 2 | Stockpiles covered or seeded to prevent wind and rain erosion |  |  |  |  |
| 3 | Silt curtains deployed in or adjacent to watercourses during in-channel or near-bank works |  |  |  |  |
| 4 | Haul routes maintained to minimise sediment tracking (road sweeping, wheel wash, rumble strips) |  |  |  |  |
| 5 | Wheel wash facility operational with contained effluent (not discharged to surface water) |  |  |  |  |
| 6 | Exposed ground areas minimised and temporary vegetation or erosion blankets applied where practicable |  |  |  |  |
| 7 | Earth bunds or cut-off ditches installed to intercept runoff from construction areas |  |  |  |  |
| 8 | Dewatering from excavations passed through settlement prior to discharge |  |  |  |  |

Section C: Chemical, Fuel, and Cement Control

| No. | Chemical / Fuel Control | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | All fuel, oil, and chemicals stored in bunded areas (110% capacity of largest container or 25% of total) |  |  |  |  |
| 2 | Bund integrity checked regularly: no cracks, gaps, or accumulated rainwater |  |  |  |  |
| 3 | Drip trays under all refuelling operations, generators, hydraulic connections, and stationary plant |  |  |  |  |
| 4 | Spill kits positioned at all fuel stores, watercourse crossings, and high-risk areas |  |  |  |  |
| 5 | Spill kit contents checked weekly and restocked after any use |  |  |  |  |
| 6 | Concrete and cement washout contained in designated lined and bunded washout facilities |  |  |  |  |
| 7 | No concrete or cement slurry discharged to ground, drains, or watercourses |  |  |  |  |
| 8 | Chemical stores located minimum 10m from watercourses and surface water drains |  |  |  |  |
| 9 | SDSs available for all chemicals on site; COSHH assessments completed |  |  |  |  |
| 10 | Biodegradable hydraulic oils used in plant operating near or over watercourses where practicable |  |  |  |  |

Section D: Monitoring and Emergency Response

| No. | Monitoring Requirement | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Visual monitoring of watercourses upstream and downstream of site conducted daily |  |  |  |  |
| 2 | Turbidity monitoring conducted where specified by discharge consent or environmental management plan |  |  |  |  |
| 3 | Monitoring results recorded and compared against consent conditions |  |  |  |  |
| 4 | Any discolouration, sheen, foam, or odour in watercourses reported immediately to site environmental coordinator |  |  |  |  |
| 5 | Environment Agency notified immediately of any pollution incident (0800 80 70 60) |  |  |  |  |
| 6 | Rainfall forecasts checked daily; enhanced controls deployed before heavy rain events |  |  |  |  |
| 7 | Emergency environmental response plan in place and communicated to all site personnel |  |  |  |  |

Actions Arising

| No. | Action Required | Responsible | Priority | Target Date | Status |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |

Inspector Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Inspector |  |  | DD/MM/YYYY |
| Site Manager |  |  | DD/MM/YYYY |

Document Control

| Rev. | Date | Author | Description |
| --- | --- | --- | --- |
| 0 | DD/MM/YYYY |  | Initial issue |