Silica Dust Risk Assessment

Respirable Crystalline Silica (RCS) is a Group 1 carcinogen. Exposure causes silicosis, lung cancer, and COPD. There is no cure.

In accordance with COSHH 2002, EH40/2005, and HSE construction dust guidance INDG463

| Document Reference: | SIL-RA-001 |
| --- | --- |
| Revision: | 0 |
| Date: | DD/MM/YYYY |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| Location / Area: | Enter specific work area |
| Assessed By: | Enter name and role |
| Approved By: | Enter name and role |

1. Workplace Exposure Limit (Reference)

| Substance | WEL (8-hr TWA) | Short-Term (15 min) |
| --- | --- | --- |
| Respirable Crystalline Silica (RCS) | 0.1 mg/m³ | No STEL assigned |
| Inhalable dust (general) | 10 mg/m³ | No STEL assigned |
| Respirable dust (general) | 4 mg/m³ | No STEL assigned |

2. Activity and Silica Source Identification

| Activity / Task: | Describe the dust-generating activity |
| --- | --- |
| Material Being Worked: | e.g., Concrete, sandstone, mortar, brick, granite, engineered stone |
| Silica Content (approx): | e.g., Sandstone ~70%, Concrete ~25%, Granite ~30%, Brick ~10-30% |
| Duration of Exposure: | Hours per day |
| Frequency: | e.g., Daily, weekly, occasional |
| Number of Workers Exposed: |  |
| Location (Indoor / Outdoor): | Indoor work significantly increases exposure risk |

3. Silica Dust Sources Present

| No. | Dust-Generating Activity | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Cutting, grinding, or drilling concrete, morite, or cement products |  |  |  |  |
| 2 | Cutting or grinding sandstone, granite, or natural stone |  |  |  |  |
| 3 | Cutting or chasing brickwork or blockwork |  |  |  |  |
| 4 | Demolition of concrete or masonry structures |  |  |  |  |
| 5 | Scabbling or needle-gunning concrete surfaces |  |  |  |  |
| 6 | Dry sweeping or compressed air blowing of silica-containing dust |  |  |  |  |
| 7 | Angle grinding paving slabs, kerbs, or precast concrete |  |  |  |  |
| 8 | Tunnelling, mining, or excavation in silica-bearing rock or sand |  |  |  |  |
| 9 | Mixing or handling dry cement, mortar, or plaster |  |  |  |  |
| 10 | Shot blasting or abrasive blasting of concrete or steel with silica-containing media |  |  |  |  |

4. Control Measures

Apply controls in the following hierarchy: eliminate, substitute, engineering controls (water/LEV), administrative controls, and RPE as a last resort. RPE alone is never sufficient as the sole control.

| No. | Control Measure | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Alternative work methods considered to eliminate or reduce dust generation (e.g., pre-fabrication, mechanical fixing instead of chasing) |  |  |  |  |
| 2 | Substitution of silica-containing materials with lower-risk alternatives where practicable |  |  |  |  |
| 3 | Water suppression used on all cutting, grinding, and drilling operations (on-tool water feed) |  |  |  |  |
| 4 | Local exhaust ventilation (LEV) / on-tool dust extraction fitted and functioning on all power tools |  |  |  |  |
| 5 | LEV systems examined and tested every 14 months (COSHH Reg. 9) with records maintained |  |  |  |  |
| 6 | Enclosed or semi-enclosed work areas used where practicable to contain dust |  |  |  |  |
| 7 | Cutting and grinding operations conducted outdoors or in well-ventilated areas where possible |  |  |  |  |
| 8 | Dust suppression used during demolition operations (water sprays, misting cannons) |  |  |  |  |
| 9 | Wet sweeping or industrial vacuum cleaners (H or M class filter) used for clean-up — never dry sweeping |  |  |  |  |
| 10 | Work area damped down before and during dust-generating activities |  |  |  |  |
| 11 | Exclusion zones established around dust-generating operations with warning signage |  |  |  |  |
| 12 | Work scheduled to minimise number of persons exposed (out of hours, zoning) |  |  |  |  |
| 13 | RPE provided and worn: minimum FFP3 disposable or half-mask with P3 filters for silica dust |  |  |  |  |
| 14 | RPE face-fit tested for each wearer with valid certificate held on site |  |  |  |  |
| 15 | RPE training delivered: correct fitting, seal checking, storage, and replacement schedule |  |  |  |  |
| 16 | Overalls or disposable coveralls provided to prevent contamination of personal clothing |  |  |  |  |
| 17 | Adequate washing facilities available for workers exposed to silica dust (warm water, soap, clean towels) |  |  |  |  |
| 18 | Workers briefed not to eat, drink, or smoke in dust-contaminated areas |  |  |  |  |

5. Health Surveillance

| No. | Health Surveillance Requirement | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Health surveillance programme established for all workers regularly exposed to respirable crystalline silica (RCS) |  |  |  |  |
| 2 | Baseline health assessment completed before first exposure (including lung function test / spirometry) |  |  |  |  |
| 3 | Follow-up health surveillance conducted at intervals recommended by occupational health provider |  |  |  |  |
| 4 | Health surveillance records maintained for at least 40 years (COSHH Reg. 11) |  |  |  |  |
| 5 | Workers informed of their individual health surveillance results |  |  |  |  |
| 6 | Occupational health provider notified of the substances, exposure levels, and control measures in use |  |  |  |  |
| 7 | Procedure in place for action if health surveillance identifies adverse health effects |  |  |  |  |

6. Exposure Monitoring

| No. | Monitoring Requirement | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Exposure monitoring conducted by a competent person where significant exposure to RCS is likely |  |  |  |  |
| 2 | Personal air sampling used to determine individual worker exposure (gravimetric analysis for RCS) |  |  |  |  |
| 3 | Results compared against the UK Workplace Exposure Limit for RCS: 0.1 mg/m³ (8-hour TWA) |  |  |  |  |
| 4 | Monitoring records maintained for at least 40 years |  |  |  |  |
| 5 | Action taken where monitoring results approach or exceed the WEL |  |  |  |  |
| 6 | Monitoring repeated after any change in work process, controls, or materials |  |  |  |  |

7. Notes / Additional Comments

Enter any additional comments, references to COSHH assessments for specific products, or monitoring reports.

Approval and Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Assessor |  |  | DD/MM/YYYY |
| Reviewer |  |  | DD/MM/YYYY |
| Approver |  |  | DD/MM/YYYY |

Document Control

| Rev. | Date | Author | Description |
| --- | --- | --- | --- |
| 0 | DD/MM/YYYY |  | Initial issue |