Hot Works Permit Form

HOT WORKS MUST NOT COMMENCE WITHOUT A VALID PERMIT. FIRE WATCH MUST BE MAINTAINED FOR MINIMUM 60 MINUTES AFTER COMPLETION.

In accordance with the Regulatory Reform (Fire Safety) Order 2005 and HSE fire safety guidance for construction sites

| Permit Number: | HW-PTW- |
| --- | --- |
| Revision: | 0 |
| Date: | DD/MM/YYYY |
| Start Time: | HH:MM |
| Permit Valid Until: | DD/MM/YYYY at HH:MM (maximum shift duration) |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| Location of Hot Works: | Enter precise location (building, floor, area, grid reference) |
| Issued By (Authorised Person): | Name and role |
| Permit Holder: | Name, role, and employer |
| Fire Watch Person: | Name (must remain at location during and after hot works) |

1. Description of Hot Works

| Description of Work: | Describe the hot works activity |
| --- | --- |
| Equipment Used: | e.g., MIG welder, angle grinder, oxy-acetylene set, tar boiler |
| Risk Assessment Ref.: |  |
| Method Statement Ref.: |  |

2. Type of Hot Works (tick all applicable)

| No. | Hot Works Type | Tick | N/A |
| --- | --- | --- | --- |
| 1 | Welding (MIG, TIG, MMA/stick, oxy-acetylene) |  |  |
| 2 | Burning / flame cutting (oxy-fuel cutting) |  |  |
| 3 | Brazing or soldering |  |  |
| 4 | Grinding (angle grinder, bench grinder) |  |  |
| 5 | Disc cutting (metal, concrete, masonry) |  |  |
| 6 | Bitumen / asphalt heating (tar boilers, bitumen kettles) |  |  |
| 7 | Lead work / plumbing with naked flame |  |  |
| 8 | Use of blow torch or heat gun |  |  |
| 9 | Other hot works (specify in comments) |  |  |

3. Pre-Work Fire Safety Checks

| No. | Pre-Work Requirement | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Work area inspected for combustible materials within a 10-metre radius of the hot works |  |  |  |  |
| 2 | All combustible materials removed or protected with fire-resistant covers/blankets |  |  |  |  |
| 3 | Flammable liquids, gases, and aerosols removed from the area |  |  |  |  |
| 4 | Floor swept clean of combustible dust, debris, wood shavings, and packing materials |  |  |  |  |
| 5 | Fire-resistant screens or welding curtains erected to contain sparks and spatter |  |  |  |  |
| 6 | Gaps, cracks, and openings in walls, floors, and ceilings sealed to prevent spark travel to adjacent areas |  |  |  |  |
| 7 | Fire detection systems in the area temporarily isolated and replaced with fire watch (if applicable) |  |  |  |  |
| 8 | Sprinkler system heads within the hot works area protected from damage but not obstructed |  |  |  |  |
| 9 | Adjacent rooms, floors below, and concealed spaces checked for combustible materials |  |  |  |  |
| 10 | LPG cylinders stored upright, secured, and at a safe distance from the work area (minimum 3m) |  |  |  |  |
| 11 | Flashback arrestors fitted on all oxy-acetylene equipment at both regulator and torch |  |  |  |  |
| 12 | All gas hoses and connections inspected and in good condition (no damage, leaks, or repairs with tape) |  |  |  |  |
| 13 | Appropriate fire extinguisher(s) readily available at the point of work (minimum 1x CO2 + 1x water or foam) |  |  |  |  |
| 14 | Fire watch personnel nominated, trained, and equipped with extinguisher and communication device |  |  |  |  |
| 15 | LEV / fume extraction provided where welding or cutting fume exposure is likely (COSHH requirement) |  |  |  |  |
| 16 | RPE provided where fume extraction is insufficient (minimum FFP2 for mild steel, FFP3 for stainless steel/galvanised) |  |  |  |  |
| 17 | Adjacent activities assessed for potential conflict (e.g., painting, solvent use, fuel storage nearby) |  |  |  |  |

4. Permit Authorisation

I confirm that the above fire precautions are in place. Hot works may proceed subject to the conditions of this permit.

| Role | Name | Signature | Date / Time |
| --- | --- | --- | --- |
| Authorised Person (Issuer) |  |  | DD/MM/YYYY HH:MM |
| Permit Holder (Hot Works Operative) |  |  | DD/MM/YYYY HH:MM |
| Fire Watch Person |  |  | DD/MM/YYYY HH:MM |

5. Fire Watch Log

The fire watch person must remain at the hot works location during all hot works and for a minimum of 60 minutes after completion. Record each check below.

| Date | Time | Observation / Status | Fire Watch Name | Initials |
| --- | --- | --- | --- | --- |

6. Post-Work Checks

| No. | Post-Work Requirement | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Hot works area inspected immediately on completion of work for signs of fire or smouldering |  |  |  |  |
| 2 | Fire watch maintained for a minimum of 60 minutes after hot works cease (or as per site policy) |  |  |  |  |
| 3 | Final inspection of the hot works area, adjacent areas, and floors below carried out at end of fire watch period |  |  |  |  |
| 4 | All hot works equipment turned off, secured, and stored safely |  |  |  |  |
| 5 | Gas cylinders turned off at the regulator and keys removed |  |  |  |  |
| 6 | Area confirmed clear and returned to normal condition |  |  |  |  |
| 7 | Fire detection systems reinstated (if previously isolated) |  |  |  |  |
| 8 | Hot works area re-inspected at the end of the shift / before site closes |  |  |  |  |

7. Permit Closure

| Hot Works Completed At: | HH:MM |
| --- | --- |
| Fire Watch Ended At: | HH:MM (minimum 60 mins after completion) |
| Final Inspection Clear: | Yes / No |
| Fire Detection Reinstated: | Yes / No / N/A |
| Permit Closed By: | Name, signature, date/time |

Notes / Additional Comments

Enter any additional comments.

Document Control

| Rev. | Date | Author | Description |
| --- | --- | --- | --- |
| 0 | DD/MM/YYYY |  | Initial issue |