Noise at Work Risk Assessment

In accordance with the Control of Noise at Work Regulations 2005 and HSE guidance L108

| Document Reference: | NOISE-RA-001 |
| --- | --- |
| Revision: | 0 |
| Date: | DD/MM/YYYY |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| Location / Area: | Enter specific work area |
| Assessed By: | Enter name and role |
| Approved By: | Enter name and role |

1. Noise Action Levels (Reference)

The Control of Noise at Work Regulations 2005 set the following exposure action values and limit values for a daily or weekly personal noise exposure:

| Action Level | dB(A) LEPd | Required Action |
| --- | --- | --- |
| Lower Exposure Action Value | 80 dB(A) | Provide information and training; make hearing protection available on request |
| Upper Exposure Action Value | 85 dB(A) | Reduce exposure so far as reasonably practicable; designate hearing protection zones; enforce wearing of hearing protection; provide health surveillance |
| Exposure Limit Value | 87 dB(A) | Must not be exceeded (taking account of hearing protection attenuation) |
| Peak Sound Pressure (Lower) | 135 dB(C) | As per lower exposure action value |
| Peak Sound Pressure (Upper) | 137 dB(C) | As per upper exposure action value |

2. Noise Sources Identification

Identify all significant noise sources in the work area. Include plant, equipment, tools, and processes. Record manufacturer data where available or measured noise levels.

| No. | Noise Source / Equipment | Mfr. Data dB(A) | Measured dB(A) | Duration (hrs/day) | Persons Exposed |
| --- | --- | --- | --- | --- | --- |
| 1 | Concrete breaker / demolition hammer | 105–115 |  |  |  |
| 2 | Angle grinder (230mm) | 95–105 |  |  |  |
| 3 | Circular saw (bench / hand-held) | 95–105 |  |  |  |
| 4 | Compressor (diesel) | 80–95 |  |  |  |
| 5 | Piling rig | 90–110 |  |  |  |
| 6 | Generator | 75–90 |  |  |  |
| 7 | Excavator / Dumper | 80–95 |  |  |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  |  |  |  |

3. Noise Monitoring Record

| Noise Survey Date: | DD/MM/YYYY |
| --- | --- |
| Surveyor Name: | Name and competence (e.g., IOA certificate, BOHS) |
| Sound Level Meter Make / Model: |  |
| Serial Number: |  |
| Calibration Date: | DD/MM/YYYY |
| Field Calibration Check (before): | dB at HH:MM |
| Field Calibration Check (after): | dB at HH:MM |
| Dosimeter Used: | Yes / No — If yes, record make, model, serial number |

4. Personal Daily Noise Exposure (LEPd) Summary

Calculate the daily personal noise exposure for each worker or group exposed. Use HSE noise exposure calculator or equivalent method.

| No. | Worker / Group | LEPd dB(A) | Peak dB(C) | Action Level | Controls Required |
| --- | --- | --- | --- | --- | --- |

5. Noise Control Measures

| No. | Control Measure | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Noise reduction at source considered (e.g., quieter plant, different work method, damping, enclosure) |  |  |  |  |
| 2 | Low-noise purchasing policy applied when selecting plant and equipment |  |  |  |  |
| 3 | Noisy activities scheduled to minimise the number of persons exposed (e.g., out of hours, zoning) |  |  |  |  |
| 4 | Job rotation implemented to reduce individual daily exposure where practicable |  |  |  |  |
| 5 | Engineering controls installed (e.g., noise barriers, acoustic enclosures, anti-vibration mounts) |  |  |  |  |
| 6 | Hearing protection zones established, demarcated, and signed (mandatory above 85 dB(A)) |  |  |  |  |
| 7 | Suitable hearing protection selected providing adequate attenuation without over-protection |  |  |  |  |
| 8 | Hearing protection available for all workers exposed above 80 dB(A) lower action value |  |  |  |  |
| 9 | Hearing protection mandatory and enforced for all workers exposed above 85 dB(A) upper action value |  |  |  |  |
| 10 | SNR (Single Number Rating) or HML method used to verify hearing protector attenuation is adequate |  |  |  |  |
| 11 | Workers trained on noise risks, action levels, hearing protection use, and health surveillance programme |  |  |  |  |
| 12 | Health surveillance (audiometry) programme established for workers regularly exposed above 85 dB(A) |  |  |  |  |
| 13 | Baseline audiometry completed before first exposure for all workers in the health surveillance programme |  |  |  |  |
| 14 | Follow-up audiometry scheduled at regular intervals (annually for first 2 years, then 3-yearly if stable) |  |  |  |  |

6. Hearing Protection Selection

| Protector Type | Make / Model | SNR (dB) | Effective Level dB(A) | Adequate? | Notes |
| --- | --- | --- | --- | --- | --- |
| Disposable ear plugs |  |  |  |  |  |
| Reusable ear plugs |  |  |  |  |  |
| Ear defenders (muffs) |  |  |  |  |  |
| Helmet-mounted defenders |  |  |  |  |  |

7. Notes / Additional Comments

Enter any additional comments, reference to noise survey reports, or action items.

Approval and Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Assessor |  |  | DD/MM/YYYY |
| Reviewer |  |  | DD/MM/YYYY |
| Approver |  |  | DD/MM/YYYY |

Document Control

| Rev. | Date | Author | Description |
| --- | --- | --- | --- |
| 0 | DD/MM/YYYY |  | Initial issue |